.@0CAM

.@1CAMSET

.T @0

.=m5

.s15

.c @0

.s

.c Interface/Control Language

.c for special needs users.

.s3

.c Version 1.6

.s

.c Revised 23-Feb-02

.s5

.c for Cameron

.s5

.c ?COPY.TXT 1982-2005 Dave Dunfield

.c **See COPY.TXT**.

.P

> INTRODUCTION

@0 is a special purpose version of the BASIC (Beginners All purpose Symbolic

Instruction Code) computer language, which has extensions for controlling

non-standard hardware and control functions specifically designed for the

purpose of providing a computer interface for a child with special needs.

Although designed to help children, @0 is by no means limited to this

application. It can be applied to any project which could benefit from

its interfaces, such as automated displays, presentations, simulations

etc. The possibilities are endless.

@0 is not intended for "general purpose" software development. As a

programming language it is quite simple, but adaquate for the control and

interface functions for which it is intended. It's real value lies in it's

"built in" ability to interface with and control a variety of unconventional

I/O hardware.

The interfaces currently supported by @0 are:

.f-

1 - Standard PC keyboard and video display

2 - A separate Four-button input keypad

3 - Beeps and melodies via the PC speaker

4 - Sound input/output via a "Sound Blaster" card

5 - Text-to-speech via a "Sound Blaster" card

6 - Infa-red "remote" control of external devices

7 - Analog knobs, dials, levers etc.

8 - Digital I/O via "parallel" port.

9 - X10 "Home Control" devices via CP290 interface

.f+

WARNING! Installing and connecting the external interfaces requires some

knowledge and skill with electrical wiring and mechanical devices. If you

do not have these talents, you are STRONGLY advised to seek the assistance

of someone who does. Poorly designed, poorly constructed or improperly

wired interfaces can be dangerous!

In the development of @0, I have tried to insure that it will run on minimal

hardware, so that you can take advantage of older equipment that may be

available at little or no cost. The program itself will run on 8088 and

80286 based computers, although newer Sound Cards and associated drivers may

dictate a higher level platform.

@0 is available free of charge for non-profit use. All I ask is that if you

create anything with this system that could be of benefit to others, that

you also make it available free of charge. Updated versions of @0 as well

as any submissions from the users will be made available on my BBS (See

enclosed CATALOG file).

Also, if you redistribute this software, please do so in the form of

my original .ZIP archive.

.P

> EXTERNAL INTERFACES

Since @0 is designed to provide an interface for someone who may not be

able to use the keyboard and video display, several external interface

methods are provided.

>> Remote Keyboard

The remote keypad will consist of four buttons, although fewer button

can be implemented if the applications will not require four. In most

cases, the keypad will be large easy to operate buttons or levers. A

variety of colors, shapes, textures etc. can be used to distinguish

the buttons, depending on the capabilities of the intended user. If

desired, large knobs controlling one or more analog inputs (see below)

can be physically located with the remote keypad.

The remote keypad can be interfaced either via the game (joystick) port,

or a parallel (printer) port on the PC.

>>> GAME port

The keypad is interfaced by the A1,A2,B1 and B2 switch inputs.

Each keypad button should be a Normally Open SPDT momentary contact

push button connected between the signal/pin indicated below and any one

of the "Ground" pins (4,5,12) of the PC's DB-15 game port connector.

.f-

Button#      Pin#     SIGNAL NAME

1            2          A1

2            7          A2

3           10          B1

4           14          B2

n/a       4,5,12        Ground

.f+

NOTE: That some game ports do not support the B1,B2 inputs (The ones on

Sound Blaster cards do).

>>> Parallel port

The keypad is interfaced using the -ERROR, BUSY, PE and SLCT input lines.

Each keypad button should be a Normally Open SPST momentary contact push

button connected between the signal/pin indicated below and any one of the

"Ground" pins (18-25) of the PC's DB-25 parallel port connector.

.f-

Button#      Pin#     SIGNAL NAME

1           15         -ERROR

2           11          BUSY

3           12          PE

4           13          SLCT

n/a         18-25       Ground

.f+

.P

NOTE: Most parallel ports will work with only the ground connection

indicated, however some may also require "pull-up" resistors (5k) to be

connected between the signal input and +5volts. You can obtain +5volts

from the RED wire of a disk drive power connector, or from the GAME port

(see below).

>> Analog inputs

The analog inputs are read from the game (joystick) port. Each input

consists of a variable resistance (0-150K) connected between the

signal/pin indicated below, and the "+5volts" pins (1 or 9) of the

PC's DB-15 game port connector.

.f-

Input#       Pin#     SIGNAL NAME

1            3          AX

2            6          AY

3           11          BX

4           13          BY

n/a        1,9         +5volts

.f+

>> Infared remote control

The Infared remote control is an 8031 based dedicated controller that

interfaces to the PC via a serial port. Complete details, drawings,

firmware etc. for this device (called IR232) are contained in separate

files accompanying this package.

.P

>> Sound Card

The sound interface uses Creative Labs SOUND BLASTER card, and the

CT-VOICE.DRV software device driver (included with all DOS versions of the

SOUND BLASTER). To use the text-to-speech features, you must also have the

SBTALKER.EXE driver. If you are considering using a "clone" card, make sure

that it includes these drivers (unlikely).

The sound card must be installed and configured according to the manufacturers

instructions, including:

.f-

- SOUND environment variable set to Sound Card directory.

- CT-VOICE.DRV located in DRV subdirectory of above.

- BLASTER environment variable set with card settings.

- Low level drivers loaded (if necessary).

- SBTALKER.EXE loaded (if text-to-speech is desired).

.f+

@0 has been tested and verified to work correctly with SOUND BLASTER 1.5,

2.0, SBPRO and SB16 cards. Older versions of these cards require only the

CT-VOICE.DRV file to be located in a \DRV subdirectory under the "home"

directory indicated by the SOUND environment variable. Later versions of the

cards also require "Low level" device drivers (CTSB, CTSB2, CTSBPRO or

CTSB16.SYS and CTMMSYS.SYS) to be loaded in your CONFIG.SYS files. Consult

your sound card documentation for details on installing and configuring your

particular card.

If the BLASTER environment variable exists (See sound card installation docs),

@0 will attempt to reconfigure to the Card type, Address, interrupt and Dma

channel to the parameters indicated, otherwise the default settings configured

in the driver will be used. (see INST-DRV/INSTDRV program supplied with some

cards). Note @0 uses 'Tn' setting from BLASTER to determine the type of MIXER

on the card. If incorrect, MIX command will not work.

Sound files for use by @0 applications can be prepared with the many sound

editors and related utilities that are available. We found the VEDIT2 .VOC

file editor from Creative Labs to be very useful, due to its ability to

manipulate individual blocks and insert marker records (for sequencing).

NOTE1: VEDIT2 is apparently no longer supplied with Sound Blaster cards, and

older versions of VEDIT2 may not work correctly with newer cards due to a

new record type (9) implemented in the CT-VOICE driver. The last version of

VEDIT2 (2.15) works correctly with these drivers, and is is available from

the Creative Labs web page (see below), and probably their Compuserve forum

and BBS (unverified).

NOTE2: Although VEDIT2 was not officially supported on the SB1.5 cards, and

did not work with them "out of the box", I discovered that the updated DOS

SB15 drivers from Creative Labs web page enabled this combination to work.

.P

As manufacturers embrace new graphical operatings systems and industry trends,

they have tended to drop support for the older DOS environment on which @0

relies. The CT-VOICE, SBTALKER and VEDIT2 drivers/utilities used by @0 are

(or were at one time) standard features included with SOUND BLASTER cards. As

these programs are the proprietary property of Creative Labs, you will have to

contact them if you are missing any of them:

Good places to obtain updated Creative Labs files is:

.f-

Creative Labs web page:     http://www.creaf.com

Creative Labs BBS system:   (405) 742-6660

Creative Labs Compuserve Forum

Creative Labs, Inc.  Technical Support

1523 Cimarron Plaza, Stillwater, OK 74075 U.S.A.

TEL (405) 742-6622          FAX (405) 742-6633

.f+

Files which are of special interest on the above sites:

.f-

SB15DU.EXE      - Updated DOS drivers for: SB 1.5

SB20DU.EXE      - Updated DOS drivers for: SB 2.0

SBPDU.EXE       - Updated DOS drivers for: SB PRO

SBMDU.EXE       - Updated DOS drivers for: SB MCV

SBP2DU.EXE      - Updated DOS drivers for: SB PRO-2

SBPMDU.EXE      - Updated DOS drivers for: SB PRO MCV

SBBASIC.EXE     - Basic DOS/WIN31 install disk for SB16/32

CTCMBBS.EXE     - Creative "Plug & Play" Configuration Manager

VEDIT215.EXE    - VEDIT2 version 2.15 (latest)

.f+

The above files should provide you with a compatible CT-VOICE.DRV file for

any of the listed cards, and the VEDIT2 voice editor. I was unable to find

any version of the SBTALKER.EXE program on the web site. I received SBTALKER

with the last non-P&P SB16 that I purchased, however the latest ones that I

have obtained (P&P/W95 versions) did not include this program. Again, contact

Creative Labs at the voice/fax numbers above.

I apologize for the problems involved in finding suitable drivers for recent

cards, however I would like to point out that the audio interfaces of @0 were

developed (and made to rely on these files) using the guidelines set out in

Creative Labs own "Developer Kit for Sound Blaster Series"...

New in @0 1.6: To simplify sound card setup, @0 now looks for the

CT-VOICE.DRV driver in the @0 install directory if the SOUND= environment

variable does not exist. I have also included SBDRV, a utility which will

create a preconfigured CT-VOICE.DRV driver for most ISA Sound Blaster cards.

.P

> STARTING @0

The syntax of the @0 command line is:

.c @0 [/? /Bvalue /D /Icom /Kadr /Padr,inv /Sseed] [filename [parms]]

The '/B' option specifies the "debounce" count (/B1 - /B65535) for the

remote keypad buttons. Once a button is pressed, the system will not

recognize it's release until it has been OFF for an amount of time set

by this parameter (higher values = longer time). The exact time is

dependant on your system speed (A given debounce value will result in LESS

time on a faster computer).

The '/D' option enables debugging messages in the various interfacing

functions which have been added to the language.

The '/I' option specifies the communications port (/I1 - /I4) to be used

for the IR232 infrared remote control interface. If this parameter is not

given, the IR232 interface is disabled.

The '/K' option specifies a hexidecimal parallel port address which is to be

used for reading the remote keypad buttons. If this parameter is not given,

the game (joystick) port A1,A2,B1,B2 inputs are used instead. Typical values

for PC parallel ports are:

.f-

LPT1: /K3BD         LPT2: /K379         LPT3: /K279

.f+

The '/P' option specifies a hexidecimal parallel port address to be used for

the control signals of the OSET, ON and OFF commands. 16 bits of I/O will be

written to the address given, and the address +2. An optional "invert" value

can be specified which inverts the corresponding I/O bits, allowing for

negative logic signals. Typical values for PC parallel ports are:

.f-

LPT1: /P3BC         LPT2: /P378         LPT3: /P278

.f+

The '/S' option specifies the RANDOM SEED value which will be used. This

value determines the sequence of numbers that will be returned by the

RND() function. Starting @0 with a preset seed value allows you to test

a program with the same random numbers every time. If not specified, the

seed is initialzed to the BIOS clock tick, providing an undetermined

sequence.

The /B, /I, /K and /P options can be set permanently with the @1 utility

which patches the @0 program directly. Use /? to see a brief summary of the

@0 or @1 options.

If a 'filename' is specified, @0 will automatically load and run it, then

terminate. Additional parameters may be specified on the command line, and

they will be made available as string variables A1$, A2$, A3$ etc. A0$ will

contain the name of the program.

.P

> LANGUAGE SUMMARY

@0 in built around a fairly simple but functional subset of the well

known BASIC computer language. The details of this implementation are:

>> Numbers

@0 uses "16 bit integer" numbers. This means that any numeric values

used in a program must fall in the range of -32768 to 32767. For @0's

intended purposes of control and interface functions, this type of number

is more than adaquate. Within expressions, numbers can be expressed in

hexidecimal by preceeding them with '$'.

>> Character strings

@0 can also manipulate character strings. Character strings are identified

by the double-quote (") character, and are limited to 100 characters or less

in length.

>> Variables

@0 provides a total of 780 "variables" which may be used to store numbers

and character strings. These are divided into three "types" of 260 variables

each. The variables of each type are identified with a single character from

the alphabet followed by digit from 0 to 9; ie: A0-Z9.

>>> Simple numeric variables: A0 - Z9

These variables are the most commonly used, and can each store one

number (-32768 to 32767). They are used for loop counters, input values,

and general purpose calculations.

>>> String variables: A0$ - Z9$

These variables can hold character strings. These "string" variables

are identified by a '$' appended to the name. They are normally used for

input strings, names, and any other non-numeric data.

String variables allow indexing to extract individual characters:

.f-

A0$(n)  = Return character n (0 to length-1)

.f+

>>> Numeric arrays: A0(index) - Z9(index)

These variables can each hold multiple numbers. Unlike the others,

variables of this type must be explicitly created (with the 'DIM' statement).

They are typically used for tables, lists and any applications which uses

a group of numbers.

.i-

As a matter of convience, the '0' variables can be referenced by letter

only. Ie: A is equivalent to A0, B$ is equivalent to B0$, and Z(index) is

equivalent to Z0(index)

.P

>> Functions

@0 includes a number of "built in" functions which can be used as

values in an expression:

>>> CHR$(value)

This function converts a number into an ASCII character. For example:

the ASCII value of the character 'A' is 65. Coding CHR$(65) would be

equivalent to "A". This is the reverse of the ASC() function.

>>> STR$(value)

This function converts a number into a character string. Coding STR$(1234)

would be equivalent to "1234". This is reverse of the NUM() function.

>>> ABS(value)

This function returns the "absolute value" of a number, which a number

of the same magnitude, with a positive sign. For example: ABS(5) and ABS(-5)

would both evaluate to a number of 5.

>>> ASC(char)

This function converts a character into its numeric ASCII value. For

example: coding ASC("A") would be equivalent to 65. This is the reverse of

the CHR$() function.

>>> INP(port)

This function reads an 80x86 I/O port. It is used to directly manipulate

hardware that is not supported by @0.

>>> KEY()

This function tests to see if a key has been pressed on the PC keyboard,

and if so, returns its ASCII value. If no key has been pressed, KEY() returns

0.

>>> LEN(variable)

Returns the length of the specified string or array variable. If the variable

has a '$' suffix, the length return is the number of characters in the

character variable. If no '$', the length returned is the number of elements

in the array variable.

>>> NUM(string)

This function converts a character string containing a number into a

numeric value. Coding NUM("1234") would be equivalent to 1234. This is the

reverse of the STR$() function.

.P

>>> RND(value)

This function generates a "random" number which will fall in the range

of 0 to (value-1). For example: RND(10) would generate a number ranging from

0 to 9.

>>> BUTTON(mask)

This function tests for a button press on the external keypad. The

mask parameter identifies which buttons are to be allowed. The value of

mask is determined by adding up values from the following table:

.f-

Button1     1       <- Allow button1

Button2     2       <- Allow button2

Button3     4       <- Allow button3

Button4     8       <- Allow button4

Wait        256     <- Wait for a button before proceeding

Immediate   512     <- Don't sound tone/wait till release

.f+

If the "Wait" function is included, BUTTON() will stop and wait until

an allowed button has been pressed, and then return the button number (1-4).

If the "Wait" function is not specified, BUTTON() will return 0 if no button

is being pressed, otherwise the button number as above.

If the "Immediate" function is specified, BUTTON() will return with the

button number immediately, otherwise it will sound a unique tone identifying

the button which will persist until the button is released.

The BUTTON() function will also accept the numeric keys 1-4 on the PC

keyboard to simulate a button press. ESCAPE can be used to force BUTTON()

to return 0 (no key pressed).

>>> JOYSTICK(index)

This function provides access to the game (joystick) port. An index value

of 1-4 reads the corresponding analog input. A value from 0 - <n> is

returned, where <n> is dependant on your computer speed and game port

hardware.

An index value of 0 is a special case, and provides direct access to the

game port digital inputs (buttons). A value is returned with bits 0,1,2,3

containing the status (1=Closed) of the A1,A2,B1,B2 digital inputs.

.P

>>> TIME(index)

This function reads the computers real time clock. The value returned depends

on the index value. NOTE: TIME(1-8) are not re-read from the system when

different TIME() functions are used on a single line:

.f-

0   - BIOS tick     (increments every 1/18 sec)

1   - Second        (0-59)

2   - Minute        (0-59)

3   - Hour          (0-23)

4   - Day           (1-31)

5   - Month         (1-12)

6   - Year          (1980-2099)

7   - Day of week   (0=Sun ... 6=Sat)

8   - Time since midnight in 5 second intervals (0-17280)

.f+

.P

>> Operators

Numbers, Strings, Variables and Function may be formed into expressions and

manipulated with the following operators:

.f-

+           Addition and String concatination   ($)

-           Subtraction

*           Multiplication

/           Division

%           Modulus (remainder)

&           Bitwise AND

|           Bitwise OR

^           Bitwise EXCLUSIVE OR

=           Assignment / Test EQUAL             ($)

<>          Test NOT EQUAL                      ($)

<           Test LESS THAN

<=          Test LESS THAN or EQUAL

>           Test GREATER THAN

>=          Test GREATER THAN or EQUAL

!           Bitwise NOT (complement)

.f+

Operators denoted with ($) can be applied to strings.

When applied to strings, '+' performs string concatination, which is the

operation of joining two strings into one. For example: "ABC"+"DEF" is

equivalent to "ABCDEF".

The "test" operators (=, <>, <, <=, >, >=) can be used in any expression,

and evaluate to 1 of the test is TRUE, and 0 if it is FALSE.

.P

>> Statements

All actions performed by the @0 program are coded as "statements", which

are commands directing the interpreter. Statements often contain expressions

as described above to perform calculations.

Most statements can be used either from within a program, or interactively

from the command line "Ready" prompt. Those statements which are only

available in one or the other mode are noted.

>>> BEEP frequency,milliseconds

Generates a tone from the PC speaker at the indicated frequency (hz),

for the specified duration (milliseconds).

10 REM Generate a 1000hz tone for 1/2 second

20 BEEP 1000,500

>>> CLEAR

Erases all variables. All numeric variables are set to zero. All string

variables are set to null (0 length), and all arrays are removed.

10 REM clear all variables

20 CLEAR

>>> CLOSE#unit

Closes a previously opened file (0-9). See 'OPEN#unit'.

10 REM Close file # 5

20 CLOSE#5

>>> CLS [attribute]

Clears the video screen to blanks, using the specified attribute. If

no attribute is given, a value of 7 (white on blank) is used.

10 REM Clear the screen

20 CLS

.P

>>> CONTROL "string" | function

Issues an I/R control command through the IR232 interface. This has the

effect of activating the corresponding function as if that button has been

pushed on a "remote" control.

If the argument to CONTROL is a character string, then the available I/R

control sequences are loaded from a file by that name. A numeric operand

causes the corresponding I/R control sequence from the last loaded file

to be transmitted. The sequences are selected based on the order in which

they were recorded by the IREDIT utility program. CONTROL 0 activates the

first code (0)...

10 REM Load the I/R control codes from @0.IRD

20 CONTROL "@0.IRD"

30 REM Issue command#1 to remote device

40 CONTROL 1

>>> DATA

Codes numeric and string variables in the program text such that they

can be read with the READ statement. See 'READ' and 'ORDER'.

DATA statements encountered during the normal flow of the program are

ignored. They are only used as sources of data for the READ statement.

DATA can only be used within a program (non-interactive).

10 REM Data for READ

20 DATA 1,2,3,"text"

30 DATA 4,5,6,"more text"

>>> DELAY milliseconds

Causes the program to pause for the specified number of milliseconds.

10 REM Pause for 2 seconds

20 DELAY 2000

>>> DIM variable(size)[, ...]

This command "dimensions" (creates) a numeric array variable. The size

parameter defines the maximum index value which can be used with the array.

If an array of the same name was previously defined, it is erased first.

All elements of the new array are set to zero.

Note that @0 starts numbering its elements from 0, while most other BASIC's

start numbering elements from 1.

10 REM Define arrays 11 elements (0-10) and 100 elements

20 DIM A(10), A1(99)

.P

>>> DOS "command"

Passes a string to the DOS command interpreter to be executed as a DOS

command (See also 'QDOS').

10 REM Display graphic with CSHOW (shareware viewer)

20 DOS "CSHOW "+A$+"+X"

>>> END

Terminate execution of the program. No message is issued.

10 REM End program quietly

20 END

>>> EXIT

Terminates @0 and exits back to DOS.

10 REM Terminate @0

20 EXIT

>>> FOR variable=value TO limit [STEP increment]

Begins a counted loop. The variable begins with the specified value.

The program segment between FOR and a corresponding NEXT statement

is repeated until the variable exceeds the specified limit. After

each loop, the variable is incremented by the increment value. If no

increment value is specified, a default value of 1 is used.

FOR can only be used within a program (non-interactive).

10 REM Count to 10 and print it

20 FOR i=1 to 10

30 PRINT I

40 NEXT I

>>> GOSUB line

Calls a subroutine. The program segment beginning at the specified line

number is executed until a RETURN statement is encountered, after which

the program continues at the next statement after GOSUB.

GOSUB can only be used within a program (non-interactive).

10 REM Call a subroutine

20 PRINT "start"

30 GOSUB 80

40 PRINT "middle"

50 GOSUB 80

60 PRINT "end"

70 END

80 PRINT "subroutine"

90 RETURN

.P

>>> GOTO line

Causes a "jump" to the specified line number. The program will continue

at the first statement in the indicated line.

GOTO can only be used within a program (non-interactive).

10 REM goto example

20 PRINT "Hello"

30 GOTO 20

>>> GOTOXY x,y

Positions video output cursor at the specified x and y coordinates.

10 REM postion to cursor and then output a message

20 GOTOXY 10,10:print "Hello"

>>> IF test THEN statement/line

Performs a statement conditionaly upon a test. The indicated test expression

is evaluated, and if the result is non-zero, the specified statement is

performed. As a shorthand convience, a statement consisting of only a number

is assumed to be "GOTO number".

10 REM IF example

20 FOR i=1 TO 10

30 IF i=5 THEN PRINT "I is five"

40 IF i<>8 THEN 60

50 PRINT "I is eight"

60 NEXT i

>>> INPUT ["prompt",]variable

Prompts for a variable. The specified prompt is displayed, and a single line

of input is accepted from the console, which is assigned to the indicated

variable. String variables receive the input unchanged. Numeric variables

cause the input line to be evaluated to a number.

If no "prompt" string is given, a default of "?" is assumed. Note that

you cannot directly use a character variable as the prompt (Input assumes

that you are inputting into the variable), but you can use one concatinated

to a string.

10 REM INPUT example

20 INPUT A

30 PRINT "A=",A

40 INPUT A$

50 INPUT ""+A$,B

60 PRINT "B=",B

.P

>>> INPUT#unit,variable

Reads the contents of a variable from a file previously opened with

OPEN#unit.

10 REM open example

20 OPEN#1,"TEST.FIL","r"

30 INPUT#1,A$

40 CLOSE#1

50 PRINT "A$=",A$

>>> LET variable = expression

Assigns the value of the expression to the specified variable.

The "LET" keyword is optional, since the interpreter will assume LET

for any statement which it does not recognize.

10 REM LET example

20 LET a=10

30 A$="test string"

>>> LIF test THEN statements

Performs a "long if". This statement behaves exactly like the IF statement,

except that ALL statements on the line are conditional on the test.

10 REM IF/LIF example

20 INPUT "Enter a number?",A

30 IF A = 1 THEN PRINT "A=1" : PRINT "TEST1"

40 LIF A = 2 THEN PRINT "A=2" : PRINT "TEST2"

50 IF A<>0 THEN 20

>>> LIST [start,[end]]

Displays the program on the console. If one line number is given, only

that line is listed. If both start and end lines are given, only that

range is listed.

LIST

LIST 10

LIST 10,50

>>> LIST#unit

Behaves exactly like LIST except that the program is written to a

previously opened file (0-9).

10 REM List to file demo

20 OPEN#1,"PGM.BAS","wv"

30 LIST#1

40 CLOSE#1

.P

>>> LOAD "file"

Loads the specified @0 program file into memory. Any previous program

is erased. The extension ".BAS" is assumed.

When used interactively, all variables are erased prior to loading the

new program.

When used within a program, LOAD performs a "chaining" function. The

new program is loaded, and immediately begins to run. In this case, the

variables are NOT erased, and may be used as a means of communication

between the two programs. NOTE: If you do not need to pass variables to

a chained program, it is a good idea to execute CLEAR just prior to LOAD.

This makes it much easier for @0 to manage memory, and also insures that

the new program will start with a clean set of variables.

LOAD "program"

10 REM LOAD demo

20 CLEAR

30 A$="String to pass"

40 LOAD "next"

>>> NEW

Erases the program and all variables.

NEW

>>> NEXT [variable]

Terminates a FOR loop (see FOR). If a variable is specified it must

match the one in the corresponding FOR statement.

NEXT can only be used within a program (non-interactive).

10 REM FOR/NEXT demo

20 FOR i=1 TO 100 STEP 2

30 PRINT I

40 NEXT

>>> OFF value

Turns OFF digital I/O devices connected to the parallel port. All signals

corresponding to the 1 bits in the operand are deactivated.

>>> ON value

Turns ON digital I/O devices connected to the parallel port. All signals

corresponding to the 1 bits in the operand are activated.

.P

>>> OPEN#unit,"filename","options"

Opens a file for input or output using the specified unit number. The

"options" available are:

.f-

a   - Append: Append when WRITEing

r   - Read: Open file for READ

w   - Write: Open file for WRITE

v   - Verbose: Display message if error

q   - Quit: Terminate program if error

b   - Binary: Do not perform CRLF translation

.f+

10 REM OPEN/PRINT to file

20 OPEN#1,"test.fil","w"

30 PRINT#1,"This is a test"

40 CLOSE#1

50 REM OPEN/INPUT from file

60 OPEN#2,"test.fil","r"

70 INPUT#2,A$

80 PRINT "A$=",A$

90 CLOSE#2

>>> ORDER line

Positions the DATA/READ pointer to a specific line. The line MUST begin

with a DATA statement. This command must be given before a READ statement

can be used at the beginning of a program, and again after all data has

been read from a data block.

Once the data pointer has been set, READ will automatically accept data

from multiple DATA statements, but it will not automatically skip over

non-DATA statements.

10 REM ORDER/DATA demo

20 ORDER 30

30 DATA 1,9,2,8,3

40 DATA 7,4,6,5,0

50 PRINT "Nothing much happening yet"

>>> OSET value

Sets the parallel I/O bits (/P) to the specified value. All signals

corresponding to 1 bits in the operand are activated, and all signals

corresponding to 0 bits in the operand are deactivated.

>>> OUT port,value

Outputs a value to the specified 80x86 I/O port. This is used to

directly manipulate hardware installed in the PC which is not supported

by @0.

10 REM Zero LPT1 data port

20 OUT 956,0

.P

>>> PLAY "filename" [,button_mask [,marker]]

Plays the indicated file (".VOC" extension assumed) through a Sound Blaster

card.

If the button_mask parameter is specified, it identifies keypad buttons

which are allowed to interrupt the file output. Add up the button values

from the following table:

.f-

Button1     1       <- Allow button1

Button2     2       <- Allow button2

Button3     4       <- Allow button3

Button4     8       <- Allow button4

Wait        256     <- Wait for a button before proceeding *

Immediate   512     <- Don't sound tone/wait till release

.f+

* The "Wait" feature will cause playback to pause after one block until

an allowed key is pressed, after which playback will terminate.

There are two special "button_mask" values: 0 invokes "sequence" mode,

which uses embedded markers in the file (1-4) to stop and wait for a

specific keypad button to be pressed before playback proceeds. A mask

value of 16 plays the file without allowing it to be interrupted by the

buttons (default).

The marker parameter specifies a .VOC file marker at which to begin

playback. Playing will continue until another marker (or the end of the

file) is encountered. The default value (0) of this parameter is to play

the entire file. You cannot specify a marker parameter without also

giving a button_mask parameter.

10 REM PLAY demo

20 PLAY "story1",0

>>> PRINT expression[,expression][,]

Prints the values of expressions on the console. Either numeric values or

character strings may be printed. Unless a trailing ',' is supplied, PRINT

will output a Line-feed/Carriage-return to reposition output to a new line

after the expressions are printed.

NOTE: Print automatically preceeds numbers with a space, however it does

no such special processing for character strings. The STR$ function can be

used to convert a number into a string and avoid this space.

10 REM PRINT demo

20 FOR i=0 TO 10

30 PRINT "I=",i,"!"

40 PRINT "I=",STR$(i),"!"

50 NEXT I

.P

>>> PRINT#unit,expression[,expression],

Behaves exactly like the PRINT command above except that the output is

directed to a previously opened file.

10 REM PRINT#unit demo

20 OPEN#1,"TEST.FIL","w"

30 FOR i=0 TO 10

40 PRINT#1,"I=",I

50 NEXT I

60 CLOSE#1

>>> QDOS "command"

Quick DOS command. This is the same as 'DOS', except that it is faster to

execute, but reduces the memory available to the DOS command by more than

128K. QDOS is normally used for small utilities and DOS internal commands.

10 REM Perform a directory listing of the directory in A$

20 QDOS "DIR "+A$

>>> READ variable[,variable ...]

Reads data into a variable from DATA statements in the program. An

ORDER statement must be used to set the data pointer before READ can

be used.

10 REM ORDER/READ/DATA demo

20 ORDER 60

30 READ A

40 PRINT "A=",A

50 IF A <> 0 THEN 30

60 DATA 1,9,2,8,3

70 DATA 7,4,6,5,0

>>> RETURN

Returns program execution from a subroutine to the statement immediately

following the GOSUB that invoked the subroutine.

RETURN can only be used within a program (non-interactive).

10 REM Call a subroutine

20 PRINT "start"

30 GOSUB 80

40 PRINT "middle"

50 GOSUB 80

60 PRINT "end"

70 END

80 PRINT "subroutine"

90 RETURN

.P

>>> REM

Inserts a comment in the program. The remainder of the line is ignored

by the interpreter. Execution continues on the next line.

10 REM this is a comment

>>> RUN [line]

Starts the program executing. All user variables are cleared first. If a

line number is specified then the program will be started at that line,

otherwise execution begins on the first line of the program.

RUN can only be used from the interactive command line (non-program)

RUN

>>> SAVE ["name"]

Saves the program in the specified file. The extension ".BAS" is assumed.

If no filename is given, then it's name from the last LOAD or SAVE command

issued is assumed.

SAVE can only be used from the interactive command line (non-program).

SAVE "test"

>>> SAY expression[,expression ...]

Speaks the values of the expressions using the SBTALKER text-to-speech

driver, which must be loaded. Note that the VOICE command must be issued

prior to SAY in order to initialize the text-to-speech engine.

10 VOICE 0,5,5,5

20 SAY "Hello ",N$," Welcome to CAM"

>>> SCREEN "filename"

Loads the video text buffer with the contents of the file. This may be

used to rapidly display previously prepared "slides". Video images can

be created and edited with the stand alone screen utility.

10 REM Load a "slide" screen

20 SCREEN "SLIDE1.SCR"

.P

>>> SOUND frequency

Generates a tone of the specified frequency via the PC speaker. The tone

will continue until it is explicitly turned off, or another tone is

generated. A frequency value of zero turns off the tone.

10 Generate a 1000hz tone while running a subroutine

20 SOUND 1000

30 GOSUB 1000

40 SOUND 0

>>> STOP

Terminate execution of the program with an informational message.

10 REM End program with an announcement

20 STOP

>>> TUNE tune

Plays a selected short tune (0-6) via the PC speaker. These tunes are of

varying length, and as useful as "fillers" when waiting for some other

operation. The following tunes are currently defined:

.f-

#   Name                    Approx time (sec)

---------------------------------------------

0   Simple Up Scale             2

1   Simple Down Scale           2

2   Misc. melody #1             2

3   Misc. melody #2             4

4   Mary had a little lamb      8

5   This old man                10

6   Twinkle-Twinkle             12

7   Ittsy-Bitsy-Spider          15

8   Doe-A-Deer                  23

.f+

10 REM Play a little tune

20 TUNE 1

>>> VOICE tone,volume,pitch,speed

Initializes the SBTALKER text-to-speech engine (which must be loaded prior

to starting @0). The operand values and allowable ranges are: (tone:

0=Bass/1=Treble, volume: 0-9, pitch: 0-9, speed: 0-9)

10 VOICE 0,5,5,5

20 SAY "Hello ",N$," Welcome to CAM"

.P

>>> MIX <output>,<left-level>[,<right-level>]

Control the output volume levels of the Sound Blaster 2.0, Pro and 16

mixer hardware. <output> indicates the output channel to adjust:

0 = Master volume

1 = Voice (digital sound) level

2 = MIDI (digital music) level

3 = CD (CD audio) level

4 = LINE level (SB-PRO and SB16 only)

5 = MIKE level (SB-PRO and SB16 only)

The <level> values range from 0 (off) to 31 (full volume). Note that the

actual number of separate volume steps varies with the sound card hardware.

When an output has fewer than 32 physical settings, the 0-31 values are

scaled to cover the physical range of the card (ie: 0 is always off, 31

is always maximum).

.f-

SB2.0   = Outputs 0-3 have 8 steps, output 4 has 4 steps.

SBPRO   = Outputs 0-4 have 8 steps, output 5 has 4 steps

SB16    = All outputs have 32 steps

.f+

If only one level value is supplied, BOTH channels (left & right) of the

selected output are set to that level.

The microphone output (5) is mono for all cards, and is set to the

<left-level> value. If a <right-level> value is supplied, it is ignored.

Sound Blaster 2.0 cards have only a mono mixer, and therefore always

adjust both channels of ALL outputs to the supplied <left-level> only.

Additional mixer features available on some cards may be accessed via the

DOS/QDOS commands by using the command line mixer program included with the

card (SBP-MIX, SB16MIX etc.)

.P

>> Error Messages

The following error messages are produced by @0. If the error occurs

during the execution of a program, it will be followed by "in line n"

where n is the offending line unmber.

>>> Illegal program error

Results from an attempt to use a statement within a program which is

only allowed as an interactive command.

>>> Illegal direct error

Results from an attempt to use a statement as an interactve command

which is only available within a program.

>>> Syntax error

Results from any statement which does not follow the @0 syntax rules.

>>> Line number error

Results from an attempt to reference a line number which does not exist

>>> Wrong type error

Results from an attempt to use a value in a context where a different type

(number or string) is expected.

>>> Divide by zero error

Results from an attempt to perform division by zero.

>>> Nesting error

Results from incorrect nesting of FOR/NEXT of GOSUB/RETURN constructs,

or from overflow of the internal control stack which manages these

constructs.

>>> File not open error

Results from an attempt to perform I/O to a unit number that does not

have a corresponding open file.

>>> File already open error

Results from an attempt to open a file using a unit number that already

has a corresponding file opened.

>>> Input error

Results an incorrect numeric entry in response to an input request for

a numeric variable.

.P

>>> Dimension error

Results from an attempt to access a numeric array that has not been defined,

or use of an index value that is outside of the range declared for the

array.

>>> Data error

Results from attempt to READ when the data pointer does not point to a

data statement containing the correct data type for the variable you

are attempting to use.

>>> Out of memory error

Results when @0's available memory pool has been used up. Available

memory is depleted by program lines, character strings, and numeric

arrays.

>>> Out of range error

Results from an operand to TUNE or CONTROL which is larger than the

defined selections for those statements.

>>> Interface error

Results from a failure to communicate with attached hardware.

>> Control-C

@0 uses the keyboard Ctrl-C character (Press 'C' while holding CTRL) to

signal an ABORT condition. @0 will stop it's current activity and return

to the interactive prompt as quickly as possible when this key combination

is detected.

.P

> UTILITY PROGRAMS

>> The RENUMBER utility

In the course of editing and debugging a @0 program, you will find

that the line numbers become unsightly, with gaps where you have

deleted lines, and extra numbers where you have added lines. A major

problem can occur if you "use up" all the line numbers that you allowed

between two lines, and wish to insert another one.

To solve these problems, @0 includes a stand alone RENUMBER utility,

which reads a @0 source file, and writes a new one with all the lines

renumbered in sequence. Line number references in "GOTO", "GOSUB", "ORDER"

and "IF/THEN" statements are adjusted to reference the correct new line

numbers.

The syntax of the RENUMBER utility is:

.c RENUMBER oldfile newfile [start [increment]]

The @0 source is read from 'oldfile', and the renumbered source is

written to 'newfile'. The optional 'start' and 'increment' parameters

allow you to specify the first line number to be assigned, and the

numerical spacing of the new line numbers. These values both default

to 10 (ie: start at line 10, and increment by 10 for each subsequent

line).

Example:

.c RENUMBER PROGRAM.BAS NEWPROG.BAS 1000 100

This reads the source program from PROGRAM.BAS, and writes a renumbered

version to NEWPROG.BAS. The new program will have line numbers starting

at 1000, and incrementing by 100 for every line.

.P

>> The SOUND utility

The SOUND utility is a DOS command line program to record and play .VOC

sound files. It is useful with @0, because it:

- Insures .VOC blocks will be less than 64K.

- Allows .VOC blocks to be split at defined points.

- Allows markers to be inserted into the file.

SOUND is invoked with the following command line:

.c SOUND [/R /Bsize /D /P /Q /Srate /Ttime] filename ...

The '/R' option indicates that the specified files are to be be recorded,

otherwise they are played.

The '/Bsize' option is effective for recording only, and sets the block

size to 'size' (default 65500).

The '/D' option causes SOUND to output debugging messages showing the

status of various internal operations.

The '/P' option causes SOUND to startup in playback or recording PAUSE

mode. This requires you to press a key before proceeding. When recording,

this option can be useful if you wish to begin the .VOC file with a

marker. Ie: Start in recording-pause mode, then press a key to insert

the marker and proceed.

The '/Q' option tells SOUND to be QUIET! This inhibits the normal startup

and progress messages.

The '/S' option is effective for recording only, and sets the sampling

rate to 'rate' (Default 8000).

The '/T' option forces a time limit. SOUND will stop recording or playing

after this number of 1/18th second intervals.

When playing a file, you can press ESC at any time to terminate playback,

or SPACE to pause the output (any other key resumes).

When recording a file, you can press ESC to stop recording, SPACE to pause

recording, ENTER to force a new block to be started, or 1-9 to insert a

marker and begin a new block.

NOTE: Although @0 does not provide it's own recording capability, you

can record sounds from within @0 by using the DOS command and SOUND:

DOS "SOUND /R /T90 recfile":REM Record 5 seconds to RECFILE.VOC

.P

>> The SCREEN utility

The SCREEN utility program is used to generate text screens which can be

rapidly displayed with @0's "SCREEN" command. Note that this does not

provide graphical capabilities, but the use of the PC's graphic character

set can provide quite nice looking effects.

SCREEN in invoked by typeing its name and the name of the screen file you

wish to create/edit as an argument:

.C SCREEN filename

Once activated, SCREEN provides a number of function keys to allow you to

position the cursor and select characters and attributes for insertion into

the screen. A complete list of the keys/commands available can be obtained

by pressing ALT-F1.

>> The IREDIT utility

The IREDIT utility is used to record and modify infrared (I/R) control

codes for use by the "CONTROL" statement. It is described in more detail

in the IR232 documentation file: IR232.TXT

>> The CP290 utility

The CP290 utility provides a means of controlling X10 devices via the X10 -

CP290 computer control interface. CP290 can be used within CAM via the DOS

statement. CP290 allows you to turn any X10 device ON or OFF, as well as

to DIM lamp modules. For a description of the CP290 command, type CP290

with no operands at the DOS prompt.

>> The MICROCAD utility

MICROCAD is a simple VGA resolution drawing program. It is included with

the @0 package, mainly to allow viewing/printing of the schematic drawing

of the IR232 interface, however it can also be used to display or print

graphical drawings from within @0 via the DOS statement.

MICROCAD is described in detail in the documentation file: MICROCAD.TXT

.P

> SAMPLE APPLICATIONS

To help you become familier with the @0 system, we have included a few

simple applications. Note that these do not use all of @0's capabilities,

and as you gain experience with programming the system, you will be able

to make much more comprehensive applications which are taylored to your

childs particular needs.

>> Main Menu

This application (MENU.BAS) presents a menu of the other applications

which can be selected to run. This is the focal point of the example

interface. All of the other example applications exit by reloading this

menu.

Note that this implementation is set up so that an operator selects the

application which will run for the child. It is entirely possible to

design a completely different interface in which the child could select

applications himself. This might be suitable as the child becomes more

familier with the system. The possibilities are endless.

>> Sequence training

This application (SEQUENCE.BAS) contains a list of files that it will

play (in "random order) using the "sequence" mode of the PLAY function.

This means that embedded markers within the files cause the playback

to pause until a specific button is pressed.

Sequence training can appear as several different applications. For

example, a familier story can be recorded which pauses and waits for

a key every few sentences. This provides a simple method to teach the

basic operation of the buttons.

Another possibility is questions and answers. For example: Marker#1

preceeds questions, and marker#2 preceeds answers. Ie: Press button 1

and hear "What does the COW say?", then presses button 2 to hear "Moooo".

Many such questions and answers can be stored in a single file.

Before using this application you must record suitable audio files and

set the filenames in the DATA statements near the end of SEQUENCE.BAS.

>> Story Book

The story book application (STORY.BAS) provides four audio files which

can be played back by pressing a corresponding button. This gives the

child the satisfaction of being able to choose by himself.

Before using this application, you must record suitable audio files and

set the filenames in the DATA statements near the end of STORY.BAS.

.P

>> CD Jukebox

Although originally designed for a 5-disk CD changer, the CD Jukebox

(JUKEBOX.BAS) works equally well with a TV, VCR or other IR controlled

device. This application is essentially a four button remote control

which can be programmed to activate any number of functions with the

push of a single button.

This application REQUIRES the IR232 remove control device to be

operational.

Before using this application, you must use the IREDIT utility to

record suitable control codes for the device in question, and set

the control codes in the DATA statements near the end of JUKEBOX.BAS.

>> Joystick Beeper

Although originally written as a test program, the joystick beeper

(JBEEP.BAS) can be entertaining to children. It simply provides a

variable tone and bar-graph which go up and down with the position

of the analog input #1.

>> Circle tray

In this application (CIRCLE.BAS), the input buttons are replaced with a

wooden tray containing up to four large circles, each fitted into a hole

in the tray. Handles on the top of the tray allow the child to easily

remove and replace them. Magnetic "reed" switches of the type used in

security systems close the button contacts when a circle is inserted

into the tray.

When run, this application simply informs the child when a circle has been

successfully inserted or removed, and then announces the number of circles

currently in the tray.

>> Note about voice files

We have not included pre-recorded voice ".VOC" files with the system, for

two reasons:

A) The child will be most comfortable with the system if the voice

messages are recorded in a familier voice (Mother, Father, Teacher etc.)

B) .VOC files are lengthy, and could make the download image unacceptably

large.

Before running the sample applications which use voice files, the user

must first record suitable files. See the file VOCFILES.TXT for a summary

of the required files and their content.

NOTE: To increase the response time in playing voice files on older (slower)

systems, try using FASTOPEN and/or increasing the BUFFERS= settings in your

CONFIG.SYS file.

.P

> PC CONNECTIONS AND PINOUTS

This section shows the connection pinouts and signal names for the common

PC interfaces.

All connectors are shows as viewed when looking at the rear of the PC.

.f-

>> Game connector pin assignments

+-------------------------------+

| 8   7   6   5   4   3   2   1 |

\ 15  14  13  12  11  10   9  /

+---------------------------+

1 Game1 +5V           9 Game2 +5V

2 Game1 switch 1     10 Game2 switch 1

3 Game1 X position   11 Game2 X position

4 Game1 Ground       12 Game2 Ground

5 Game1 Ground       13 Game2 Y position

6 Game1 Y position   14 Game2 Switch2

7 Game1 switch 2     15 No connection

8 No connection

>> Printer connector pin assignments

+----------------------------------------------------+

| 13  12  11  10   9   8   7   6   5   4   3   2   1 |

\  25  24  23  22  21  20  19  18  17  16  15  14  /

+------------------------------------------------+

1 -STROBE        * 10 -ACK

2 DATA 0         * 11 +BUSY

3 DATA 1         * 12 +PAPER OUT

4 DATA 2         * 13 +SELECT

5 DATA 3           14 -AUTO FD

6 DATA 4         * 15 -ERROR

7 DATA 5           16 -INIT

8 DATA 6           17 -SELECT INPUT

9 DATA 7        18-26 GROUND

Signals marked with '*' originate from the printer (inputs).

.P

>> Serial connector pin assignments

PC (25 pin)                              AT (9 pin)

+---------------------------------------------------+ +-------------------+

| 1   2   3   4   5   6   7   8   9  10  11  12  13 | | 1   2   3   4   5 |

\ 14  15  16  17  18  19  20  21  22  23  24  25  /   \  6   7   8   9  /

+-----------------------------------------------+     +---------------+

Signal name               Origin   25-Pin(PC)  9-Pin(AT)

--------------------------------------------------------

Protective Ground   (GND)   ---         1          1

Transmit Data       (TXD)   DTE         2          3

Receive Data        (RXD)   DCE         3          2

Request To Send     (RTS)   DTE         4          7

Clear To Send       (CTS)   DCE         5          8

Data Set Ready      (DSR)   DCE         6          6

Signal Ground       (GND)   ---         7          5

Data Carrier Detect (DCD)   DCE         8          1

Data Terminal Ready (DTR)   DTE        20          4

Ring Indicator      (RI )   DCE        22          9

>> Keyboard connector pin assignments

PC/AT DIN-5 connector

___ ___

/   U   \     1 +Clock

/           \   2 +Data

|             |  3 -Reset

|  3       1  |  4 Ground

\  5     4  /   5 +5 Vdg

\   2   /

-----

PS/2 minature DIN-6 connector

__ __

/ 6 U 5 \     1 Data

/           \   2 Reserved

|  4       3  |  3 Ground

\           /   4 +5 Vdc

\ 2   1 /     5 Clock

-----       6 Reserved

>> Video connector pin assignments

EGA/CGA/MDA Connector

+-------------------+   1 Ground         6 Secondary Green/Mono intensity

| 5   4   3   2   1 |   2 Secondary RED  7 Secondary Blue/Mono video

\  9   8   7   6  /