.T ITS

.=M4

.S20

.d+

.C Interactive Terminal Switch

.d-

.s10

.C Dave Dunfield

.C Aug 16, 1987

.P

> INTERACTIVE TERMINAL SWITCH

This document describes the operation of the Interactive Terminal Switch

as would be seen by a user sitting at a terminal.

>> Starting a Session

To access ITS from a terminal, an attention character must first be

entered from that terminal. The attention character for a given terminal

may be defined by the switch administrator as any character, but is

usually a carriage return.

Once the attention character is entered, a connect message will be

displayed, and ITS will wait for your commands with a '>' prompt.

If this does not happen, it may be for one of the following reasons:

.i+3

.i-3 1) You are using the wrong attention character, verify that the

switch is expecting the same attention character that you are using.

.i-3 2) The terminal is set to the wrong baud rate or parity, check

the settings and insure that ITS is set the same as the terminal.

.i-3 3) The terminal port is set for "AUTO" - baud rate detection, if this

is the case, insure that the terminal is set to one of 300, 600, 1200 1800

or 2400 baud, and enter the attention character several (Maximun five)

times. ITS should respond with the 'connected' message.

.i-3 4) The terminal port is busy, check that no other port is connected

to the one you are trying to use.

.i-3 5) The terminal port is not optioned to allow origination of calls

through ITS.

.i-3 6) The terminal port has been disabled.

.i-3 7) The terminal port is not optioned to display network messages.

if this is the case, the '>' prompt will appear without the connect

message.

.i-3 8) The terminal port has a direct call set on it, in which case it

will attempt to establish a call as soon as the attention character is

received, and will never enter command mode.

.i-3 9) The terminal port has a local password. If this is the case,

you will be prompted to enter a password before the connect message

will be issued.

.i-3

If you are unable to get the connected message, and none of the above

seems to be the problem, then the terminal, the cableing, or the switch

itself may be faulty.

.P

The backspace or delete keys on the terminal may be used to correct

typing mistakes on the command line, each time one of the above

keys is pressed, the terminal cursor will backup one character position,

and the last character will be deleted from the command input buffer.

>> Originating a Call

Once a connection has been established with ITS, and the '>'

prompt is visible, you may establish a connection to another ITS

port, simply by typing its name or address.

Each port on ITS is identified by a unique address,

ranging from '00' up to the maximum number of ports on the switch. A port

may be called by address with the '#' character. Ie: '#3' indicates the

port with address '3'.

ITS ports are assigned a name when the system is installed, names

of accessable ports may be seen with the 'DISPLAY' command (See ITS

user commands section). Note that ports which are not accessable to the

port from which the display command is issued WILL NOT BE DISPLAYED.

In order for a port to be accessable by another port, it must be optioned

to accept incomming calls (See 'I' bit in the 'o=' parameter under 'SET'

console command).

If the port being called has been assigned a password, the password may

be specified, seperated from the name or address by a comma.

.C Eg: 'name,password' - '#03,password'

If the password is not given, you will be prompted for it.

Once a port name or address has been entered, ITS will attempt to

establish the call and respond with a 'Call connected' message, or a

message indicating why the call could not be established. (See ITS

network messages section).

.P

>> User Commands

In addition to entering a port name or address, ITS accepts

the following commands in response to the '>' prompt. Capital letters

indicate the minimum abbreviation for each command.

The following commands may be entered from any port:

.i+3

CLear - CLEAR CALL AND DISCONNECT

This command clears the call, and disconnects the terminal from the

switch. If no call is active, then terminal is simply disconnected.

DIsplay - DISPLAY ACCESSABLE PORTS

The DISPLAY command provides a list of the port names and addresses

of all ports on the switch which may be connected to by the terminal from

which the DISPLAY is issued. A status indication is also provided, indicating

the call processing state of that port.

HElp - DISPLAY HELP SUMMARY

This command outputs a brief summary of all commands which will be

accepted at the time that the HELP command is issued.

HOld - PLACE CALL ON HOLD

The HOLD command causes your call to be placed on hold, allowing you to

connect to another port. You may reconnect to the original port by using the

'RETrieve <port>' command. No other port may retreive your held call, and the

password (if any) is not required.

PArk - PARK CALL

This command "parks" your call, such that someone else at another terminal

may "retreive" the call without disconnecting the remote end. If the port has a

'R'emote password defined, it will be required to retreive the call.

RELease - RELEASE CALL

This command releases the connection to the remote port, remaining at

the '>' prompt, allowing you to immediately setup another call.

.P

RESume - RESUME CALL

The RESUME command returns to data transfer mode, after a call has

been interrupted with the ESCAPE TO COMMAND MODE data transfer command.

RETreive - RETREIVE A HELD OR PARKED CALL

The RETREIVE command will connect you to a call which has been previously

placed on hold by you, or parked by another MainStreet user.

STatus - DISPLAY PORT STATUS

This command displays the setup configuration and call state for the

port from which it is issued.

.i-3

The following commands are restricted, and may only be used from ports

which are optioned to use them:

.I+3

COnnect <port1> <port2>  - CONNECT THIRD PARTY CALL

This command establishes a third party connection between the

specified ports. The first port specified becomes the originator of the

call, and must be optioned to originate calls. The second port specified

becomes the call receiver, and must be optioned for incomming calls.

If the ports to be connected require passwords, then the passwords may

be specified, following each port name, separated by commas.

.C EG: 'connect port1,password1 port2,password2'

If the passwords are not supplied, you will be prompted for them.

DISConnect <port> - DISCONNECT THIRD PARTY CALL

The DISCONNECT command terminates a third party call. The port

specified may be either of the ports involved in the call. Only the

port which connected a third party call may disconnect it.

MEssage <port> <message text> - SEND MESSAGE

This command sends the given message text to the specified port.

The port may be specified by name or address. The message is prefixed

the name of the switch, followed by the name of the originating port in

brackets, and may only be sent to ports which have been optioned to receive

messages.

.I-3

.P

>> Data Transfer Commands

When a call is established, ITS is said to be in 'data transfer mode'.

While in this state, ITS transfers all data to and from the destination,

except for a character called the ESCAPE character, which when entered

from the port originating a call, indicates to ITS that a single

character data transfer mode command is to follow.

The escape character for a given terminal may be defined by the

switch administrator as any character, but is usually a DATA LINK ESCAPE

(DLE) character, which may be entered from a terminal as a CONTROL P

(Press 'P' while holding down the CONTROL key). This escape character

should not be confused with the ESCAPE key found on many terminal

keyboards, as they are NOT the same.

Note that if a call is made to a port which is optioned to receive

transparent data, the ESCAPE character will be passed through to the

destination, and none of the data transfer mode commands may be used.

The following data transfer mode commands are supported:

.i+3

? - HELP SUMMARY

This data transfer command displays a short summary of the available

data transfer commands.

<cr> - ESCAPE TO COMMAND MODE

This command causes ITS to exit data transfer mode, and return to

command mode, prompting with the '>' prompt. The call is not cleared,

and may be reentered with the 'RESUME' command.

B - SEND BREAK

This command sends a break character to the destination.

C - CLEAR CALL

This command clears the call, and disconnects the terminal from

the switch as per the "CLEAR" user command.

H - HOLD CALL

This command places the call on hold as per the "HOLD" user command.

P - PARK CALL

This command parks the call as per the "PARK" user command.

Q - SEND XON (CONTROL-Q)

This command sends an XON ($11 or '^Q') through to the destination.

R - RELEASE CALL

This command releases the call, returning to the '>' prompt as per

the "RELEASE" user command.

.P

S - SEND XOFF (CONTROL-S)

This command sends an XOFF ($13 or '^S') through to the destination.

T - SWITCH REMOTE TO TRANSPARENT

This command causes the port at the other end of the call to act as if

it had the 'T' option bit set (see console 'SET' command) for the duration

of the call only.

U - SWITCH LOCAL TO TRANSPARENT

This command causes your port to act as if it had the 'T' option bit set

(see console 'SET' command) for the duration of the call only.

.i-3

The following data transfer mode commands apply only to calls to ports

which are optioned for automatic baud rate selection. They will have no

effect on other ports:

.i+3

1 - SELECT 300 BAUD

This command causes the outgoing port to switch to 300 baud.

2 - SELECT 600 BAUD

This command causes the outgoing port to switch to 600 baud.

3 - SELECT 1200 BAUD

This command causes the outgoing port to switch to 1200 baud.

4 - SELECT 1800 BAUD

This command causes the outgoing port to switch to 1800 baud.

5 - SELECT 2400 BAUD

This command causes the outgoing port to switch to 2400 baud.

.i-3

Any other (non-command) characters which are entered following the

escape character are passed directly through to the destination. This

provides a method of sending the escape character through (by entering

it twice in succession).

.P

>> Network Messages

All ITS status or error messages to the user are called NETWORK

messages. NETWORK messages are easily identified because they are always

prefixed by the name given to the ITS switch when it is installed.

The following network messages are issued by ITS:

.i+5

.i-3 Permission denied.

Your command or connect request has been rejected by the switch for

one of the following reasons:

.i+3

.i-3 1) You have attempted to use a command which your port is not

privileged to use.

.i-3 2) You have not supplied the correct password.

.i-3 3) Destination does not receive calls.

.i-3 4) You have attempted to connect to an internal process, and your

port is not privileged to do so.

.i-3 5) You have attempted to connect a port to itself.

.i-3

.i-3 Call already active with <nn>.

You have attempted to connect to a port, when you already have a call

in progress with port <nn>.

.i-3 Call connected.

ITS has established a connection with the requested port, and you are

now in data transfer mode, sending and receiving data to and from that port.

.i-3 Connected, Port: <nn>

ITS has initiated a session with your terminal, which is connected

through the port address shown.

.i-3 Connected to <nn>, owned by <oo>

ITS has connected this port to port <nn>, as a result of a third party

connect request initiated by port <oo>.

.P

.i-3 Disconnecting.

ITS is terminating your terminal session.

.i-3 Destination busy.

The port you have called is busy, and therefore your call could not

be established.

.i-3 Queued at position <n>

The port you have called is busy, and since your port is optioned to

queue on busy calls, ITS will connect you as soon as it becomes

available. The number <n> indicates at what postion in the queue you

have been entered. Wheneved <n> changes, ITS will output a new

message, indicating the new queue position. Entering a carriage return

will terminate the queueing operation.

.i-3 Destination has disconnected.

Your call is being terminated, because the destination port has

gone not ready, powered off, or has been physically disconnected from

the switch.

.i-3 Destination off-line.

The port you have called is not available, because the device attached

to it is either not ready, or not powered on.

.i-3 Destination out of service.

The port you have called is not available, because it has been disabled

by the administrator of the ITS switch.

.i-3 Timeout expired.

ITS has timed out, while waiting for you to enter a command, set up

a call, or transfer data through the switch.

.i-3 No active call.

You have attempted to RESUME, RELEASE, PARK or HOLD to a call, when no

call has been established.

.i-3 Unknown port.

The port name or address you have entered does not correspond to any

of the ports on the ITS switch.

.i-3 Password?

The switch is requesting a password which must be supplied before

the requested operation can be performed.

.P

.i-3 Call Cleared.

Your call has been cleared, and you are no longer connected to the

remote port.

.i-3 Call to <nn> held.

Your call to port <nn> has been placed on hold.

.i-3 Call to <nn> parked.

Your call to port <nn> has been parked.

.i-3 No held call.

You have attempted to RETREIVE a port which was not on hold by you or parked.

.i-3 Warning - Unresolvable speed mismatch.

You have established a connection between to ports operating at different

speeds, and the higher speed port does not support flow control. The call

may proceed, however, data may be lost if the higher speed port sends

characters too quickly.

.i-3 Network is busy - try again later.

The ITS you are attempting to access is operating at its load

capacity, and is unable to service your connect request.

.i-7

.P

> INTERNAL PROCESSES

In addition to other physical ports, a port can be connected to a

program running within the ITS switch. Addresses above '#90' are reserved

for these "internal" processes, which do not have names assigned.

Some of the internal processes are used to control and maintain the

switch, and have the capacity to severly undermine the operation of the

switch if used incorrectly. For this reason, a port must have the 'N'

(Network process access) option bit in order to have access to them.

The currently implemented internal processes are:

>> Test Process - #90

The TEST process provides a few simple test functions which are useful

when installing devices on the switch. It prompts with 'Test>', and accepts

the following commands:

.i+3

CHaracters - DISPLAY CHARACTER SET

This command provides a short display of all printable ASCII characters.

CLear - CLEAR CALL

This command clears your call, and disconnects from the switch.

Echo - BEGIN ECHO TEST

This command excutes the ECHO test, which echo's any character sent to

the switch back to your port. Press <ESCAPE> to exit.

Help - HELP REQUEST

This command displays a short summary of the available TEST commands.

Loadgen - BEGIN LOAD GENERATOR

This command continuously displays lines of data contains the ASCII

characters from '0' to '}'. Press <ESCAPE> to exit.

Release - RELEASE CALL

This command clears your call to the TEST process, and returns you

to call setup.

.i-3

.P

>> Display Process - #91

The DISPLAY process maintains an up to date status display of all ports

on the switch. This display includes the port states, which are updated

whenever a state change occurs.

Pressing the <space> bar will refresh the screen, press the <escape>

key to exit.

In order to access this process, the terminal must have a terminal type

defined (See 'T=' option to SET).

>> Console Process - #92

This process allows control and maintainence of the ITS data

controller through interactive commands.

.P

> ITS CONSOLE PROCESS

>> Console Commands

The following commands are accepted by the ITS console process:

Capital letters indicate minimum abbreviation for a command.

.i+3

CAlls - DISPLAY ACTIVE CALLS

The CALLS command produces a display of all the calls which are

currently underway on the switch. The addresses of both the originating

and called ports are displayed. If a port is connected to ITS command

mode but has not established a call yet, the display indicates 'Call setup'

Third party calls also display an OWNER field, which indicates the port which

connected the call.

CLear - CLEAR CONSOLE CALL AND DISCONNECT TERMINAL

This command terminates the console session.

DISplay <port>[,port] - DISPLAY PORT STATUS

The DISPLAY command produces a status display of the specified port or

range of ports. This display includes address, name, options (see SET

command), attention character, escape character, messaging format, data

format, baud rate, call state, automatic call destination (if present),

Idle call timeout (if present), and password protection indications. In

addition, if <port> is specified as 'ALL', display will produce a display of

all ports on the switch, and their respective states. If <port> is specified

as 'ACTIVE', display will display all ports which are not IDLE, OFFLINE, or

DISABLED.

DISAble <port>[,port] - REMOVE A PORT FROM SERVICE

This command disables a port or range of ports, preventing it from

originating or receiving calls. If a call is curently in progress on the

port, it is terminated.

ENable <port>[,port] - RESTORE A PORT TO SERVICE

This command cancels the effect of the DISABLE command on a port or

range of ports which has been disabled. The call is not restored, the port

is set IDLE.

EXamine <memory page> - EXAMINE A PAGE OF MEMORY

The EXAMINE comand displays the indicated page of main memory (256 bytes).

The operand value must be in the range of 00 to FF hex. Memory is displayed

in both hex and ASCII. This command if intended for use only by persons

who are familiar with the internal operation of the switch, and should not

be used under normal operating conditions.

.P

HElp - Display help summary.

Displays a short summary of the console commands.

MEssage <port>[,port] <message text>

Sends a message to the specified range of ports, as per the user MESSAGE

command. In addition, if <port> is specified as 'ALL' then message is sent

to all ports. If <port> is specified as 'ACTIVE' then message is sent to all

active ports.

.i+6

.i-6NOTE: Ports with neither the 'M' or 'B' options will not receive messages

sent to 'ALL' or 'ACTIVE'.

.i-6

MOnitor <port>

This command monitors (displays) all data that is being received by the

specified port. Such data may only be monitored when the port is actually

involved in a call. To stop monitoring, press the <escape> key.

The monitor command will not affect the call being monitored in any way,

which means that if the monitoring port asserts flow control, or is at a

lower speed than the port being monitored, the buffer may wrap around, and

some data may be lost to the monitoring port.

NAme <new name>

The NAME command changes the network name, which is displayed at the

beginning of all network messages.

Ports [number] - DISPLAY/SET NUMBER OF ACTIVE PORTS

This command will display the number of ports which may be used on the

switch if no operand is given. If given an operand, it sets the

number of ports scanned by the switch to that value.

PAssword <proc> <new password>

The PASSWORD command changes the password to the specified internal

process.

RESET - RESET DATA SWITCH

The RESET command re-initializes the data switch. All calls are cleared,

all ports are set to IDLE status, and the startup diagnostics are run. This

is the same effect as powering the unit off and then on again.

.P

SET <port>[,port] <configuration elements>

The SET command sets or changes the charactistics of the specified range

of ports. If only one port is specified, that port only is changed.

A configuration element can be removed by specifing it without a value. For

example, to remove a remoite password from a port, use 'set <port> r='.

Multiple elements may be specified, and are applied to the port in the order

that they occur in the command line (important for the 'c=' element).

The following elements may be specified:

.i+5

.i-3 A=[character]  - SET ATTENTION CHARACTER

This element sets the attention character for the port. The

character may be specified as a hexidecimal value, an ASCII character

preceded by a single quote, or a CONTROL code using the '^' symbol.

.i-3 C=<port>  - COPY PORT OPTIONS

This element causes all configuration elements except the name to be

copied from the specified port. Any configuration changes except for 'N='

which are specified BEFORE a 'C=' element will have NO EFFECT, as that

information will be overriden when the configuration is copied. Configuration

elements which are specified AFTER the 'C=' will be performed after the

copy operation, and hence will remain in effect.

.i-3 D=[port]  - DEDICATED CONNECTION

This element sets an automatic call destination for a port. Once this

option is set on a port, that port is prevented from entering command mode,

and will attempt to call this specified port whenever it initiates a session

with the switch (attention character received). If the called port has a

password it is NOT required for an automatic connect.

.i-3 E=[character]  - SET ESCAPE CHARACTER

The 'E=' element sets the escape character for a port, which precedes

all data transfer mode commands.

.i-3 F=<number of data bits><parity>[number of stop bits]

The 'F=' configuration element sets the data format for the

port. The number of data bits may be one of 5, 6, 7, or 8. The parity may be

specified as (M)ark, (S)pace, (E)ven, (O)dd, or (N)one. The number of stop

bits may be specified as 1 or 2, and defaults to one if not specified. If

5 bits, no parity, and 2 stop bits are specified ('F=5N2'), then 1.5 stop

bits are actually used, to be compatable with industry 5 bit commuinications

standards.

.P

.i-3 I=[minites][,seconds]  - SET INACTIVITY TIMEOUT

This element allows you to specify a timeout value of from one to

1092 minites, which informs ITS that any call involving this port

which excedes the specified period of time without any data transfer is

to be cleared.

.i-3 K=[minimum][,maximum] - SET DISCONNECT TIMING

This element allows you to specify the minimum and maximum time that

DTR/DSR from the switch will drop when performing a disconnect. The time

is specified in 1/2 second increments. When the switch disconnects, It

will drop DTR/DSR for the minimum time specified, and complete the disconnect

when either the maximum time specified has elapsed, or the device responds

by dropping its DSR/DTR line. The state of DTR/DSR from the switch will then

be set according to the setting of the 'S'ignal option bit.

.i-3 L=[password] - SET LOCAL PASSWORD

This element sets the password which is required whenever a terminal or

device attempts to initiate a session with the switch through this

port.

.i-3 M=[character] - SET MESSAGE CODE PREFIX

This configuration element sets the prefix character which is output

immediately preceding coded network messaes. When this option has

been specified, network messages are output as a single character, in a

range beginning with 'A'. This two character (prefix) & (code) format

of network messages provide's an easy interface for automatic recognition

and interpretation of switch responses when using computer controlled

call setup. If no operand is specified, standard text messages will be

produced.

.i-3 N=<name>  - SET PORT NAME

This configuration element sets the name of the specified port, the

name must be a maximum of 12 characters long.

.P

.i-3 O=[+|-]<option bits>

The 'O=' element sets the port option bits to the specfied value.

Option bits may be specified in any order. The available option bits are

as follows:

.i+3

.i-3 B - BROADCAST MESSAGES

Enables the port to send and receive messages.

.i-3 C - CONNECT THIRD PARTY

Enables the port to connect and disconnect third party calls.

.i-3 D - DTR FLOW CONTROL (Used for some printers)

Indicates to ITS that this port supports DTR flow control, and therefore

may be called by ports of a different speed. Note that if this bit is not

set on a port, and that port drops DTR, ITS will assume that the device

attached to that port has gone off-line, and will clear the call.

.i-3 E - ECHO COMMAND INPUT (For full duplex terminals)

Causes ITS to echo each character that is typed in command mode.

.i-3 H - HUNT TO NEXT PORT

Causes ITS to try the next sequential port if this port is called and

is busy or disabled.

.i-3 I - ACCEPT INCOMMING CALLS

Allows this port to be called by other ports.

.I-3 M - DISPLAY NETWORK MESSAGES

Enables the display of network messages on this port. Network messages

are all messages which are preceded by the switch name.

.I-3 N - ALLOW CONNECT TO NETWORK PROCESSES

Enables this port to connect to ITS internal network processes.

.I-3 O - ORIGINATE CALLS

Allows this port to call other ports.

.i-3 P - SUPPRESS PASSWORD CHECKING

Allows this port to connect to password protected ports without specifing

the password.

.P

.I-3 Q - QUEUE ON BUSY

Causes ITS to queue the call request when a call is made to a busy port

from this port.

.i-3 S - SIGNAL

When this option is set, ITS indicates that a port is idle by dropping

the DTR/DSR signal lead on the interface. The DTR/DSR lead will be

asserted any time that a session is initiated by the port, or a call

is connected to the port. If the 'S' bit is NOT set, DTR/DSR from the switch

will be asserted at all times except during DISCONNECTION. If the 'S' signal

option is set on a port that originates calls, the 'W' wired connection must

be used to initiate a session with the switch. The attention character will

NOT be detected.

.i-3 T - TRANSPARENT (Used for graphic printers etc)

Indicates to ITS that this port is to receive all data exactly as it

is received by ITS at the other end of the call. Only the ESCAPE TO

COMMAND MODE sequence is recognized. All other characters INCLUDING

XON/XOFF FLOW CONTROL CODES are passed through.

.i-3 W - WIRED CONNECTION

When this bit is set, ITS attempts to initiate a session with this

port whenever it detects a OFF to ON transition of the DTR/DSR signal

lead on the interface. This options is normally used in conjuction with

the 'D=' dedicated connection feature.

.i-3 X - SUPPORTS XON/XOFF FLOW CONTROL

Informs ITS that this port supports XON/XOFF flow control, and may be

called by ports of different speeds. Note that if both the 'X' and 'D'

bits are set, ITS will accept either form of flow control, but will use

XON/XOFF to flow control data coming in from this port.

.i-3

.I-3 P=[value] - SET DISPLAY PAGE SIZE

If this element is set, ITS will stop after outputing the

specified number of lines, and wait for a character before proceding.

.i-3 R=[password]  - SET REMOTE PASSWORD

This element sets the password required to access the port

through the switch.

.P

.i-3 S=<speed>  - SET PORT SPEED

The 'S=' element sets the speed of the port. The speed values which

may be used are: 50, 75, 110, 134.5, 200, 300, 600, 1800, 2400, 3600,

4800, 7200, 9600, 19200. 'AUTO/<default>' may be used

for automatic baud rate selection of 300, 600, 1200, 1800 or 2400 baud

(<default> selects the default outgoing speed, and must be one of the above).

.i-3 T=[tty type]

This element defines the type of terminal display which is attached.

The currently implemented terminal types are: VT100, VT52, VC4404 and

ADM3A.

.i-3 U=[character]  - SET UPLINK CHARACTER

The 'U=' element defines a character which is sent to the device

attached to this port, whenever a call is connected to this port. This

element is usually used in inter-switch connections to automatically

bring up command mode on the remote switch.

.i-3 W=<value> - SET FLOW CONTROL WINDOW

This option allows you to specify the number of characters before the

buffer is full at which the switch will assert flow control on the port.

Lower values result in more efficent operation of the switch especially when

handling calls of mismatched speeds, but requires that the attached device

be capable of responding to the flow control within the specified number of

characters.

.i-5

.P

SHOWsys [<port #>]

This command displays the contents of certin internal variables within

the system. If a port# is specified, the system variables associated with the

call processing task for that port are displayed. If no port number is given,

the variables for the system task are displayed. The SHOWSYS command is

intended for use by people familiar with the internal operation of the switch,

and is NOT intended for use in normal operation.

STore <page> <offset> <data1> [data2 ...]

This command stores data into main memory at the specfied page and

offset. The STORE command is intended for use by people familiar with

the internal operation of the switch, and NOT for use in normal

operation. THE SWITCH CONFIGURATION MAY BE CORRUPTED BY IMPROPER USE OF

THIS COMMAND!!!

TEST <starting port>[,ending port]

The TEST command performs loopback testing on one or more ports.

The ports under test must have a loopback connector pluged into them, which

has receive and transmit looped together, as well as DTR and DSR. When this

command is issued, ITS will test the specified ports, and issue error

messages for any faults detected. If a starting and ending port is specified,

ITS will test that range of ports. If only the starting port is

specified, ITS will test that port only. Hit the ESCAPE key to stop

the test.

.i-3

.P

> DEFAULT CONFIGURATION

When ITS is first powered on, it will recognize that it does not

have a valid configuration in RAM, and will generate the following

default configuration:

>> Port Default Configuration

.f-

Port: xx

Name: TTYxx

Options: BCEIMNOX

Atten. character: '^A'

Escape character: '^P'

Uplink character: <none defined>

Messaging format: Text

Display tty type: <none defined>

Display page size: <none defined>

Disconnect Timing: 0,6

Inactivity timeout: <none defined>

Flow control window: 30

Dedicated connection: <none defined>

Data format: 7 bits, Mark parity, 1 stop bit

Speed: 9600

State: IDLE

No local password

No remote password

.f+

>> System Default Configuration

.f-

System name: ITS

Number of ports: 8

Password to #90 test process: <none defined>

Password to #91 display process: 'display'

Password to #92 console process: 'console'

.f+

.P

> CONFIGURATION HINTS

When configurating the individual ports on the switch, it is important

to select the correct options to gain maximum functionality from each port.

>> Option Bits

.i+4

.i-4 B - This bit should be set on all terminal and modem devices which you

wish to allow to send and receive messages to one another. Messages provide

the only method of communicating with a user of another computer. The 'B'

option bit should NOT be set on: Computer ports, printers, modems or ports

used for downloading or any port which may be interfered with by the extra

data resulting from someone sending a message.

.i-4 C - This bit should be set on all terminals modem and computer devices

which are to be allowed to make third party connections. Third party

connections are used for such things as connecting a printer or a modem

to an unused computer port so that it can be accessed.

.i-4 D - This bit should be set on all devices which perform flow control

of data through the DTR bit on the RS-232 port. Although the switch never

sends data to any port if DTR is not present, if the 'D' bit is not set,

the call will be cleared with a 'Destination has disconnected' message

if DTR remains down for longer than a fraction of a  second. The 'D' bit

can also be used to prevent this automatic DTR drop cleardown on ports

using XON/XOFF flow control by setting it along with the 'X' bit.

.i-4 E - This bit should be set on all interactive terminals and other ports

which operate in full duplex. It is normally NOT set on half duplex ports,

and ports which are not used interactivly, such as computer controlled

automatic connections etc.

.i-4 H - This bit is normally set on all but the last of a group of ports

which are connected to the same device (such as a computer). The ports

must be at sequentialy numbered addresses in order for hunting to work.

The LAST port in such a hunt group is not assigned the 'H' bit, so that

hunting will stop there it they are all busy.

When the ports in a hunt

group are not all operating at the same speed, it is usually best to place

the slower ports at the rear (higher addresses) end of the hunt group.

Ports which may also originate calls, such as a port used for printing

should be placed at the very rear of the hunt group, keeping them free

unless all other ports are busy.

The system only checks passwords on the first port called, allowing you

to place passwords (or different passwords) on selected ports at the end

of the hunt group, preventing them from being used unless the preceding ports

are busy.

.i-4 I - This bit should be set an all ports which are to be called from

other ports. It should not be set on interactive terminals etc.

.P

.i-4 M - This bit should be set all all devices from which interactive

connections will be made. It should NOT be set on computers or other

devices which do not need the messages. Note that if a port is not optioned

to originate calls (see 'O' bit), and hence would never see messages anyway,

the 'M' bit in conjunction with the 'B' bit still controls the display of

console broadcast messages on that port.

.i-4 N - This bit should be set on all ports which require access to the

console process. It is very important that this bit not be set on remote

terminals or dial in ports, as this would weaken the security of the

console process. When problems are experienced with a port, that port

can be temporarily assigned the 'N' options, allowing access to the

console and test processes.

.i-4 O - This bit should be set on all ports which have to originate

calls through ITS. The switch will only initiate a session

with a port if the 'O' bit is set.

.i-4 P - This bit should be used with care, as it disables all password

protection (except internal process passwords) on accesses made from a

given port. It has two main purposes: to allow a system administrator

access to all ports without the bother of remembering passwords, and

to allow local terminals access to all resources on a switch, while

restricting remote and dial in ports to access to ports for which passwords

are known.

.i-4 Q - This bit should be set based on the preferences of the

particular user of a port.

.i-4 S - This bit is usually used to inform a device that a call has been

connected to it through the switch, or that its request to initiate a

session has been granted.

.i-4 T - This bit should be set on such devices as graphics printers,

binary download ports, and any other device that uses all possible

character codes.

.i-4 W - This bit is often used in conjuction with the 'D=' dedicated

connection feature to automatically establish a call when a device is

powered on or brought online.

.i-4 X - This bit should be set on all ports which support flow control

via the XON/XOFF (DC1/DC3) characters. The 'D' bit may be also be used

to allow flow control on DTR down as well.

>> Port Speed

The speed of a particular port should be set to the value which is

used on the device connected to it. Where several speeds are possible,

it is generally best to use the fastest speed which is available, up to

9600 baud.

.P

>> Data Format

This should be set to the same values (bits, parity, stop bits)

that is used on the device to connected.

>> Attention Character

The attention character is usually set to a carriage return (0D hex)

on interactive terminal ports. For connections to computers and other

inteligent devices, it is often desirable to use a character that is not

a standard printable character. Ie: computers usually output a carriage return

at the end of any line of text that is printed, and if such output occurs

on the data switch port, it would initiate a session with an attention

character of 0D. For such devices, use a character value that has little

chance of being accidently output.

If a port is given no attention character, but has the 'O' option bit set,

It may only innitiate a call via third party connect, or wired connection

(See the 'W' option bit).

>> Escape Character

The escape character is usually set to a DATA LINK ESCAPE (DLE) character

(10 hex), but can be altered to suit the tastes of the terminal operator. For

computer to computer connections, it may be desirable to use an alternate

escape character, to facilitate the codes which have to be transmitted between

the particular application programs used.

If a port is given no escape character, it will not have access to any of

the data transfer mode commands.

>> Uplink character

The uplink is normally used when switches are networked, on the links

between switches. If such a port has an uplink character defined which is

the same as the attention character of the remote end, the command mode

prompt from the remote switch will come up automatically whenever a call is

connected to this port.

>> Dedicated connection

Dedicated connections are most useful for restricting a particular terminal

or dial in port to connecting to a single port or hunt group. They may also

be useful where a port always connects to only one other port (such as with

computers shareing a common printer), and allow call setup to occur with

only a single character (the attention character) to be transmitted, or

when the device is brought online (See 'W' option bit).

.P

>> Passwords

Local passwords are used mainly to prevent unauthorized persons access to

any of the switch resources. It is usually a good idea to have local passwords

on any remote or dial in ports on the network.

Remote passwords are used mainly to restrict access to a particular port.

by regular users of the switch. See the hints for the 'H' and 'P' option bits

for some other effects that can be created with the use of remote passwords.

Remember, any password system can be broken, and it is advisable that

additional security procedures be in place.

>> Inactivity Timeout

The inactivity timeout provides a method of automaticaly releasing a

resource on the switch when a user has forgotten to. Unless a device is

using DTR flow control, calls will automatically clear when either end

is powered off. However, some devices such as printers and computers

may not be powered off at the end of a call. The inactivity timeout

can be used to clear such calls after the specified number of minites

has elapsed without any data transfer in either direction.

The optimal inactivity timeout value will clear abandoned calls as

quickly as possible, without clearing non-abandoned calls. This value

will depend on the application, and will have to be determined by

experiment.

As a general rule, calls involving users at terminals will need

longer inactivity timeouts that calls originated by intelligent devices.

The inactivity timeout also applies to ports which have been placed

on HOLD, or have been PARKED.

.i-4

.P

>> Code format Messages

For ports connected to terminals, or which otherwise will be

communicating with a human user during call setup, it is recomended

that text messages be used.

For ports from which call setup will be performed automatically

by some sort of computerized equiptment, the code message format of

network messages is recomended. The code message prefix character

should be chosen such that it can be easily recognized by the application,

and not conflict with other data characters and protocols which may

be in use from that device.

Here is the format of the coded messages, and the corresponding

text message which is output by the switch.

.f-

<prefix>A - Connected, Port: <nn>

<prefix>B - Connect to <xx>, owned by <oo>

<prefix>C - Disconnecting.

<prefix>D - Call connected.

<prefix>E - Call cleared.

<prefix>F - Unknown port.

<prefix>G - Destination busy.

<prefix>H - Queued at position <n>

<prefix>I - Destination off-line.

<prefix>J - Destination out of service.

<prefix>K - Destination has disconnected.

<prefix>L - Permission denied.

<prefix>M - Call already active with <xx>.

<prefix>N - No active call.

<prefix>O - Timeout expired.

<prefix>P - Password?

<prefix>Q - Call to <xx> held.

<prefix>R - Call to <xx> parked.

<prefix>S - No held call.

<prefix>T - Warning - Unresolvable speed mismatch

<prefix>U - Network is busy - Try again later.

.f+

.P

> PORT STATES

Each port in the ITS data controller is always said to be

in an internal "STATE". This state determines the capabilities available to

and the resources used by the port. The "STATE" of a port is visible in the

used "DISPLAY" and "STATUS" commands, and in the console "DISPLAY" and "CALLS"

commands. The various states of a port are as follows:

>> Idle

The port is free to originate or receive a call.

>> Offline

The port is IDLE, but the device attached to the port is not powered on,

or is not properly attached to the ITS data controller.

>> Call setup

The port has initiated a session with ITS, or released a previous

call, and has not yet connected to a destination port.

>> Connected to <xx>

The port is involved in a call with the indicated port.

>> Suspended by <xx>

The port is involved in a call with the indicated port, but that port

has temporarily entered command mode using the data transfer mode command.

>> Held by <xx>

The port has been placed on hold by the indicated port.

>> Parked by <xx>

The port has been parked by the indicated port.

>> Queued on <xx>

The port is waiting in the queue for the indicated port.

>> Disconnecting

The port is in the process of disconnecting from ITS. It is

performing the hardware handshakeing nessary to "hang-up" the device

attached to it. Depending on the state of that device, it will soon go

to the "Idle" or "Offline" states.

>> Disabled

The port has been taken out of service via the console process

'DISABLE' command.