.T VT100 - Checkup

.=m5

.s20

.C VT100 - Checkup

.s

.c A demonstration  and

.c checkout program for

.s

.c VT100 (ANSI)

.c TERMINALS

.s5

.c Release 1.0

.s

.c Revised 05-Jan-98

.s10

.c Dunfield Development Systems

.c ----------------------------

.c High quality tools for

.c Embedded Development

.c at low prices.

.s

.c http://www.dunfield.com

.s2

.c ?COPY.TXT 1988-2005 Dave Dunfield

.c **See COPY.TXT**

.P

> INTRODUCTION

"VT100 - Checkup" is a program which is useful for testing a terminal

or terminal emulation software package, and evaluating its conformance

to the ANSI specifications. This is a comprehensive evaluation consisting

of more that 20 individual tests. There are very few terminals or terminal

emulators which pass all tests.

> The "VTEST" command

To begin using "VT100 - Checkup", use the command:

.c VTEST <port> [LOCAL]

where <port> is the communications port (COM1 or COM2) to which you

have the terminal to be tested attached. The LOCAL option tells VTEST

that you wish to control the test from the local PC keyboard, rather

that the terminal under test.

Once VTEST starts up, you will be presented with a menu showing the

(B)audrate, (P)arity, (D)ata bits, and (S)top bits. You may press the

corresponding key ('B', 'P', 'D', or 'S') to cycle through the available

settings for that particular option. Once you have completed this selection,

press <RETURN> to start the test.

VTEST will execute all of the validation tests, pausing between each

one with the prompt:

.c Press <return> to proceed...

At this point, you may either press the <return> key to proceed to the

next test, or the <escape> key to exit VTEST. If the "LOCAL" keyword was

given to the VTEST command, these entries are expected on the local IBM

PC keyboard, otherwise they should be entered on the terminal keyboard.

.P

> The TESTS

The following tests are performed by VTEST:

>> Cursor positioning and movement

The first test verifies the standard cursor positioning (using

"<ESC>[n;nH") by drawing a large 'X' on the left hand side of the screen.

If you do not see a large 'X' (made of the 'H' character), then this type

of cursor positioning is not functioning correctly.

The second test verifies the alternate cursor positioning (using

"ESC>[n;nf") by drawing a large 'X' on the left hand side of the screen.

If you do not see a large 'X' (made of the 'f' character), then this type

of cursor positioning is not functioning correctly.

The third test tests the single cursor motion commands, by drawing

the corresponding command code 'A', 'B', 'C' and 'D' on the screen,

with an arrow positioned UP, DOWN, FORWARD and BACKWARD from them. Verify

that the arrow characters are positioned correctly to the command characters

as identified by the labels on the screen.

>> Screen/Line clearing

The forth test draws double spaced lines over the entire screen, and

then executes a clear screen. If the screen clears properly, you should

see only the message "Erase ENTIRE screen" on the center of the display.

The fifth test draws double spaced lines over the entire screen, and

then perform "Clear from beginning of screen", and "Clear to end of

screen" commands. If these command function correctly, you should see a

single complete line in the center of the screen, with partial lines

above and below it.

The sixth test draws double spaced lines over the entire screen, and

then performs "Clear from beginning of line" and "Clear to end of line"

commands. If these commands function correctly, you should see that the

third line from the top has had the left portion erased, and that the

fifth line from the top has had its right portion erased. All other lines

should be intact.

.P

>> Character Attributes

The seventh test displays 16 lines, showing the 16 attribute combinations.

Verify that the displayed attributes are correct with the text shown on

the line.

The eighth test verifies that screen clear works properly when attributes

are enabled, by drawing a block of characters ("*****>") with ALL attributes

set, and then waiting for <return>. Verify that the attributes are shown

correctly, and  press <return>. The screen will be cleared with all attributes

enabled, and another block of data will be shown. Check that the screen was

cleared as normal, and that the next block of data ("*****>") is still shown

with all attributes enabled.

The ninth test draws a matrix of ANSI color codes with labels for the

foreground and background colors. Verify that the colors shown are correct

for each position in the matrix.

>> Save/Restore feature

The tenth test verifies the operation of the cursor save/restore feature,

by drawing a title, saving the cursor, drawing a diagonal line of 'X's by

repositioning the cursor, switching to graphics mode, drawing some more text

(with graphics), restoring the cursor, and finally outputing the word 'Here'.

Verify that the graphics was printed correctly, that the 'Here' was printed

correctly (not graphics, with no attributes) and in the correct location

(Opposite the title).

>> Set/Clear TABs

The next test verifies the operation of the TAB set/clear function. First,

TABS are set every 8 spaces, and a 'T' is shown at that position for reference.

The digits 1 to 7 are then printed out using tabs between each one. Each digit

should line up under the corresponding 'T'. Then, tab stop number 5 is cleared,

and the above step repeated. The digits 5 and higher should be shifted to the

right by one tab stop. Finally, all tabs are cleared, and the digits 1 to 7 are

printed again. This should result in a display of 7 lines, with a digit at the

far right of every line.

>> Double width/height

The next test displays normal, double width and double height lines, Check

that they are shown correctly.

.P

>> Forward/Reverse scrolling

The next test verifies NEWLINE and REVERSE index. Using cursor positioning,

a vertical line of 'X's is drawn down the left hand side of the screen, and

a diagonal line is drawn back up to the top, after which the program waits

for <return>. When you have verified the above, press <return>. The diagonal

line will be continued upward, with the screen scrolling in REVERSE until the

vertical line has completely scrolled off the bottom of the screen.

The next test verifies the operation of scrolling regions, by drawing a BOX

and scrolling the numbers up to 50 both forwards and backward in it. Verify

that the remainder of the screen (above and below the box) is unaffected. NOTE:

Since the VT100 does not allow scrolling regions to be defined over columns,

the sides of the box will be scrolled off the screen during the test. This is

normal operation.

>> Character set

The next test displays the entire graphics character set, which you can

verify against your terminal manual. The US '#' and UK 'POUND' sign are

also displayed, to test the US and UK character sets.

>> Linefeed/Newline and Auto-Wrap

The next test verifies the operation of the LINE-FEED/NEWLINE and

the AUTO-WRAP features. The first block of lines 'One', 'Two', 'Three'

should all be moved over against the left margin. The next block should

have each line occurring just below and one position to the right of the

end of the previous line. The 'Auto wrap Set:' line should wrap around past

the end of the screen, and begin another line. The 'Auto wrap Reset:' line

should not wrap, but appear as a single line.

>> Cursor X/Y Origin mode

The next test verifies the operation of absolute verses relative cursor

positioning (ORIGIN MODE). A scrolling region is established within two

horizontal lines on the screen, and two 'X' are drawn. the first 'X' should

be ABOVE the first line (top left hand corner of screen). The second 'X'

should be WITHIN the two lines. Both 'X's should be the same size and shape.

.P

>> VT52 Mode tests

The next test verifies VT52 mode cursor positioning the same way as the

first VT100 test does.

The next test verifies VT52 mode cursor movement in the same way as the

third VT100 test does.

The next test verifies VT52 mode clearing functions in a manner similar

to the VT100 tests.

The next test verifies VT52 mode reverse line-feed by drawing a diagonal

line from the bottom to the top of the screen, and continuing it in a

manner which causes reverse scrolling of the screen contents.