.T Data Line Monitor

.=m4

.s20

.C D L M

.S

.c An IBM P.C. Based

.C RS-232 Asynchronous

.C Data Line Monitor

.s5

.c Release 1.9

.s

.c Revised 25-Aug-06

.s10

.c Dunfield Development Systems

.c ----------------------------

.c High quality tools for

.c Embedded Development

.c at low prices.

.s

.c http://www.dunfield.com

.s2

.c ?COPY.TXT 1989-2005 Dave Dunfield

.c **See COPY.TXT**

.P

> INTRODUCTION

Have you ever been in the situation where you have two serial devices

(such as a PC and a printer), which SHOULD work but just don't seem to

communicate? Or perhaps you are trying to debug a new communications

protocol, and when you start the file transfer everything just LOCKS UP!!!

In such situations, wouldn't it be nice to be able to SEE exactly what

is happening over the RS-232 cable? To SEE when characters were received in

relation to characters going out? To SEE flow control characters and their

effect? To SEE the modem carrier drop?

Wouldn't it be nice to do all this WITHOUT spending $5,000 - $10,000

on a commercial data scope?

If your requirements are simple monitoring of ASYNCHRONOUS RS-232

serial communications, DLM (Data Line Monitor) is for you!!!

Dunfield Development Services (DDS) offers software and firmware

development services specializing in systems and embedded applications.

For more information, visit: http://www.dunfield.com

DLM is provided on an "as is" basis, with no warranty of any kind.

In no event shall the author be liable for any damages arising from its use

or distribution.

.P

>> What is DLM?

DLM is an easy to use DATA LINE MONITOR for use on the IBM P.C. It

displays DTE and DCE data in a split line format (DTE over DCE), and records

up to 32767 events (characters and/or signal transitions) in a circular buffer

for later review (If the buffer overflows, the LAST 32767 events are

available).

Recorded data may be saved to disk, allowing a problem to be captured

in the field, and the results viewed at another time and place. Since the

data is stored as a standard DOS file, it may be transmitted over a modem

and viewed (using DLM) at a remote location. This allows an engineer at the

main office to provide better (and faster) support to a technician in the

field, since he can also SEE the problem.

It supports ASYNCHRONOUS operation at speeds up to 57600 BPS; 5,6,7 and

8 data bits; Even, Odd, Mark, Space and No parity; and 1 or 2 stop bits.

The DTE configuration may be set independently of the DCE configuration for

split speed/format operation.

The configuration may also be saved to disk, allowing you to have several

"canned" setups for common situations.

In addition to the data line monitor, DLM incorporates a SIGNAL MONITOR

(LIGHT BOX) for real time monitoring of the hardware signals.

>> Hardware Required

All that is required to use DLM is an IBM P.C. (or compatible) which has

two serial (COMM) ports. A special "Y" adapter cable (described later) must

be constructed to allow monitoring of both DTE and DCE data.

.P

> HARDWARE SETUP

Before you use DLM, you must construct a special "Y" adapter cable which

allows it to monitor "both sides" (DTE and DCE) of a single communications

line using the IBM P.C. serial ports (COM1 and COM2).

>> IBM P.C. serial ports

The serial ports of the IBM P.C. are RS-232 DTE configuration, and will

be in either a 25 or 9 pin configuration. The ports have the following

pinout:

.f-

Signal name                Origin   25-Pin(PC)  9-Pin(AT)

---------------------------------------------------------

Transmit Data       (TXD)   DTE         2           3

Receive Data        (RXD)   DCE         3           2

Request To Send     (RTS)   DTE         4           7

Clear To Send       (CTS)   DCE         5           8

Data Set Ready      (DSR)   DCE         6           6

Signal Ground       (GND)   ---         7           5

Data Carrier Detect (DCD)   DCE*        8           1

Data Terminal Ready (DTR)   DTE        20           4

Ring Indicator      (RI )   DCE        22           9

*   In order to be able to record this signal in data capture

mode, DLM records it as if it were originated by the DTE.

.f+

Note that since the PC serial port is a DTE, it drives the signals

with DTE origin, and monitors the signals with DCE origin. Therefore,

all connections to the line being monitored will be to the DCE signals

of the COMM port.

.P

>> Passive Monitor Cable

DLM monitors DTE data on COM1, and DCE data on COM2. Therefore, you

must connect the input (DCE) lines from COM1 to the DTE origin signals

on the monitored line, and the input (DCE) lines from COM2 to the DCE

origin signals.

To do this, you must construct a "Y" adapter cable, which contains a

set of "straight through" connectors which are inserted in the RS-232 line

being monitored, and also connects the DTE and DCE signals to the

appropriate input (DCE) pins on COM1 and COM2. Be sure to mark which COMM

port each of the two "PC" connectors connects to. If you get them backward,

the DTE/DCE data and signals will appear reversed.

.f-

"Straight  through"                          "IBM PC"

Line Monitor Connector                   COMM port connectors

DB-25    Male                    /----- COM1 (Monitor DTE)

| | | | | | |                   /

+-+-+-+-+-+-+==================<

| | | | | | |                   \

DB-25  Female                    \----- COM2 (Monitor DCE)

The signals must be connected as follows:

Line   Pin          Port    25-Pin(PC)  9-Pin(AT)

-------------------------------------------------

TXD     2   ----    COM1        3           2

RXD     3   ----    COM2        3           2

RTS     4   ----    COM1        5           8

CTS     5   ----    COM2        5           8

DSR     6   ----    COM2        6           6

GND     7   ----  COM1+COM2     7           5

DCD     8   ----    COM1       22           9

DTR    20   ----    COM1        6           6

RI     22   ----    COM2       22           9

Aux 1*  -   ----    COM1        8           1

Aux 2*  -   ----    COM2        8           1

*   These signals will be shown in "signal monitor" function,

but will not be recorded in "data capture" function.

You may use them to monitor any signals you like.

.f+

.P

> USING DLM

To set up DLM, connect the "straight through" end of the "Y" adapter

cable to the line to be monitored, and the "PC" connectors to the

appropriate COMM ports on the IBM P.C.

Execute DLM from the DOS command line, and you will be greeted with

The following menu:

.f-

+-------------------------+

| Begin data capture      |

| Review captured data    |

| Clear capture buffer    |

| RS-232 Signal monitor   |

| Serial port parameters  |

| Disk transfer functions |

| Exit to DOS             |

+-------------------------+

.f+

Each of the above menu items is described below:

>> Begin data capture

This option causes DLM to begin monitoring the data line. Any data or

signal transitions detected are displayed in a "split line" screen format

(in real time), as well as recorded in the circular capture buffer.

The data stream appears in pairs of two lines on the screen. DTE data

DTE data appears on the upper line, and DCE data will appears below the line.

A cursor is displayed below the line pair indicating where the next data item

will be displayed.

Signal transitions and line errors are displayed as an "arrow" which points

UP if the event was from the DTE, and DOWN if the event was from the DCE. One

exception to this is the DCD signal which occurs from the DCE, but is recorded

by DLM as if it were from the DTE. This is because each "direction" (DTE/DCE)

may only have three (3) signals recorded in the capture buffer. The DCE signals

are DSR, CTS and RI; The DTE signals are DTR, RTS and DCD. The exact signal or

error occurring may be displayed in "Review captured data".

Press ESCAPE at any time to exit this function, and return to the main

menu.

.P

>> Review captured data

In this function, the same "split line" screen as above is displayed,

however it shows data previously captured. You may press F1 to toggle

between the ASCII display, and a HEX display, in which the data values are

displayed in HEX (occupies both lines), and the DTE data in hilighted.

The ARROW keys may be used to move the cursor around the display. The

PGUP and PGDN keys may be used to advance and backup by entire screens. The

HOME key returns you to the beginning of the buffer, the END key positions

you on the last entry in the buffer.

When the cursor is positioned under an entry, it will contain a 'T' for

entries originating from the DTE, or 'C' indicating that the entry originated

from the DCE (In most cases this will be obvious since data will be shown in

the upper (DTE) or lower (DCE) line).

The time stamp of the entry is shown near the center of the status line

at the top of the screen. This marks the time (in seconds) at which this

entry occurred relative to the first entry in the buffer. Note that although

the display shows 1/100th seconds, the actual resolution of the timer is

approx 1/18th second. Also, the timestamp wraps to zero after 3640 seconds

(slightly over one hour).

The hex value of the data under the cursor is shown to the right of

the timestamp, and to the right of that are four "data flags":

.f-

F   - Framing error

P   - Parity error

O   - Overrun error

D   - Data is present

.f+

To the right of that, three signals will be shown:

.f-

DTE             DCE

---------       ---------

X   - DCD       X   - RI

T   - DTR       S   - DSR

R   - RTS       C   - CTS

.f+

At the extreme right side of the status line will be shown two numbers.

The first is the current position in the capture buffer, and the second is

the total number of entries in the capture buffer.

Press ESCAPE at any time to exit this function, and return to the main

menu.

.P

>> Clear capture buffer

This function removes any data which has been previously recorded

from the "capture" buffer. It is useful to remove old data before

starting a new monitoring session, otherwise any new data will be

appended to the old data already in the buffer.

When you execute this function, DLM displays the number of entries

currently residing in the capture buffer. Press ENTER to clear the buffer,

or ESCAPE to exit and leave it unchanged.

>> RS-232 Signal monitor

This function provides a "light-box", which shows all of

the hardware signals being monitored (DTR, DSR, RTS, CTS, DCD, RI, AS1 and

AS2). Each signal has a "light" which will be ON (white) if the signal is

asserted, and OFF (dark) if the signal is not asserted.

Changes occurring in the signal states are shown in "real-time", so that

the display always reflects the actual state of the line signals.

Press ESCAPE at any time to exit this function, and return to the main

menu.

>> Serial port parameters

This function allows you to set the speed and data format of the

ports monitoring DTE and DCE data. In order for DLM to correctly monitor

the data line, these parameters must be set up the same as the devices

being monitored.

Sub-menus are provided which allow you to set both the DTE and DCE

parameters simultaneously, or individually. Most modern serial communications

operate the DTE and DCE at the same speed and format.

Press ESCAPE at any time to exit each menu level, and return to the

next higher menu.

.P

>> Disk transfer functions

This function provides you with a sub-menu, which allows you to load/save

configuration or captured data to/from disk files:

.f-

+--------------------+

| Load Data Image    |

| Load Configuration |

| Save Data Image    |

| Save Configuration |

+--------------------+

.f+

Once you select a transfer operation, you will be prompted for a file

name. The following special keys may be used:

.f-

Left arrow      - Move to left

Right arrow     - Move to right

Home            - Move to beginning of name

End             - Move to end of name

PgUp            - Clear entire field

PgDn            - Clear from cursor to end of field

ESC             - Cancel and return to higher menu

ENTER           - Accept filename & perform operation

.f+

>> Exit to DOS

This function terminates DLM, and returns to the DOS command

prompt.

.P

> COMMAND LINE OPTIONS

DLM accepts the following options on the command line when it is

started:

.f-

DTE=1-4   - Specify DTE COM port number (COM1-COM4)

DTE=5+    - Specify DTE COM port address in HEX (5-FFFF)

DTE!      - Disable DTE monitoring (Requires no DTE port)

DCE=1-4   - Specify DCE COM port number (COM1-COM4)

DCE=5+    - Specify DCE COM port address in HEX (5-FFFF)

DCE!      - Disable DCE monitoring (Requires no DCE port)

/R        - Review mode - no COM ports - same as DTE! + DCE!

A=xx      - Specify video attribute used for the display

of DTE data when reviewing in HEX mode. This

attribute is used to differentiate the DTE from

the DCE data (displayed in normal video). The

default is 0F, which displays the DTE data as

intensified. To see the effect of various video

attribute values on your PC, please run the

VIDEO.COM utility program provided.

C=file    - Loads a previously saved DLM configuration.

F=file    - Loads a DLM VGA font from the specified file, and

uses it when Capturing/Reviewing data. The following

optional font files are provided with DLM:

CONTROL.DLF - Shows 00-1F as two letter words

HEX.DLF     - Shows 00-1F,7F-FF as HEX values

You may create and customize your own fonts with

the supplied DLMFE.COM program.

NOTE! DLM FONTS WORK ONLY WITH VGA DISPLAYS!

Also note: DLM uses standard VGA (16 line) fonts.

Some video systems (noteably laptops) have "stretch"

modes which use custom fonts having more lines. If

more than 16 lines are found in the active font, DLM

will attempt to stretch its fonts to match. If you

experience font problems on such a system, try turning

off any special modes and using standard VGA text.

Finally: ALLHEX.DLF is a special all-hex display font

which works with MDLM's dual character set mode only.

It does not provide a meaningful display with DLM.

M=xx      - Specify a mask value (hex) to be ANDed with each

character before it is displayed. Most commonly

used to remove the high bit from ASCII data (M=7F).

.f+

.P

> DEBUGGING TIPS

Here are several common problems which occur with serial communications,

and information on how you could use DLM to diagnose them:

>> Hardware Connections

You can easily check out the hardware signals by using the "RS-232 Signal

monitor".

Connecting the DTE to the monitor cable should "raise" the DTR and RTS

signals. If this is not the case, either your connections are incorrect, or

the DTE device is not online properly. (In some rare cases this may be a

normal state for the device).

Connecting the DCE to the monitor cable should "raise" the DSR and CTS

signals. If this is not the case, either your connections are incorrect, or

the DTE device is not online properly. (RI and DCD are not always ON, so

you can't expect them to be in a particular state).

Once you get the above signals correctly connected, switch to "Begin

data capture".

Data transmitted from the DTE should appear ABOVE the dashed separating

line, data transmitted from the DCE should appear BELOW the line.

>> Speed and Data format

If the speed of the two sending and receiving devices is not the same,

no communication will occur. Assuming you know what the speed SHOULD be, it

is easy to recognize data at an incorrect speed with DLM.

Capture a good sample of ASCII data with DLM set to the speed and format

which you think the communication should be occurring at, and then look at it

in review mode.

If the data completely readable, the speed and format is correct.

If the data is partly legible, but about half of the characters seem

to be corrupted, the speed is correct, but you have incorrect PARITY.

If the data is completely unreadable, examine the HEX values of several

characters (by moving the cursor and looking at the status line). If the data

has the high bit set (80-FF), check to see if it would be readable without

the high bit set. If this is the case, the speed is correct, but the data

format (Data bits/Parity) is incorrect. DATA BITS is the most likely suspect.

.P

If the above test does not reveal any readable data, the speed is

probably incorrect. This will also manifest itself with "lots" of

'F'raming and 'O'verrun errors.

>> Flow Control

Many modern devices are not capable of receiving data continuously

at the highest speeds supported on the interface. An example of this is

a printer which prints 200 Characters per second (CPS), and has a serial

interface running at 9600 bits per second which is about 960 CPS.

Such devices usually have an internal buffer, which absorbs data at

full speed, and stores it so that it can be printed at a slower speed.

When this buffer approaches being full, the printer will assert FLOW

CONTROL on the sending device, which causes it to stop sending data

until it is later instructed to proceed.

If flow control is not working properly, the usual symptom is that the

first few lines of data are correctly received by the device, after which

symptoms vary from "lost data" to "lock up".

There are two common forms of flow control, one uses hardware signals,

and is called HARDWARE FLOW CONTROL. The other uses reserved data characters,

and is called SOFTWARE FLOW CONTROL.

>>> Hardware Flow Control

With HARDWARE flow control, the receiving device will deassert one of

its hardware output leads (DTE=DTR/RTS, DCE=DSR/CTS) to inform the sender

to stop sending. When it is ready to receive more data, it will reassert

the signal, and transmission will proceed.

Using DLM, you can monitor the hardware signals during data transmission,

and see if one of them is lowered to perform flow control. You can also see

if the sending devices responds to the flow control (stops sending), and how

quickly it responds (by noting how many characters are sent after the signal

drops).

If the signal dropping appears to have no effect on the data stream,

then you know that hardware flow control is not being recognized or acted

upon.

.P

>>> Software Flow Control

With SOFTWARE flow control, the receiving device transmits a character

to the sending device to command it to stop sending. This character is

usually CONTROL-S ($13). When it is ready to receive more data, a different

character is transmitted. This character is usually CONTROL-Q ($11).

Using DLM, you can monitor the data to and from the receiving device,

and check for flow control characters. You can also see if the sending device

responds to the flow control (stops sending), and how quickly is responds (by

noting how may characters are sent after the CONTROL-S is received).

If a CONTROL-S is transmitted, and it appears to have no effect on the

data stream, then you know that software flow control is not being recognized

or acted upon.

.P

> FILE FORMAT

This section describes the format of the data file which is saved by

the "Save Data Image" function, should you have need to manipulate this

data from an external program.

The image file is 131080 bytes in size, which contains 65540 16 bit

entries. Each entry is stored in Little Endian format (the least significant

byte occurs first). The meaning of the entries is as follows:

.f-

Entry        Address              Description of entry

--------------------------------------------------------------------

1        (0000-0001)    Pointer to start of valid data in buffer

2        (0002-0003)    Pointer to end of valid data in buffer

3        (0004-0005)    Status of DTE at end of last capture

4        (0006-0007)    Status of DCE at end of last capture

5-32772   (0008-10007)   Data buffer (see below)

32773-65538  (10008-32770)   Time-Stamps (DOS timer tick count)

.f+

The start and end pointers indicate the area of valid data in the

circular data buffer. To read the entire buffer content, you should begin

reading at an offset into the data buffer indicated by the start pointer,

and read each entry, wrapping at 64K bytes (32K entries) back to the

beginning, and continuing until you reach the offset in the end pointer.

The DTE/DCE status entries are used internally by DLM, and should be

ignored.

Each data buffer entry has the following format:

.f-

Bit(s)                  Description

--------------------------------------------------------

15  - Framing error has occurred

14  - Parity  error has occurred

13  - Overrun error has occurred

12  - Data byte is available for this entry (bits 7-0)

11  - 0=Entry applies to DTE, 1=Entry applies to DCE

10  - DCD/RI  signal status

9  - DTR/DSR signal status

8  - RTS/CTS signal status

7-0  - Data if Bit12=1, otherwise undefined

.f+

.P

> ASCII code chart

.f-

.s5

Most Significant Figure

HEX||  0  |  1  |  2  |  3  |  4  |  5  |  6  |  7  |

===++=====+=====+=====+=====+=====+=====+=====+=====+

0 || NUL | DLE |     |  0  |  @  |  P  |  `  |  p  |

---++-----+-----+-----+-----+-----+-----+-----+-----+

1 || SOH | DC1 |  !  |  1  |  A  |  Q  |  a  |  q  |

L   ---++-----+-----+-----+-----+-----+-----+-----+-----+

e    2 || STX | DC2 |  "  |  2  |  B  |  R  |  b  |  r  |

a   ---++-----+-----+-----+-----+-----+-----+-----+-----+

s    3 || ETX | DC3 |  #  |  3  |  C  |  S  |  c  |  s  |

t   ---++-----+-----+-----+-----+-----+-----+-----+-----+

4 || EOT | DC4 |  $  |  4  |  D  |  T  |  d  |  t  |

S   ---++-----+-----+-----+-----+-----+-----+-----+-----+

i    5 || ENQ | NAK |  %  |  5  |  E  |  U  |  e  |  u  |

g   ---++-----+-----+-----+-----+-----+-----+-----+-----+

n    6 || ACK | SYN |  &  |  6  |  F  |  V  |  f  |  v  |

i   ---++-----+-----+-----+-----+-----+-----+-----+-----+

f    7 || BEL | ETB |  '  |  7  |  G  |  W  |  g  |  w  |

i   ---++-----+-----+-----+-----+-----+-----+-----+-----+

c    8 || BS  | CAN |  (  |  8  |  H  |  X  |  h  |  x  |

a   ---++-----+-----+-----+-----+-----+-----+-----+-----+

n    9 || HT  | EM  |  )  |  9  |  I  |  Y  |  i  |  y  |

t   ---++-----+-----+-----+-----+-----+-----+-----+-----+

A || LF  | SUB |  *  |  :  |  J  |  Z  |  j  |  z  |

F   ---++-----+-----+-----+-----+-----+-----+-----+-----+

i    B || VT  | ESC |  +  |  ;  |  K  |  [  |  k  |  {  |

g   ---++-----+-----+-----+-----+-----+-----+-----+-----+

u    C || FF  | FS  |  ,  |  <  |  L  |  \  |  l  |  |  |

r   ---++-----+-----+-----+-----+-----+-----+-----+-----+

e    D || CR  | GS  |  -  |  =  |  M  |  ]  |  m  |  }  |

---++-----+-----+-----+-----+-----+-----+-----+-----+

E || SO  | RS  |  .  |  >  |  N  |  ^  |  n  |  ~  |

---++-----+-----+-----+-----+-----+-----+-----+-----+

F || SI  | US  |  /  |  ?  |  O  |  _  |  o  | DEL |

---++-----+-----+-----+-----+-----+-----+-----+-----+