.T Line Error Simulator

.=m4

.s20

.C L E S

.S

.c An IBM P.C. Based

.C RS-232 Asynchronous

.C Line Error Simulator

.s5

.c Release 1.1

.s

.c Revised 28-May-99

.s10

.c Dunfield Development Systems

.c ----------------------------

.c High quality tools for

.c Embedded Development

.c at low prices.

.s

.c http://www.dunfield.com

.s2

.c ?COPY.TXT 1990-2005 Dave Dunfield

.c **See COPY.TXT**

.P

> INTRODUCTION

LES is a program which intercepts the data being transmitted over

an asynchronous communications line, and simulates varying degrees of

line quality by injecting errors.

LES is an excellent tool for evaluating the performance of a

transmission protocol in the face of line errors. Since the frequency and

type of errors injected into the data stream can be strictly controlled,

various protocol recovery situations can be easily created and reproduced.

> HARDWARE REQUIREMENTS

The LES program runs on any IBM compatible personal computer, with

two COMM ports. A special "split" cable must be constructed for use in

most situations.

>> IBM P.C. Serial ports

The serial ports of the IBM P.C. are RS-232 DTE configuration, and will

be in either a 25 or 9 pin configuration. The ports have the following

pinout:

.f-

Signal name                Origin   25-Pin(PC)  9-Pin(AT)

---------------------------------------------------------

Transmit Data       (TXD)   DTE         2           3

Receive Data        (RXD)   DCE         3           2

Request To Send     (RTS)   DTE         4           7

Clear To Send       (CTS)   DCE         5           8

Data Set Ready      (DSR)   DCE         6           6

Signal Ground       (GND)   ---         7           5

Data Carrier Detect (DCD)   DCE         8           1

Data Terminal Ready (DTR)   DTE        20           4

Ring Indicator      (RI )   DCE        22           9

.f+

.P

To use LES you will have to construct a "split" cable which passes

hardware control signals straight through, but routes the data streams

through the P.C. where errors can be injected.

.f-

"Split"                                 "IBM PC"

Line Monitor Connector                   COMM port connectors

(DTE)

DB-25    Male                    /----- COM1 (DTE)

| | | | | | |                   /

+-+-+-+-+-+-+==================/

|     | | | |

+-+-+-+-+-+-+==================\

| | | | | | |                   \

DB-25  Female                    \----- COM2 (DCE)

(DCE)

The signals must be connected as follows:

DTE   DCE  Pin          Port    25-Pin(PC)  9-Pin(AT)

-------------------------------------------------

TXD         2   ----    COM1        3           2

RXD         3   ----    COM1        2           3

TXD   2   ----    COM2        2           3

RXD   3   ----    COM2        3           2

GND   GND   7   ----  COM1+COM2     7           5

.f+

.P

> USING LES

To set up LES, connect the DTE side of the split cable to COM1, and

the DCE side to COM2.

Execute LES from the DOS command line, and you will be greeted with the

following menu:

.f-

+-----------------------+

|Begin Error SImulation |

|Configure error types  |

|Configure serial ports |

|Disk transfer functions|

|Exit to DOS            |

+-----------------------+

.f+

Each of the above options is described below:

>> Begin Error Simulation

This options causes LES to begin simulation the line errors. A display

is shown and updated in real time indicating the number of characters which

have been processed from each of the DTE and the DCE.

Any programmed errors will be injected into the data streams when the

selected number of characters have been received.

Press ESCAPE to terminate the simulation, and return to the main menu.

>> Configure error types

This options allows you to select one of 5 programmed errors to be

setup. Once you pick an error number, you will be asked for "BOTH", "DTE"

or "DCE", this allows you to decide which data stream the error will have

effect on.

If you select "DTE", the error will be injected only into the data stream

from the DTE.

If you select "DCE", the error will be injected only into the data stream

from the DCE.

If you select "BOTH", the error will be injected into whichever stream

causes the "frequency" count to be exceeded (See below).

You will then be prompted for the error type:

>>> Disabled

This option prevents the error from occurring. It is used for error

numbers which you don't want to use.

.P

>>> Drop char

Causes one character to be dropped from the data stream whenever the

"frequency" count expires (See below).

>>> Replace char

Prompts for a character value (in hex), which will replace one character

from the data stream whenever the frequency count expires.

>>> Flip bits

Prompts for a bit mask (in hex), which will be exclusive OR'ed into one

character from the data stream whenever the frequency count expires.

.i-

Once you have picked the error type, you will be prompted for the

"Frequency", which is a decimal number from 0 to 65535.

For every character that is received, the "frequency count" values for

any errors which apply to that data stream (DTE, DCE or BOTH) are incremented.

When the frequency count for a particular error occurs, the error is

"triggered" and injected into the data stream.

When an error is "triggered", the frequency count is reset, causing it

to wait and retrigger when enough characters have again been received

from the data stream.

By using the five available errors wisely, you can simulate a wide

range of error conditions and "hit rates".

>> Configure serial ports

This function allows you to set the speed and data format of the COMM

port. In order for LES to correctly pass data between the devices, these

parameters must be set up the same as the devices.

Sub menus are provided for all of the data communications parameters.

Press ESCAPE at any time to exit each menu level, and return to the

next higher menu.

.P

>> Disk transfer functions

This function provides you with a sub-menu, which allows you to load/save

the LES configuration:

.f-

+--------------------+

| Load Configuration |

| Save Configuration |

+--------------------+

.f+

Once you select a transfer operation, you will be prompted for a file

name. The following special keys may be used:

.f-

Left arrow      - Move to left

Right arrow     - Move to right

Home            - Move to beginning of name

End             - Move to end of name

PgUp            - Clear entire field

PgDn            - Clear from cursor to end of field

ESC             - Cancel and return to higher menu

ENTER           - Accept filename & perform operation

.f+

>> Exit to DOS

This options terminates LES, and returns to the DOS command prompt.