.T Multi-channel Data Line Monitor

.=m4

.s20

.C M D L M

.S

.c An IBM P.C. Based

.c Multi-Channel

.C RS-232 Asynchronous

.C Data Line Monitor

.s5

.c Release 1.0

.s

.c Revised 25-Sep-01

.s10

.c Dunfield Development Systems

.c ----------------------------

.c High quality tools for

.c Embedded Development

.c at low prices.

.s

.c http://www.dunfield.com

.s2

.c ?COPY.TXT 1989-2005 Dave Dunfield

.c **See COPY.TXT**

.P

> INTRODUCTION

>> What is MDLM?

MDLM is an easy to use DATA LINE MONITOR for use on the IBM P.C. which

can simultaneously record data and signal changes on up to 24 serial lines.

As many as 32767 events can be recorded for later review.

Recorded data may be saved to disk, allowing a problem to be captured

in the field, and the results viewed at another time and place. Since the

data is stored as a standard DOS file, it may be transmitted over a modem

and viewed (using MDLM) at a remote location. This allows an engineer at the

main office to provide better (and faster) support to a technician in the

field, since he can also SEE the problem.

It supports ASYNCHRONOUS operation at speeds up to 57600 BPS; 5,6,7 and

8 data bits; Even, Odd, Mark, Space and No parity; and 1 or 2 stop bits.

The DTE configuration may be set independently of the DCE configuration for

split speed/format operation.

MDLM's configuration is loaded from a disk file, allowing you to have

several "canned" setups, and to easily switch between them.

In addition to the data line monitor function, MDLM incorporates a SIGNAL

MONITOR (LIGHT BOX) for real time monitoring of the hardware signals.

>> Hardware Required

All that is required to use MDLM is an IBM P.C. (or compatible) with one

or more serial ports. You must have a serial port for each channel that you

wish to monitor.

.P

> HARDWARE SETUP

Before you use MDLM, you must install enough serial ports in your

computer for the number of channels that you wish to monitor. The PC

architecture supports only 4 standard serial ports, however there are

a add-on cards available which provide additional serial ports beyond

this number.

>> IBM P.C. serial ports

The serial ports of the IBM P.C. are RS-232 DTE configuration, and will

be in either a 25 or 9 pin configuration. The ports have the following

pinout:

.f-

Signal name                Origin   25-Pin(PC)  9-Pin(AT)

---------------------------------------------------------

Transmit Data       (TXD)   DTE         2           3

Receive Data        (RXD)   DCE         3           2

Request To Send     (RTS)   DTE         4           7

Clear To Send       (CTS)   DCE         5           8

Data Set Ready      (DSR)   DCE         6           6

Signal Ground       (GND)   ---         7           5

Data Carrier Detect (DCD)   DCE         8           1

Data Terminal Ready (DTR)   DTE        20           4

Ring Indicator      (RI )   DCE        22           9

.f+

Note that since the PC serial port is a DTE, it drives the signals

with DTE origin, and monitors the signals with DCE origin. Therefore,

all connections to the line being monitored will be to the DCE signals

of the COMM port.

.P

>> Serial Port Settings

MDLM communicates directly with the serial port hardware, which

must be a standard 8250 (or compatible) UART device.

The four standard PC serial ports addresses are:

.c 03F8 = COM1, Interrupt=4

.c 02F8 = COM2, Interrupt=3

.c 03E8 = COM3, Interrupt=4

.c 02E8 = COM4, Interrupt=3

Note that COM1 and COM3 share an interrupt, and that COM2 and COM4

also share an interrupt. The way the PC shares interrupts allows only

one of the COM ports sharing an interrupt to use it at any one time,

which is quite restrictive. MDLM works around this problem by not using

interrupts on any of the COM ports, and in fact turns all COM port

interrupts OFF.

For compatibility with standard software, the first 4 serial ports

installed in your PC should be configured for the addresses and interrupts

listed above. MDLM can be configured with any serial port address, allowing

additional serial port to be installed at any available location (Remember

that each serial port occupies 8 locations in the I/O map ...ie: COM1 at

03F8 actually uses addresses from 03F8 to 03FF).

.P

> USING MDLM

To set up MDLM, connect all of the devices to be monitored to the

"DCE" inputs of the corresponding PC serial ports. NOTE: MDLM cannot

prevent the PC from driving the DTE outputs of the ports, so these should

NOT be connected. You will probably have to make up special "one way"

cables for this.

Execute MDLM from the DOS command line, and you will be greeted with

The following menu:

.f-

+-------------------------+

| Begin data capture      |

| Review captured data    |

| Clear capture buffer    |

| Signal monitor          |

| Serial port parameters  |

| Disk transfer functions |

| Post Processor menu     |

| Edit comment            |

| Exit to DOS             |

+-------------------------+

.f+

Each of the above menu items is described below:

>> Begin data capture

This option causes MDLM to begin monitoring the data lines. Any data or

signal transitions detected are displayed in a "multi line" screen format

(in real time), as well as recorded in the circular capture buffer.

The data stream appears in sets of lines on the screen, with each entry

shown in chronological order, positioned in the line corresponding to the

channel where the event occurred. The Channel 1 line is shown in a different

color, which makes it easy to see the logical groups of data lines on the

screen.

Signal transitions and line errors occurring on a channel are displayed

using a different color indicator, as specified in the MDLM configuration

file. The actual signal/error status is recorded and can be viewed later in

the "Review captured data" function.

A vertical "cursor" through all channel lines indicates the next location

on the screen where an event will be recorded.

The F1 key can be used to cycle between the loaded display fonts. This

is normally used to view data in HEX, or as Control characters.

Press ESCAPE at any time to exit this function, and return to the main

menu.

.P

>> Review captured data

In this function, the same "multi line" screen as above is displayed,

however it shows data previously captured. You may press F1 to toggle

between the display fonts.

The ARROW keys may be used to move the cursor around the display. The

PGUP and PGDN keys may be used to advance and backup by entire screens. The

HOME key returns you to the beginning of the buffer, the END key positions

you on the last entry in the buffer.

When the cursor is positioned under an entry, details of that entry will

be displayed in the title bar at the top of the screen:

.f-

REVIEW: ... fn  ne:sp  cp:cn  hx  si  ti

...     = Key prompts

fn      = Font name (if one selected)

ne      = Total number of entries captured

sp      = Position of first entry shown on screen

cp      = Position of entry under cursor

cn      = Channel number of entry under cursor

hx      = Hex value of data character (-- if none)

si      = Signal indicators (BFPODCRSC - *1)

ti      = Time index *2

.f+

.i-3 *1 The signal indicators are programmable through the MDLM configuration

file, but always have this meaning (in this order from left to right), shown

with the default indicator characters:

.f-

F   - Framing error

P   - Parity error

O   - Overrun error

D   - Data is present

c   - Carrier detect

r   - Ring indicator

S   = data Set ready

C   = Clear to send

.f+

.i-3 *2 This marks the time (in seconds) at which this entry occurred relative

to the first entry in the buffer. Note that although the display shows 1/100th

seconds, the actual resolution of the timer is approx 55ms. Also, the

timestamp wraps to zero after 3640 seconds (slightly over one hour).

Press ESCAPE at any time to exit this function, and return to the main

menu.

.P

>> Clear capture buffer

This function removes any data which has been previously recorded

from the "capture" buffer. It is useful to remove old data before

starting a new monitoring session, otherwise any new data will be

appended to the old data already in the buffer.

When you execute this function, MDLM displays the number of entries

currently residing in the capture buffer. Press ENTER to clear the buffer,

or ESCAPE to exit and leave it unchanged.

>> Signal monitor

This function provides a "light-box", which shows all of the hardware

signals and indicators being monitored. Each signal has an indicator which

will be 'Y' if the signal is asserted, and 'n' if the signal is not asserted.

Changes occurring in the signal states are shown in "real-time", so that

the display always reflects the actual state of the line signals.

Press ESCAPE at any time to exit this function, and return to the main

menu.

>> Serial port parameters

This function allows you to set the speed and data format of the serial

ports. In order for MDLM to correctly monitor the data lines, these parameters

must be set up the same as the devices being monitored.

The following functions are provided:

.f-

Up/Down     = Select port to change

Left/Right  = Fine adjust of port baudrate

PgUp/PgDn   = Course adjust of port baudrate

D           = Cycle data bits: 5-6-7-8

P           = Cycle parity: ODD-EVEN-MARK-SPACE-NONE

S           = Cycle stop bits: 1-2

N           = Edit port/signal names

ESC         = Exit back to main menu

.f+

.P

>> Disk transfer functions

This function provides you with a sub-menu, which allows you to load/save

configuration or captured data to/from disk files:

.f-

+--------------------+

| Load Configuration |

| Load Data Image    |

| Save Data Image    |

+--------------------+

.f+

Once you select a transfer operation, you will be prompted for a file

name. The following special keys may be used:

.f-

Left arrow      - Move to left

Right arrow     - Move to right

Home            - Move to beginning of name

End             - Move to end of name

PgUp            - Clear entire field

PgDn            - Clear from cursor to end of field

ESC             - Cancel and return to higher menu

ENTER           - Accept filename & perform operation

.f+

>> Post-Processor menu

Brings up a menu of installed post-processors. Post-processors are

external program which are used to provide additional processing and/or

specialized views of the captured data.

Use the arrow keys to position the cursor over the post-processor

you wish to activate and press ENTER.

>> Edit comment

Allows you to edit the comment, which is saved in the data capture

files. This provides a convienent way to provide a permanent description

of the captured data.

>> Exit to DOS

This function terminates MDLM, and returns to the DOS command

prompt.

.P

> COMMAND LINE OPTIONS

MDLM accepts the following options on the command line when it is

started:

.f-

/C      - Begin immediate capture. Causes MDLM to enter the Data

capture function immediately upon starting.

C=file  - Specify configuration file to load (default: MDLM.CFG)

D=file  - Specify data capture  file to load (default: none)

.f+

> CONFIGURATION FILE

MDLM loads it's initial configuration from a text file called MDLM.CFG.

You may also load a configuration file of your choosing from the "Disk

transfer menu".

The format of the MDLM configuration file is as follows:

Blank lines, as well as any line beginning with ';' (comments)

are ignored.

The following statements may occur in the MDLM configuration file:

>> Video attributes

The following statements set video attributes (colors) for the various

screen components of MDLM. <colors> is a 2 digit hex value, which specifies

forground/background colors. To see the available colors and corresponding

hex values, please run VIDEO.COM included with this package.

Some commands also specify a character value in hex. To see the available

characters and corresponding hex values, please run CSET.COM included in this

package.

MDLM supports the use of dual character set VGA fonts to allow the

captured data to be shown in a different character set than the MDLM title

and prompts... If this feature is activated (See FONT directive), you must

observe the rule that Bit3 is set in the DATA colors, and ONLY the data

colors.. The TITLE, MAIN, MENU, BACKGROUND, ENDATA and CURSOR/BACKGROUND

colors must all have Bit3 clear.

>>> TITLE <colors>

Sets the colors for the TITLE bar, which occurs at the top

of most MDLM screens.

>>> MAIN <colors>

Sets the colors for the main window/text display screens.

>>> MENU <colors>

Sets the colors for pop-up menus.

.P

>>> BACKGROUND <first_colors> <other_colors> <signal_character>

Sets the colors for the background (non-data) items in CAPTURE/REVIEW

modes. The first value is the colors for the first channel (Channel 1),

the second value is the colors for the other channels (2-24), and the third

value is the ASCII character value to be shown in a channel position when

a non-data event (signal transition) occurs.

>>> DATA <first_colors> <other_colors>

Sets the colors for the data receiption events which are shown in

CAPTURE/REVIEW modes. The first value is the colors for the first channel

(Channel 1), the second value is the colors for the other channels (2-24).

>>> ENDATA <first_colors> <other_colors> <end_character>

Sets the colors for the end-of-data indicator in CAPTURE/REVIEW modes.

This is the vertical bar shown across all channels which marks the next

location on the screen where events will be recorded in capture mode, and

marks the end of the buffer in review mode. The first value is the colors

for the first channel (Channel 1), the second value is the colors for the

other channels (2-24).

>>> CURSOR <data_colors> <background_colors>

Sets the colors for the review cursor which indicates the currently

selected position in the capture buffer during review mode. The first value

is the colors for channel positions containing data, the second value is

the colors for channel positions not containing data.

.P

>> Other directives

>>> PORT <addr> <speed> <Dbits> <Parity> <Sbits> <Signals>

MDLM supports monitoring of up to 24 serial ports for data capture.

<addr> is the base address of the serial port.

<speed> is the serial speed (max 57600) which will be monitored. If

the value specified cannot be achieved with the PC hardware, MDLM will

issue a warning and adjust the speed to the closest available one.

<Dbits> <Parity> and <Sbits> defined the DATA FORMAT: Data-bits(5-8),

Parity(O/E/M/S/N) and Stop-bits(1-2) which will be monitored.

<signals> defines the signal indicator characters which are shown

in the review title bar and signal monitor screens when the corresponding

signal is active. The physical hardware signals may also have an optional

name assigned, which will be available passed to any post-processors (MDLM

itself does not use the signal names). The character values shown below

are the defaults used if none are assigned.

.f-

T[:name]    = Indicator/name for CTS input

S[:name]    = Indicator/name for DSR input

R[:name]    = Indicator/name for RI  input

C[:name]    = Indicator/name for CD  input

D           = Indicator for DATA present

O           = Indicator for OVERRUN

P           = Indicator for PARITY error

F           = Indicator for FRAMING error

B           = Indicator for BREAK detect

.f+

>>> FONT <filename> <Y|N> <name>

MDLM allows up to 4 VGA fonts to be loaded, and later selected when in

the CAPTURE or REVIEW modes.

<filename> is the name of the file containing the font. These .DLF files

may be created and edited with the included DLMFE font editor... Note that

the "Special Characters" included with the font and used by the DLM program

are ignored by MDLM.

<Y|N> is a Yes/No flag which indicates if dual character sets should be

activated for this font. When dual character sets are active, only video

attributes with Bit3 set are shown in the selected font. Those attributes

with Bit3 clear are shown in the standard PC font. This provides a way to

show all data characters without corrupting the MDLM cursors and prompts...

<Name> is the font name (max 8 chars) to be shown on the CAPTURE/REVIEW

title bar when this font is selected.

.P

>>> COMMENT <text>

Defines a comment line (max 80 chars) which will be displaued on the

MDLM main screen, and written as the first item in the data capture files.

>>> LINES <video lines>

Overrides the number of lines detected for the video screen... Not

normally required as this information is retrieved from the BIOS, however

you can use this command to override such detection of the BIOS does not

correctly report the screen size.

>>> TEMPFILE <filespec>

Defines a filename which MDLM will use to pass data to program

spawned via the Post-Processor menu.

<filespec> is the path/name of the temporary file. To make it possible

to create unique temporary files, MDLM will perform the following substitutions

on '%' sequences occuring in the file specification.

.f-

%Y  = Year   of last capture: 4 digits

%E  = yEar   of last capture: 2 digits

%M  = Month  of last capture: 4 digits

%D  = Day    of last capture: 4 digits

%H  = Hour   of last capture: 4 digits

%I  = mInute of last capture: 4 digits

%S  = Second of last capture: 4 digits

%C  = Count of tempfiles created: 4 digits

%O  = cOunt of tempfiles created: 2 digits

%K  = Keep tempfile (do not delete)

.f+

.p

>>> PROCESS <menu text>| <filename> <command tail>

MDLM allows up to 10 post-processor programs to be configured and

selected via the "Post-processor" menu. These program are normally used to

provide customized views of the data. The MDLM data can be accessed by

the post-processor program by reading it directly from memory, or via a

temporary file (See TEMPFILE directive).

<menu text> is the name (max 50 chars) to be shown on the menu to

identify this post-processor. The name may include spaces, and must be

terminated with the '|' character.

<filename> is the fill path (including extension) of an executable

file to be called when this post-processor is selected.

<command tail> contains any command line arguments which are to be

passed to the program. The remainder of the line is copied "as is", with

the following substitutions:

.f-

%S = Segment address of uart/line status segment

Channel/data segment is +4096

Timestamp segment is +8192

%R = Buffer read pointer

%W = Buffer write pointer

Valid data (words) exists between Read and Write

pointer in circular 64K segments.

%# = Number of channels defined

%D = MDLM Data segment

%C = Offset to array of channel structs

%N = Offset to comment/Name

%T = Offset to Year/Month/Day/Hour/Minute/Second (words)

%F = Name of temp file / saves data to file before exec

%M = Like %F but frees/reallocates data capture segments.

Reloads segments from temp file after process runs.

Slower, but provides extra 192k of memory to process.

%% = Single '%'

.f+

.P

> DEBUGGING TIPS

Here are several common problems which occur with serial communications,

and information on how you could use MDLM to diagnose them:

>> Hardware Connections

You can easily check out the hardware signals by using the "Signal

monitor".

Connecting a DTE to a monitoring port should "raise" the DTR and RTS

signals. If this is not the case, either your connections are incorrect, or

the DTE device is not online properly. (In some rare cases this may be a

normal state for the device).

Connecting a DCE to the monitoring port should "raise" the DSR and CTS

signals. If this is not the case, either your connections are incorrect, or

the DTE device is not online properly. (RI and DCD are not always ON, so

you can't expect them to be in a particular state).

Once you get the above signals correctly connected, switch to "Begin

data capture".

>> Speed and Data format

If the speed of the two sending and receiving devices is not the same,

no communication will occur. Assuming you know what the speed SHOULD be, it

is easy to recognize data at an incorrect speed with MDLM.

Capture a good sample of ASCII data with MDLM set to the speed and format

which you think the communication should be occurring at, and then look at it

in review mode.

If the data completely readable, the speed and format is correct.

If the data is partly legible, but about half of the characters seem

to be corrupted, the speed is correct, but you have incorrect PARITY.

If the data is completely unreadable, examine the HEX values of several

characters (by moving the cursor and looking at the status line). If the data

has the high bit set (80-FF), check to see if it would be readable without

the high bit set. If this is the case, the speed is correct, but the data

format (Data bits/Parity) is incorrect. DATA BITS is the most likely suspect.

.P

If the above test does not reveal any readable data, the speed is

probably incorrect. This will also manifest itself with "lots" of

'F'raming and 'O'verrun errors.

>> Flow Control

Many modern devices are not capable of receiving data continuously

at the highest speeds supported on the interface. An example of this is

a printer which prints 200 Characters per second (CPS), and has a serial

interface running at 9600 bits per second which is about 960 CPS.

Such devices usually have an internal buffer, which absorbs data at

full speed, and stores it so that it can be printed at a slower speed.

When this buffer approaches being full, the printer will assert FLOW

CONTROL on the sending device, which causes it to stop sending data

until it is later instructed to proceed.

If flow control is not working properly, the usual symptom is that the

first few lines of data are correctly received by the device, after which

symptoms vary from "lost data" to "lock up".

There are two common forms of flow control, one uses hardware signals,

and is called HARDWARE FLOW CONTROL. The other uses reserved data characters,

and is called SOFTWARE FLOW CONTROL.

>>> Hardware Flow Control

With HARDWARE flow control, the receiving device will deassert one of

its hardware output leads (DTE=DTR/RTS, DCE=DSR/CTS) to inform the sender

to stop sending. When it is ready to receive more data, it will reassert

the signal, and transmission will proceed.

Using MDLM, you can monitor the hardware signals during data transmission,

and see if one of them is lowered to perform flow control. You can also see

if the sending devices responds to the flow control (stops sending), and how

quickly it responds (by noting how many characters are sent after the signal

drops).

If the signal dropping appears to have no effect on the data stream,

then you know that hardware flow control is not being recognized or acted

upon.

.P

>>> Software Flow Control

With SOFTWARE flow control, the receiving device transmits a character

to the sending device to command it to stop sending. This character is

usually CONTROL-S ($13). When it is ready to receive more data, a different

character is transmitted. This character is usually CONTROL-Q ($11).

Using MDLM, you can monitor the data to and from the receiving device,

and check for flow control characters. You can also see if the sending device

responds to the flow control (stops sending), and how quickly is responds (by

noting how may characters are sent after the CONTROL-S is received).

If a CONTROL-S is transmitted, and it appears to have no effect on the

data stream, then you know that software flow control is not being recognized

or acted upon.

.P

> FILE FORMAT

This section describes the format of the data file which is saved by

the "Save Data Image" function, should you have need to manipulate this

data from an external program. All WORD entries occur in "little endian"

format (low order byte occurs first).

.f-

MDLM File format:

c... - Data comment/name (variable length ASCII - up to 80 bytes)

0    - Delimits end of comment/name     (byte with value 0)

n    - # of configured channels         (byte)

n*66 - 'n' channel control entries      (66*n bytes):

word:   Uart status register I/O address (base address + 5)

word:   Last status read from uart *1

word:   Offset to first video line on screen

byte10: Indicators for bit signals (+ 0 terminator)

word:   Uart baudrate divisor

byte:   Number of data bits: 5-8

byte:   Parity: 0=OFF 1=Even 2=MARK 3=SPACE 4=NONE

byte:   Number of stop bits: 1-2

byte9:  Channel name (+ 0 terminator)

byte9:  Ring-Indicator name (+ 0 Terminator)

byte9:  Carrier-Detect name (+ 0 Terminator)

byte9:  Data-Set-Ready name (+ 0 Terminator)

byte9:  Clear-to-Send  name (+ 0 Terminator)

yy   - Year of last capture             (word)

m    - Month of last capture            (byte)

d    - Day of last capture              (byte)

h    - Hour of last capture             (byte)

m    - Minute of last capture           (byte)

s    - Second of last capture           (byte)

e    - Number of capture entries        (word)

e*6  - Data capture entries             (6*e bytes):

byte:   Channel number: 0-(n-1)

byte:   Uart line  status register *1

byte:   Uart modem status register *1

byte:   Data value *2

word:   Lower 16 bits of BIOS timer at time of event

*1= All status bits except for

Line  status: Break/Framing/Overrun/Parity/Data

Modem status: Cts/Rts/Ri/Cd

are masked to zero unless no capture has ever taken place,

in which case these read as $FF.

*2= If 'Data' bit is not set in uart line status, this value

will be indeterminate.

.f+

.P

> ASCII code chart

.f-

.s5

Most Significant Figure

HEX||  0  |  1  |  2  |  3  |  4  |  5  |  6  |  7  |

===++=====+=====+=====+=====+=====+=====+=====+=====+

0 || NUL | DLE |     |  0  |  @  |  P  |  `  |  p  |

---++-----+-----+-----+-----+-----+-----+-----+-----+

1 || SOH | DC1 |  !  |  1  |  A  |  Q  |  a  |  q  |

L   ---++-----+-----+-----+-----+-----+-----+-----+-----+

e    2 || STX | DC2 |  "  |  2  |  B  |  R  |  b  |  r  |

a   ---++-----+-----+-----+-----+-----+-----+-----+-----+

s    3 || ETX | DC3 |  #  |  3  |  C  |  S  |  c  |  s  |

t   ---++-----+-----+-----+-----+-----+-----+-----+-----+

4 || EOT | DC4 |  $  |  4  |  D  |  T  |  d  |  t  |

S   ---++-----+-----+-----+-----+-----+-----+-----+-----+

i    5 || ENQ | NAK |  %  |  5  |  E  |  U  |  e  |  u  |

g   ---++-----+-----+-----+-----+-----+-----+-----+-----+

n    6 || ACK | SYN |  &  |  6  |  F  |  V  |  f  |  v  |

i   ---++-----+-----+-----+-----+-----+-----+-----+-----+

f    7 || BEL | ETB |  '  |  7  |  G  |  W  |  g  |  w  |

i   ---++-----+-----+-----+-----+-----+-----+-----+-----+

c    8 || BS  | CAN |  (  |  8  |  H  |  X  |  h  |  x  |

a   ---++-----+-----+-----+-----+-----+-----+-----+-----+

n    9 || HT  | EM  |  )  |  9  |  I  |  Y  |  i  |  y  |

t   ---++-----+-----+-----+-----+-----+-----+-----+-----+

A || LF  | SUB |  *  |  :  |  J  |  Z  |  j  |  z  |

F   ---++-----+-----+-----+-----+-----+-----+-----+-----+

i    B || VT  | ESC |  +  |  ;  |  K  |  [  |  k  |  {  |

g   ---++-----+-----+-----+-----+-----+-----+-----+-----+

u    C || FF  | FS  |  ,  |  <  |  L  |  \  |  l  |  |  |

r   ---++-----+-----+-----+-----+-----+-----+-----+-----+

e    D || CR  | GS  |  -  |  =  |  M  |  ]  |  m  |  }  |

---++-----+-----+-----+-----+-----+-----+-----+-----+

E || SO  | RS  |  .  |  >  |  N  |  ^  |  n  |  ~  |

---++-----+-----+-----+-----+-----+-----+-----+-----+

F || SI  | US  |  /  |  ?  |  O  |  _  |  o  | DEL |

---++-----+-----+-----+-----+-----+-----+-----+-----+