.T SIM330

.=m5

.s15

.c SIM330

.s

.c a

.c PC SIMULATOR

.s

.c for the

.s

.c VERIFONE TRANZ-330

.c POINT OF SALE TERMINAL

.s10

.c Dunfield Development Services

.c -----------------------------

.c High quality tools for

.c Embedded Development

.c at low prices.

.s

.c http://www.dunfield.com

.s10

.c ?COPY.TXT 2000-2003 Dave Dunfield

.c  -- see COPY.TXT --.

.P

> INTRODUCTION

SIM330 is a PC/DOS based simulator for the Verifone TRANZ-330 point of sale

terminal, which offers debugging capabilities not available on the physical

terminal. Although created as a "quick and dirty" in-house tool, SIM330 has

proven to be very useful in the development of large TCL applications for

the TRANZ-330.

SIM330 was developed without assistance or endorsement from Verifone, and

without "official" information on the inner workings of the TRANZ-330. It

also runs on a physical platform which is quite different from the actual

TRANZ-330 (in particular the modem)... As a result of this, SIM330 is not

a 100% accurate simulation, and it does have it's limitations and quirks.

Most of the differences are not serious, as a case in point, I have

developed several large applications (>5000 lines) which run under SIM330

with no problems, and without having been modified for SIM330 in any way.

This document is intended for TCL developers, who have a working knowledge

of the TRANZ-330. The following sections will focus primarily on the

limitations and differences of SIM330 compared to a physical TRANZ-330

rather than on TCL and TRANZ-330 features. TCL and TRANZ-330 information

can be obtained from Verifone's web site (http://www.verifone.com).

The current configuration and capabilities of SIM330 are the result of our

in-house development requirements. Additions and enhancements to SIM330 are

available on a contract services arrangement. Please contact me for more

information:

.i+20 Dave Dunfield

.i+20 Dunfield Development Services

.i+20 115 Manion Heights Cres.

.i+20 RR#2 Carp, Ontario Canada K0A 1L0

.i+20 http://www.dunfield.com

.i+20 Fax: 613-256-5821

.P

>> SIM330 Command Line

SIM330 is started by entering it's name at the DOS prompt, and specifying

the name of a .TCL, .UDL or .330 file for it to load.

.c SIM330 <filename>.[TCL|UDL|330] [options]

Note1, SIM330 cannot load standard Trans .DTZ image files. Only .TCL, .UDL

or .330 (SIM330 Memory Image) files are recognized.

Mote2: SIM330 does not load symbolic TCL (TCLP) files. You must process the

file with TCLP into a standard non-symbolic TCL file before loading into

SIM330.

The following command line options are available:

>>> -F : Force Din8/RS-232 handshake lines

Forces the handshake lines (DSR and CTS) to always read HIGH through the

TCL '+R' command. Also, causes attempts to change the RTS line with '+R'

to be ignored (RTS remains high).

>>> -S : Start with single-step enabled

Cause SIM330 to trap to the debug interface as soon as any TCL command is

executed.

>>> C=1-4 : Specify the COM port

Identifies the PC COM port (1-4) to be used for communication with the

Din8/Modem interface.

>>> B=<speed><parity><data><stop> : Setup Din8/Modem

Sets the default speed(50-57600), parity(O.E.M.S.N), data bits(5-8), and

stop bits (1-2) for the PC Din8/Modem. Note, these settings can/will be

reset by a '*P', '*R' or '+M' TCL command.

>>> E=0-999 : Execute location

Causes SIM330 to immediately begin executing TCL commands in the specified

memory location at startup.

>>> L=filename : Record execution log

Causes SIM330 to record a log file showing every memory location and TCL

command executed. This allows a developer to trace the exact TCL execution

path through a program.

.p

>>> P=filename : Set Din6 handler

Sets the executable file which will be loaded and run whenever communication

is performed with the Din6 peripheral port.

>>> R=filename : Set cardReader file

Sets the data file which will be read to obtain card track data.

> TERMINAL HARDWARE DIFFERENCES

SIM330 runs on a PC platform, which differs from the TRANZ-330 hardware

in a number of key areas.

>> Modem

SIM330 uses a standard "Hayes AT" compatible modem attached to a PC comm

port. This modem differs quite significantly from the internal modem of

a TRANZ-330, in that it is fully automatic, whereas the TRANZ-330 modem

is entirely manually controlled.

In particular, the TRANZ-330 modem must be explicitly told to dial, then

wait for carrier, then provide originate carrier. The PC modem will do

all of this on it's own.

To compensate for this, SIM330 provides an internal state machine which

tracks the operation of the modem. Operations such as providing originate

carrier are simply "ignored" if the PC modem is progressing through a

state where it has done (or will do) this automatically. This has worked

well in my application, however take note if you are doing anything at

all "out of the ordinary" with the modem.

To work correctly with SIM330, the PC modem must be configured to:

- Indicate true carrier detect on the CD signal.

- Hangup/Disconnect when DTR signal is dropped.

- Indicating ringing with RI signal (for answer apps only).

Please consult your modem documentation to determine how your modem can

be configured to meet these requirements (Most modems are already setup

to work this way). NOTE: SIM330 does not write an "init string" to the

modem - you must configure the modem to power-up in the desired configuration

(usually with "&W" command).

.p

>> Din8 Peripheral Port

SIM330 makes no real distinction between the internal Modem and Din8

serial port. Both of these devices are mapped into a single PC Comm

port. (In a real TRANZ-330, a single serial device is switched between

the modem and Din8 port).

As a result of this, you cannot use the Modem and the Din8 serial port

together in the same application. This is not a problem for the most

common TRANZ-330 configuration which uses the Din8 port for the printer,

because SIM330 provides a separate printer emulation (See below).

>> Printer

SIM330 provides built in emulation of a standard 40 column printer.

All data output to the printer with the TCL 'N' command will scroll

up the printer window on the right side of the PC screen. This

printer is a very simple, and provides only the following special

function characters:

.f-

LF		- Eject paper one line

FF		- Eject paper 6 lines

DC2		- Toggle ribbon color (RED/BLACK)

.f+

NOTE: Unlike a real TRANZ-330 printer, the SIM330 printer is **NOT**

connected to the Din8 serial peripheral. You cannot print by sending

data to the Din8 port. Likewise, the printer will never send any

status information back through the Din8 interface.

.p

>> Din6 Peripheral Port

The Din6 peripheral port is implemented in SIM330 using a loadable

device emulation program. Instead of sending and receiving data with

a physical port, SIM330 loads a program to handle the Din6 device,

transfers a buffer of data to it, and receives a buffer of data from

it. A small amount of "permanent memory" within SIM330 is made available

to this device emulation program so that it can store state and other

information from one execution to the next if desired.

This method of handling the Din6 is nice in that you do not have to

have additional hardware connected to your PC, however it does cause

the limitation that the Din6 can really only be used to emulate

Request/Response type devices (such as a Pinpad).

When data is transmitted to the Din6 device, SIM330 will execute the

program associated with the device (See 'P=' command line parameter),

and pass it two decimal numbers which represent the memory segment and

offset of SIM330's data transfer buffer. This buffer will contain the

data to be sent to the peripheral device. Before it terminates the

peripheral device must fill in this same buffer with it's response

data. The data buffer is terminated by a zero (00 byte).

SIM330's Din6 data transfer buffer is 500 bytes in size, and will be

cleared to zeros when SIM330 starts. Since a packet cannot exceed 250

bytes, at least 250 bytes in this buffer are "unused", and can be used

by the Din6 emulation program to store data in between activations.

SIM330 is supplied with PINPAD.COM, which emulates a basic pinpad

connected to the Din6 peripheral port. For information on this device

emulation program, please refer to the appendices.

>> Card Reader

With no physical card reader, SIM330 cannot read actual cards. A

"virtual" card reader is supported which allows you to enter Track2 data

for up to 8 cards as single lines in an ASCII text file named SIM330.CRD.

When F1-F8 is pressed on the PC keyboard, SIM330 reads the corresponding

line from the file, and processes the data contained therein as data

from the card reader.

Lines beginning with ';' have special meaning:

.f-

;;data		= Card data beginning with single ';'

;=text		= Text shown instead of "swiped" (max 30 chars)

;all-else	= Comment - ignored

.f+

When F1-8 is pressed, SIM330 first looks for the SIM330.CRD file in the

current directory. If it is not found, SIM330 will look for a SIM330.CRD

file in it's home directory instead.

NOTE: If you are simulating a TRANZ-330 which utilizes a Track1 card

reader, simply put Track1 data into the SIM330.CRD file.

.p

> TERMINAL SOFTWARE DIFFERENCES

This section describes differences and limitations in the SIM330

implementation of the TRANZ-330 operating software/firmware.

>> Memory

SIM330 compresses it's memory data in a similar fashion to the TRANZ-330

(Numbers stored as 2 decimal digits to a byte), however since I did not

have access to the actual compression algorithm used by Verifone, I had

to design one of my own. This is the main reason that SIM330 cannot load

the precompressed .DTZ files produced by TCLOAD.

This difference in memory compression should be completely invisible to

the application software, since the application never "sees" compressed

memory.

SIM330 includes a memory manager, which will automatically assign and

release memory as TCL locations are read and written, however this memory

manager peforms only limited "garbage collection". This means that if

operated over a long period of time, SIM330's memory could become fragmented,

and the amount of memory available to an application would be reduced. This

is not normally a problem, as SIM330 was never intended as a long-term use

application. It's primary purpose is as a TCL debugger, and as such, it

normally is used for fairly short periods of time.

Also, to help offset the limited garbage collection, SIM330 provides more

memory than a standard TRANZ-330. Like the TRANZ-330, SIM330 has 32k of

memory, however unlike the TRANZ-330, all of this memory is available to

the application (The TRANZ provides only 24k of application memory).

The SIM330 memory manager also does not have the 120 character limitation

on memory location size that the TRANZ-330 has. This limitation is artificially

applied during loading (SIM330 will abort if a memory location in the load

file is longer than 120 characters), however SIM330 does not apply the

limitation when data is written to memory from within the TCL program.

.P

>> Buffers

SIM330 was written in 'C', and has been influenced by the 'C' notion that

a zero (00 byte) indicates the end of a string of data. For this reason,

many buffer operations will not proceed past a (00 byte). This should not

be a major problem, because the VISA protocols normally used on the

terminal do not include NUL (00 bytes) in their messages.

One case where a NUL (00 byte) can occur is in the calculated LRC to a

message. SIM330 does correctly handle this occurance in transmitted and

received data.

>> Terminal functions

The following terminal functions are not implemented in SIM330:

.f-

0			- Memory Dialer

BACKSPACE	- Post Dial

FUNC-ALPHA	- Change password

FUNC-0		- Download

FUNC-7		- Recall memory

FUNC-8		- Store memory

FUNC-*		- Unit Send

Func-#		- Unit Receive

*+3			- All diagnostics functions

.f+

SIM330 will simply ignore the '0' and BACKSPACE functions. If any of the

FUNC-x functions are executed, SIM330 will enter it's internal diagnostic

menu (see later section).

>> Internal transaction handlers

SIM330 was primarily designed to debug custom applications which are

implemented entirely in TCL code (usually starting using the 980/981

Idle loop string). Only minimal support is provided for the TRANZ

internal transaction handlers (those started by entering '1-9').

As I have little experience with the internal transaction handlers (I

have so far not made use of them), I have chosen to implement them in

TCL, so that they can be modified in the event that my interpretation

of their operation is incorrect or insufficent for your needs.

In general, the following limitations apply:

.i+2

.i-2 - Only the VISA-1 protocol is supported.

.i-2 - The application MUST provide it's own format (x07) and response

analysis strings (x11) - SIM330 does not provide any default

control strings. The format control string (x05) is not used.

.i-2 - Some communications errors are not detected, or are detected

differently from the standard TRANZ-330 (PC hardware limitations).

.i-2 - The secondary phone number is not utilized.

.i-2 - Multiple transaction mode is not yet supported.

.i-2 - Login strings are not yet supported.

.i-2

.p

I have placed the TCL implementation of the internal transaction

handlers in the Verifone reserved memory locations 991-993 - these

are not otherwise used by SIM330. When 1-9 is pressed at the idle

loop, SIM330 executes the TCL string at location 991. During the

execution of the internal transaction handler, the following special

extensions are available:

The character '@' will be replaced with the key digit that activated

the internal transaction handler. This is most commonly used to

reference the memory location corresponding to the key. For example,

"A@04" will append memory location 204 if the '2' key was pressed

to launch the transaction.

The '+Z' command is available to perform the following functions:

.c +Z1	- Load variables #1-#5 with '*T' timeouts 1-5

The following additional 'F'ixed prompts are available:

.f-

F128		"DIALING"

F129		"WAITING FOR ANSR"

F130		"CONNECTED"

F131		"TRANSMITTING"

F132		"RECEIVING"

F133		"NO ANSWER"

F134		"NO ENQ FROM HOST"

F135		"NO RESP FRM HOST"

F136		"BAD TX COMMUN"

F137		"BAD RX COMMUN"

F138		"HOST DISCONNECT"

F139		"LOST COMM W/HOST"

.f+

NOTE: The above TCL extensions are NOT available unless the terminal

is executing an internal transaction handler.

SIM330 contains a set of default transaction handlers - these are

also supplied in the file ITRANS.TCP. If necessary, you can modify

this file and include it with your application to change the default

internal transaction handlers:

TCLP ITRANS.TCP ITRANS.TCL

SIM330 myapp.tcl ITRANS.TCL

Updates/improvements to these internal handlers are welcome. If you

encounter errors, or find that they are insufficent for you needs,

Please notify me and provide details of what needs to be changed.

NOTE: If you do not modify these routines, you do NOT need to load

them into SIM330 - SIM330 contains an internal copy of the original

ITRANS strings which are available in locations 991-993 during the

execution of internal transactions if no other control string has

been loaded in these locations.

.P

> TCL DIFFERENCES

This section describes differences and limitations which should be considered

with specific TCL commands. Where a TCL command is NOT listed, no appreciable

differences in operation should be expected.

.f-

+D	- DTMF tone dial

Subject to limitations of PC modem as described earlier

Will go off-hook automatically when dialing

*H	- Encrypt PIN and Account Number

Master KEY selection is ignored (not passed to pinpad)

PINPAD.COM supports only one master key

*I	- Character IO over Din8 port

Physical COM port is shared with Modem as noted earlier

+I	- Character IO over Modem port

Physical COM port is shared with Modem as noted earlier

J	- Append Clock Data to Destination Buffer

Option 1 - Set clock, the 'w' Day of week is ignored,

as PC auto-calculates this.

*K	- Abort Process, String or Transaction

options 2&3 are identical, SIM330 does not know about

"multi-trans" which is related to the TRANZ-330 built

in transaction handlers.

+K	- Carrier Control

Subject to the limitations of the PC modem.

Functions are implemented as:

0 - If modem is IDLE, issue "ATO" (Originate)

If modem is RINGING, issue "ATA" (Answer)

If modem otherwise busy, ignore

1 - Lower DTR signal to modem (force hangup)

2 - Issue "ATA"

5&6 - Wait for carrier detect

7 - Skip indicated if carrier detected (connected).

+M	- Set Modem Parameters

a1 - Not used, modem always originates when dialed

N	- Send Destination Buffer to Printer

a and t options are ignored (SIM330's emulated printer

is always ready).

*P	- Specify Communications Characters

a1 parameter is ignored

+P	- Timeout in Milliseconds

Limited to PC's clock resolution of 55ms.

Q	- Append Write Control Character

'm' (multitrans) will never be set.

+R	- Get/Set RS-232 Handshake lines

Output handshake line is RTS on PC serial port.

Can cause interference with modem on PC serial port when Din8

is expected to connect to a printer. See -F command option.

S	- Dial or Set Multiple Transaction function

Only options 0 and 3 are implemented.

0 - Begin dialing number

3 - Issue "ATH1" to modem

.p

T	- Perform Arithmetic Operations

Arithmetic is performed as a 32 bit long, which gives

9 digits of arithmetic precision.

+T	- Character I/O over the Din6 Port

Subject to limitations of request/response interface to

the emulated Din6 peripheral as noted earlier.

*U	- Bar code reader input

Not implemented.

+U	- Set Din6 Communication parameters

Ignored (No physical Din6 communication channel)

*V	- Append Telephone Status Code

SIM330 cannot detect operation of a parallel phone.

Codes 4 and 8 will never be returned.

+V	- Append Signon Message to Buffer

Appends "TZ 330 SIM x.x"

W	- Wait until print buffer is empty

Ignored - SIM330 emulated printer is always ready

.f+

.P

> DEBUG INTERFACE

SIM330 provides a "debug interface" which provides interactive access

to the internal operation of the terminal. This debug interface will

be activated on any of the following events:

.f-

Activate terminal function: FUNC+(7/8/#/0/#).

Press F9 on the PC keyboard.

A preset "breakpoint" memory location is executed.

A format error occurs in a TCL command.

A serious error occurs in the operation of the terminal.

When "single-stepping" the debug interface is entered on

the execution of every TCL command.

.f+

>> Screen Information

SIM330 provides a great deal of information on the Debug interface

main screen, which occurs in roughly this order from top to bottom:

.f-

A text message indicating why the debug interface is active.

The last TCL command executed.

The parameters for the last command (up to 10): type/value

type: N=None, V=Value, 0-9=Variable(0-9), "=String

value: The value returned by this parameter

The amount of available free memory

The current state of the Modem and Keypad

The current content of all 10 '#' variables.

The top nine entries from the execution stack

Indicates memory location of calling '*L' command

The TCL command string currently being executed

Next command to execute is hilighted.

The content of the 5 TCL buffers.

indicators: 'd'=Dest 's'=Source 'b'=Both(Dest&source)

Each buffer write pointer is hilited (should be at end)

A "ruler" showing the current buffer/memory display position

Buffer input pointer position is hilighted

The content of 10 memory locations.

.f+

>> Command Keys

The following command keys are available within the SIM330 debug

interface.

>>> F1-F5 : Edit buffer

Pressing F1-F5 allows you to edit the corresponding TCL buffer (1-5).

Press F1 while in the editor to see the available functions.

>>> F6 : Edit memory

Pressing F6 allows you to edit the content of the memory location at

the top of the memory display list.

.P

>>> F7 : Edit variables

Pressing F7 allows you to edit the current content of the 10 '#' variables.

Press ESC to exit the variable list.

>>> F8 : Edit break list

Pressing F8 allows you to edit a list of memory location which will

"trap" to the debug interface when they are executed. This provides

a means of stopping the terminal at a specific memory location. Press

ESC to exit the breakpoint list.

>>> F9 : Review Batch Data

Pressing F9 activates a viewer which allows you to see the batch

data stored within the terminal. While in this viewer, you can scroll

up and down through the batch headers, and sideways to view the

detail records. Press '?' for more information on the commands

available.

>>> F10 : Terminate SIM330

Exits SIM330.

>>> ENTER : Continue execution

Exits the debug interface, and continues the operation of the terminal.

>>> SPACE : Single-Step

Exits the debug interface, performs one TCL instruction, and re-enters

the debug interface.

>>> TAB : View terminal screen

Hides the debug interface window, allowing you to view the main SIM330

screen. Press any key to return to the debug interface window.

>>> 0-9 : Enter memory address

Entering a three digit number will move the memory display to the entered

location.

>>> Up/Down/PgUp/PgDn : Scroll memory address

Moves the memory display address: +/-1 or +/-10 locations.

>>> Left/Right/Home/End : Scroll display view

Moves the buffer/memory display: +/-1 or +/-10 characters.

.P

>>> ALT-S : Save memory image

Saves a complete image of the simulated TRANZ-330 terminal into a file.

The extension of the file should be ".330".

This memory saves all data locations and batch records, which can be

restored at a later time by loading the .330 file into SIM330. This is

similar to the real TRANS-330 preserving memory during power-off. It is

also a handy means of easily returning to a specific terminal state over

multiple debugging sessions.

> PINPAD EMULATOR

SIM330 includes a basic pinpad emulator. PINPAD.COM is the default

handler for Din6 communication (see SIM330 P= option).

PINPAD.COM should only be run from within SIM330 (except to set the master

key or DUKPT emulation as described below). During normal operation,

PINPAD.COM communicates with SIM330 through the passed data transfer

buffer as described under "TERMINAL HARDWARE DIFFERENCES".

>> Master Key

The emulated PINPAD has only one master key. As distributed, this

key is set to 'FFFFFFFFFFFFFFFF', however it can be "permanently"

changed by executing:

.c pinpad KEY xxxxxxxxxxxxxxxx

where 'x...' is the 16 hex digit key.

>> Packet Commands

PINPAD.COM recognizes the following command packets through the data

transfer buffer. Any other <STX>...<ETX> or <SI>...<SO> packet which

has a valid LRC will result in a single <ACK> response.

Any packet with a bad LRC will activate the pinpad error mode (See below)

and return a NAK back to the calling program.

>>> Transfer Master Key

.i <SI>020xxxxxxxxxxxxxxxx<SO><LRC>

This transfers a new master key 'x...' to the pinpad. Note:

.i+2

.i-2 - The master key is stored within the calling SIM330 program, and

will therefore only remain in effect until SIM330 is terminated (after

which it reverts to the master key programmed into PINPAD.COM).

.i-2 - Only key slot '0' is recognized.

.i-2

.p

>>> Check Master Key

.i <SI>04n<SO><LRC>

Checks for the presense of a master key in the Pinpad slot 'n' (0-9). The

response is <SI>04nx<SO><LRC> where 'x' is 'F' for key present, or '0' for

key not present. PINPAD.COM returns 'F' for slot 0, and '0' for slots 1-9.

>>> Request PIN Entry

.i <SOH>70.aaaaaaaa<FS>xxxxxxxxxxxxxxxxttt<ETX><LRC>

Requests an encrypted PINblock, using the amount 'a...', the encrypted

working key 'x...' and the total amount 't...'. The PINpad will display

"Total $t.tt Enter PIN Press ENTER" and wait for a PIN entry (min. 4

digits). When it has been entered, a '71' Transfer PIN Block packet

will be formatted and sent back to SIM330:

.i <SOH>71.0ll01bbbbbbbbbbbbbbbb<ETX><LRC>

where 'll' is the length of the Pin (04+) and 'b...' is the encrypted

pin block.

>>> Display a String / Reset/Set Idle Prompt

.i <STX>Z2[SUB]text<ETX><LRC>

.i <STX>Z8text<ETX><LRC>

These two commands behave basically the same, the pinpad will activate

and display the indicated 'text' briefly. The <SUB> character used to

clear the screen on a standard pinpad will be ignored (Since PINPAD.COM

does not remain active from one call to the next, it does not have to

clear it's screen).

The only difference between the two packet commands is that for the 'Z2'

command (Display a String), a null string will display a blank screen,

whereas the 'Z8' (Reset/Set Idle Prompt) will translate a null string

into the standard '<-------' idle prompt.

Note that these commands activate the pinpad only long enough to show the

string. This allows you to see that a string has been written to the pinpad

by your application. The pinpad will NOT remain activated to continue showing

the string, since SIM330 is suspended while PINPAD.COM is running.

While the PINPAD is showing the display, you may press SPACE to terminate

it immediately.

.p

>>> Request Key Code / Request Key Value

.i <STX>Z40ttt<ETX><LRC>

.i <STX>Z42ttt<ETX><LRC>

The pinpad activates, showing whatever was last on the display, and waits

for ttt seconds (0-255) for a key. For Z40/Request Key Code, the key codes

are returned in a 'Z41' message:

.i <STX>Z41cc<ETX><LRC>

The key code values cc are:

.f-

1=1		4=5		7=9		*=13

2=2		5=6		8=10	0=14

3=3		6=7		9=11	#=15

.f+

For Z42/Request Key Value, the ASCII value of the key pressed '0-9 * #'

or '?'=timeout is returned in a 'Z43' message:

.i <STX>Z43a<ETX><LRC>

>> Error Mode

When an error is detected in the PINPAD operation (Bad command, LRC etc.)

the pinpad will open a screen window showing the cause of the error, and

the content of the received command packet, then prompts you to press ENTER

at which point PINPAD will terminate and send a NAK back to the calling

program. If it is appropriate, you will also have the option to press

SPACE to proceed and ignore the error (such as with bad LRC).

Anytime PINPAD.COM is active (pinpad visible on the screen), you can press

F10 to activate error mode. At this point, you can view the input command,

press SPACE to proceed, or ENTER to terminate and send back a NAK.

.p

>> DUKPT Emulation

The PINPAD simulator provides very limited support for DUKPT. You may enable

this support with:

.c pinpad DUKPT <10-20 digit key serial number>

When DUKPT is enabled, PINPAD will simply accept and return PIN (70 and 71

packets) in the DUKPT format. The specified key serial number is transmitted

in the (71) response packet, however the key is simply encrypted using the

terminal master key. In other words, the PINPAD operates in the DUKPT format,

however you must specify the key serial number and actual encryption key to

be used.

To disable DUKPT emulation, and set the pinpad to operate as a standard

master key pinpad, use:

.c pinpad DUKPT OFF

NOTE: Like the verifone pinpad, the set master key (02) and check master

key (04) functions are disabled when PINPAD is set to DUKPT emulation.