.T TRANZ-330 Essentials

.=m2

.$1FULL

.s15

.c TRANS-330

.s5

.c Essential Information

.c for

.c Applications Developers

.s11

.c This document contains sensitive information

.c DO NOT DISTRIBUTE

.s12

.c ?COPY.TXT 2000-2006 Dave Dunfield

.c  -- see COPY.TXT --

.s5

.c www.dunfield.com

.p

> INTRODUCTION

This document contains a brief summary of the information that I have found

to be the most useful in programming and working with the TRANZ-330 terminal.

It is intended as a "quick reference", not a detailed manual. For more

definitive information, please refer to the following official VeriFone

documents:

.f-

Document Title					 VeriFone part number

-----------------------------------------------------

TRANZ-330 Reference Manual						00483

TCL Programmer's Manual							00368

TCLOAD Reference Manual							10016

Pinpad 101  Reference and Programmer's Manual	10420

Pinpad 201  Reference and Programmer's Manual	10297

Pinpad 1000 Reference and Programmer's Manual	25392

Pinpad 2000 Reference and Programmer's Manual	25647

Printer 250 Reference and Programmer's Manual	10692

.f+

NOTE: The first three documents are available for free download on

the VeriFone web site:

http://www.verifone.com/services/techsupport/tcl_tranz.html

>> Disclaimer

This document was created without assistance or endorsement from Verifone,

and may contain errors. The author makes no guarantee that the information

is accurate.

The information contained in this document provides access to features of

the TRANZ-330 which allow modification of the stored financial application.

Operator errors or intentional misuse of these features could result in

the corruption of the financial application. Any use of this information

is entirely at your own risk - the author accepts no responsibility for

any damages resulting from such use.

.p

.f-

> KEYPAD FUNCTIONS

>> Recall memory

Press FUNC 7

Enter system password if prompted (see "Store memory")

Enter 3-digit memory location

0-9 = Select new location

ENTER = Move to next location

ALPHA = Back to previous location

BKSPACE = Modify location (see "Store memory")

>> Store memory

Press FUNC 8

Default system password: 1 ALPHA ALPHA 66831

Enter 3-digit memory location

Enter new value and press ENTER

>> Serial Receive (download)

Press FUNC #

Enter system password if prompted (see "Store memory")

Send data from PC using TCLL, TCLOAD or VLOAD

or transmit from another unit using FUNC *

>> Serial Send (upload)

Press FUNC *

Enter system password if prompted (see "Store memory")

Receive data on PC using TCLL

or into another unit using FUNC #

>> Clearing all memory

Press '*'+CLEAR

Password: 8 ALPHA 0 ALPHA 8 ALPHA 5361041 ALPHA

- All memory locations are cleared

- Factory defaults are installed

- System password is reset to: 1 ALPHA ALPHA 66831

>> TRANZDDS / ZonTalk Download

If full download, clear memory (see above)

Preset these memory locations (see above)

000=dial-number			011=dial-rate

001=serial-number		010=dialing-type

019=application-ID		960=download-speed

Press FUNC 0

Press ENTER for full, '*' for partial download.

Enter phone number, ID if prompted.

.p

>> Change system password

Press FUNC ALPHA

Enter old system password

Enter new system password

Enter new system password again to verify

>> Bypass system password

Press '*'+CLEAR

Password: 8 ALPHA 00 ALPHA 3675 ALPHA 328 ALPHA

- System password is bypassed for current session

>> Reset system password

Press '*'+CLEAR

Password: 800 ALPHA 3 ALPHA 684 ALPHA 35 ALPHA 3

- System password is reset to: 1 ALPHA ALPHA 66831

>> Set SW LOCK key

Press '*'+CLEAR

Password: 80 ALPHA 8 ALPHA ALPHA 52 ALPHA 786 ALPHA ALPHA 11

Enter SW LOCK key

- Terminal will NOT operate unless location 995 (included in

application download) contains the specified key value.

>> Diagnostics

Press '*'+'3' (together)

0=Display Messages

1=Memory (RAM) test

2=Keypad test

3=Display test

4=Card reader test

6=Ring detector test

7=Full memory test    (8085 ALPHA 36104 ALPHA 1 ALPHA)

8=Display/Memory test (8085 ALPHA 36104 ALPHA 1 ALPHA)

9=Demo Transaction

*=RS232 Port Test

ALPHA=Set clock (enter system password if prompted)

BKSPC=Bar Code Test (versions prior to 3.6 only)

CLEAR=Exit

.p

> CABLE PINOUTS

All views are of connector looking at end of cable.

>> Serial Download cable

TRANZ-330                  PC: 9-pin serial

------------------------------------------------------

2-o     o

5-o   o   o-1,6,8            5   4   3   2   1

4-o     o-3                   9   8   7   6

7-o

------------------------------------------------------

1-DCD    3-TXD    5-GND    7-RTS    9-RI

2-RXD    4-DTR    6-DSR    8-CTS

>> Serial printer cable

TRANZ-330                  Printer 200/250

------------------------------------------------------

5-o     o                    ___ ___

7-o   o   o-4                / 6 7 8 \

3-o     o-6                | 3 4   5 |

4-o                      \. 1 2 ./

------------------------------------------------------

1=N/C   2=N/C    3=RTS    4=CTS

5=RXD   6=TXD    7=GND    8=N/C

>> PinPad Cable

TRANZ 330                     PinPad

------------------------------------------------------

a-o     o-2                    1 2 3 4

1-o                      []_|_|_|_|_[]

3-o     o-4                 [___________]

b-o                          [___]

------------------------------------------------------

1=+9v for Pinad     4=Gnd (through  47ohm resistor)

2=PinPad RXD        a=+5v (through 4.7ohm resistor)

3=PinPad TXD        b=BarCode RXD

.p

> TCL SUMMARY

Am.s                  B Append data from memory location [0] to dest.

*As                   D Open new batch [s=0] #commands to skip on success

+A#v.a.[#]c.s         V Compare variable

Bs.d                  B Select source and dest buffers (reset extraction ptr)

*Ba                   D Select batch for read ops

+B#v,a                C Calculate Block Character Check of dest buffer

Ca.s                  F Verify check digits

*Ca.s                 D Copy dest buffer to batch

+Ca.s                 P Buffer comparison

Dn                    B Delete characters [1]from the end of dest buffer.

*Da.s                 D Delete batch, records or headers

+Dt                   C Dial DTMF digits in dest buffer, [t=5]x10ms duration

Es1a.j.s2.i.s3.a2.c   E Input from cardreader or Keypad

*Ea.s                 D Append batch detail to dest buffer

+En1.n2.a             E Input and display with decimal places

Fn.a                  O Display fixed prompt

*Fs.a                 D Detect data capture errors

+Fa                   C Enable [0], Disable (1) and send ACK immediately (5)

G                     B Clear dest buffer

*Ga                   O Terminal beep

+Ga.s                 D Append detail record to batch

Hs.c1.j.c2            B Search for string/constant c1 [28] in source buffer

*Hi.j.a               S Encrypt PIN and account number

+Hs                   S Wait for encrypted block from PIN pad -> dest buffer

Is.a.c                P Compare source buffer against string/constant

*Ia.t.s.n             C Character I/O over DIN 8 port

+Ia.t.s.n             C Character I/O over MODEM port (parms same as *I)

J1or'fmt'             B if '1', set time from dest buffer: WYYMMDDHHMMSS

*Ja                   O Justify display

+Ja                   B Buffer modification [0]

Ka.j.s.i              E Input from keypad only

*Ka                   P Abort process, string or transaction

+Ka.s                 C Carrier control

Lm                    P Link to control string [re-execute current]

*Lm.n                 P Call subroutine at location m, execute [n=1] times

+La                   P Lock [0] and unlock [1] the keypad

Ms.a.c                E Input from cardreader only

*M                    P Exit subroutine (breaks subroutine loops)

+Ma1.a2.a3.a4         C Set modem parameters

Nn.a.t                O Send destination buffer to printer

*N#v.[#]c             V Set variable v [0] to constant/variable [0]

On                    B Move extraction pointer within src buffer [1] (+/-n)

*O#v.o.[#]c.s         V Perform arithmetic on variables

Pm                    O Display custom prompts (update only when entry occurs)

*Pc1.c2.n.a1.a2       C Specify communications parameters

+Pt                   P Pause [t=1000] milliseconds

Qc                    B Append Write Control Character [0]

*Q#v.b.a              V Set variable to content of buffer

+Q#v.a                V Append variable to destination buffer

Rc                    B Append constant value/string [28] to dest buffer

*Ra1.a2.a3.a4         C Select communication device speed

+Ra.s                 C Get/Set RS-232 handshake lines

Sa.m                  P Dial or set multiple-transaction function

*Ss.n.a               D Verify memory available

+Sb.a.#v              B Get size of buffer b [dest] into variable #v [0]

.p

Tm.o                  B Perform arithmetic between memory location and dest

*Tt1.t2.t3.t4.t5.a    C Specify communications timeouts

+Ta.t.s.b             C Character I/O over Din6 port (parms same as *I)

Um.b                  B Copy buffer [0=dest] to memory location [0]

*Us.a1.a2.s2          E Barcode reader input

+Ua1.a2               C Set Din6 communications parameters

Vb.a.t                O Display buffer

*V                    C Append phone status to destination

+Va                   B Append signon message [0=signon message]

Wa.s                  P Wait until print buffer is empty

*Wt.s.a               P General wait (time or key -> dest buffer)

Xn.c                  B Append n [0=All] from extract ptr in src to dest

*Xm1.m2.s             E Select account number range

+Xc.a1.a2             C Transmit data to network

Ys                    B Append next non-blank memory location (for dump)

*Yt.s1.s2.s3.s4.s5.s6 E Check input devices

+Yc.t.n.a.s           C Receive network data with timeout

Zn.c.j                B Padd dest buffer to length n with char [32], with

*Za                   D Calculate # free bytes and append to dest buffer

>> Buffer Operations

1=TRANSMIT  2=RECEIVE  3=DATA_CAPTURE  4=PRINT_LINE  5=TEMP

Am.s			- Append data from memory location [0] to dest.

s: # commands to skip when location is empty

Bs.d			- Select source and dest buffers (reset extraction ptr)

s=9 = Reset extraction ptr in current source buffer.

Dn			- Delete characters [1]from the end of dest buffer.

G				- Clear dest buffer

Hs.c1.j.c2	- Search for string/constant c1 [28] in source buffer

s is number of commands to skip if not found [0]

j is maximum number of characters to search [to end],

c2 is decimal value of end marker [0],

J1or'fmt'		- if '1', set time from dest buffer: WYYMMDDHHMMSS

if 'fmt', append time to dest buffer in this format

A=yr2 B=mo2 C=dy2 D=hr12 E=min F=sec G=A/P

H=yr4 I=mo3 j=mo  K=hr24 L=dy3 M='M' N=day

O=':' P='=' Q=day(Sun=0-Sat=6)

+Ja			- Buffer modification [0]

0=Append src to dest, remove '.' and ','s

1=Append src to dest, add '$' and decimal point

2=Append src to dest, convert control chars to print

3=Copy source to dest using mask in dest

4=Logically AND src and dest -> dest

5=Logically OR  src and dest -> dest

6=Logically XOR src and dest -> dest

On			- Move extraction pointer within src buffer [1] (+/-n)

Qc			- Append Write Control Character [0]

WCC format: 01ttttmc

tttt - From c parameter (4=0001 8=0010 etc.)

m    - 0=Single 1=Multi transaction

c    - Account data came from: 0=Keypad 1=Cardreader

Rc			- Append constant value/string [28] to dest buffer

+Sb.a.#v		- Get size of buffer b [dest] into variable #v [0]

a: [0=Uncompressed]  1=Compressed

.p

Tm.o			- Perform arithmetic between memory location and dest

m = [0] memory location to be used

o = [0]: m = m + dest   Output can be up to 15 digits

1: m = m - dest   All non digit chars are ignored

2: m = m * dest	  '-' anywhere reads as minus

3: m = m / dest

4: m = m % dest

Um.b			- Copy buffer [0=dest] to memory location [0]

+Va			- Append signon message [0=signon message]

Xn.c			- Append n [0=All] from extract ptr in src to dest

c is end marker [0] (copy up to but not including)

Ys			- Append next non-blank memory location (for dump)

s [0] is # commands to skip after all have been read

Zn.c.j		- Padd dest buffer to length n with char [32], with

justification [0=Left] 1=Right 2=Center

>> Output and Display

Fn.a			- Display fixed prompt

n: Fixed prompt number

a: [0=Display if no pend input] 1=Append to dest buff

*Ga			- Terminal beep

a: 0=[Normal]  1=Error  2=Normal-none-if-turned-off

*Ja			- Justify display

a: [0=Rightmost]  1=Leftmost

Nn.a.t		- Send destination buffer to printer

n: number of CRLF pairs to append [0]

a: out-of-paper: [0=Immed] 1=Wait(t) 2=Prompt/Enter

t: 1/2second time to wait when (a==1) [0]

Pm			- Display custom prompts (update only when entry occurs)

m: memory location contining prompt [0]

Vb.a.t		- Display buffer

b: buffer to be displayed [0=dest]

a: [0=Wait/ENTER] 1=Allow multi-host 2=Wait/Key/time

t: 1/2 second time to wait when (a==2) [0]

.p

>> Data Entry

Es1.a.j.s2.i.s3.a2.c - Input from cardreader or Keypad

s1: # commands to skip when input is from card [0]

a : Input type flag

1=Alphanum 2=num [3=alphanum-chars] 4=hidden

5=alphanum/hidden 6=num/hidden 7=Alphanum/password

j : Max # of input chars from keypad [60]

s2: # commands to skip after NULL entry from kpad

i : Min # of input chars from keypad [0]

s2:a2:c - 380 only

+En1.n2.a		- Input and display with decimal places (keyboard)

n1: # decimal places to display [2]

n2: Max # input digits [10]

a : type of radix to use [0]

Ka.j.s.i		- Input from keypad only

a: Input type flag

1=alphanum-char 2=num 3=alphanum-char

4=numbers/2forcedDecimal 5=alphanum/hidden

6=num/hidden 7=Alphanum/password

j: Max # input chars [60]

s: # commands to skip after a NULL entry

i: Min # input chars [0]

Ms.a.c		- Input from cardreader only

s: # command to skip if three bad cards read [0]

a: track selection options:

[0=Trk2] 1=Trk1 2=Trks1&2 3=Trk3 4=Trks2&3

c: Track separater [29]

*Us.a1.a2.s2	- Barcode reader input

s :	#cmds to skip when barcode read (keyboard ->next)

a1: Barcode algorithm (0=3 of 9)

a2: 0=No Luhn check 1=Luhn check/code 39 if data numeric)

s2: (Indus EEPROM only) #cmds to skip on bad barcode

*Xm1.m2.s		- Select account number range (from source buffer)

m1: Memory location of first account range (0-255) [40]

m2: Memory location of last account range (0-255) [59]

s : # commands to skip if account is within range

rang_entry= low . high . data (2 '.'s required!)

*Yt.s1.s2.s3.s4.s5.s6 - Check input devices

t : seconds to wait for data from device [0=infinite]

s1-6: # commands to skip when input from:

cardreader, keypad, barcode, RS-232, modem, Pinpad

.p

>> Process and Control Flow

+Ca.s			- Buffer comparison

a: [0=source==dest] 1=source>dest 2=source<dest

s: # commands to skip if compare is true [0]

Is.a.c		- Compare source buffer against string/constant

s: # commands to skip if compare is true (+/-) [0]

a: [0=Unconditional] 1:== 2:!= 3:< 4:>

5:single-transaction 6:!first-pass-of-multitran

7:Decrement variable 'c' skip if !0

c: constant/string [28] or #variable (a==7 only)

*Ka			- Abort process, string or transaction

Clears all system buffers, stops printing.

a: [0=Idle prompt] 1=ExistingDisp

2=HangUp-leave-multi-tran-enabled-continue

3=HangUp-disable-multi-tran-continue

Lm			- Link to control string [re-execute current]

*Lm.n			- Call subroutine at location m, execute [n=1] times

+La			- Lock [0] and unlock [1] the keypad

*M			- Exit subroutine (breaks subroutine loops)

Not needed at the end of a control string

+Pt			- Pause [t=1000] milliseconds

Sa.m			- Dial or set multiple-transaction function

a: [0=Dial] 1=Enable-m-tran_no_view

2=enable-m-tran-allow-view 3=off-hookNOW

m: memory location to dial [0]

Wa.s			- Wait until print buffer is empty

a: [0=WaitEmpty] 1=SkipIfReady 2=SkipIfBufferEmpty

s: # commands to skip (a > 0 only)

*Wt.s.a		- General wait (time or key -> dest buffer)

t: Timeout in seconds (1-65) [0=No time]

s: # commands to skip when timeout or keypress

a: [0=0-9,*,#] 1=+ENTER 2=+BKSPC,ALPHA,ENTER 3=None

>> Variable Operations

+A#v.a.[#]c.s	- Compare variable

v: variable to compare [0]

a: 0:v==c 1:v>c 2:v<c [0]

c: Constant/#variable to compare against [0]

s: # commands to skip if compare is true [0]

*N#v.[#]c		- Set variable v [0] to constant/variable [0]

*O#v.o.[#]c.s - Perform arithmetic on variables

v: variable for first operand [0]

o: 0=+ 1=- 2=* 3=/ 4=% 5=AND 6=OR 7=XOR [0]

c: Constant/#variable for second operand [1 A=0-4]

s: # commands to skip if result of operation in 0 [0]

*Q#v.b.a		- Set variable to content of buffer

v: Variable to set [0]

b: buffer to be used [dest]

a: 0=string->num 1=ASCII->num

+Q#v.a		- Append variable to destination buffer

v: variable to read

a: 0=num->string 1=num->ASCII

.p

>> Communication Control

+B#v,a		- Calculate Block Character Check of dest buffer and place

in variable [0]. A: [0=LRC] 1=CRC (Initial BCC=variable)

+Dt			- Dial DTMF digits in dest buffer, [t=5]x10ms duration

+Fa			- MODEM ACK: Enable [0], Disable (1), Send immediately (5)

*Ia.t.s.n		- Character I/O over DIN 8 port

a:	[0=send TRANSMIT buffer]

1=append TERMCHAR (*P cmd) + LRC and send TXBUF

5=Wait for n characters RX, append to dest buffer

6=Wait for 1 char RX, append to dest, if STX, wait

for packet, perform LRC, NAK bad packet

7=Wait for pass character or packet. Rexmit last

data if NAK rx, append rx char or packet to dest

buffer; LRC check and NAK on bad packet

t:	RX timeout in seconds [0=notimeout] (rx only)

s:	# commands to skip if data rx (rx only)

n:	min # chars to wait if (a==5 rx only) [1]

(EOT=4 ENQ=5 ACK=6 NAK=21 CR=13 vACK=7 vWACK=17 vWAIT=18)

+Ia.t.s.n		- Character I/O over MODEM port (parms same as *I)

+Ka.s			- Carrier control

a:[0]=Raise Carrier

1=Drop Carrier (clears modem RX buffer)

2=Raise Answer Tone

5=Wait for debounced carrier detect (200ms)

6=Wait for debounced carried with 500ms silence

7=Skip s commands if carrier detect

s:	# commands to skip (a == 7 only)

+Ma1.a2.a3.a4	- Set modem parameters

a1:	[0=No change] 1=Set originate  2=Set answer

a2: [0=No change] 1=7E1pass7 2=8N1pass7 3=8N1pass8

a3: [0=No change] 1=enable 2=disable retrans on NAK

a4: [0=No change] 1=enable 2=disable retrans on ENQ

*Pc1.c2.n.a1.a2 - Specify communications parameters

c1: [ETX] character signalling end of request packet

c2: [STX] receive exit character (1-127)

n :	[3] number of NAK's sent/received before hangup

a1: [0=No change] 1=disable 2=enable multi-ACK protocol

a2: [0=No change] 1=300 2=1200 baud

*Ra1.a2.a3.a4	- Select communication device speed

a1: [0=Internal modem] 1=RS-232 port

a2: [0=300] 1=600 2=1200 3=2400 4=4800 5=9600 6=19200

a3: [0=No change] 1=NoHandshake 2=TXonMARK 3=TXonSPACE

a4: [0=No change] 1=7E 2=8N data bits/parity

+Ra.s			- Get/Set RS-232 handshake lines

a:[0]=Drop output handshake

1=Raise output handshake

5=Skip 's' commands if signal1 (DSR) high

6=Skip 's' commands if signal2 (CTS) high

7=Skip 's' commands if signal1&2 are high

s: # commands to skip (a==5-7 only) [0]

.p

*Tt1.t2.t3.t4.t5.a - Specify communications timeouts

t1: End of dial to carrier detect [45 sec]

t2: Carrier detect to ENQ [30 sec]

t3: send packet to response rx [45 sec]

t4: # t3's to allow before hang-up [1]

t5: Response packet to EOT (2 sec)

a : [0=No change] 1=enable 2=disable EOT timeout

+Ta.t.s.b		- Character I/O over DIN 6 port (parms same as *I)

+Ua1.a2		- Set DIN 6 communications parameters

a1: [0=300] 1=600 2=1200 3=2400 4=4800 5=9600

a2: [0=No change] 1=7E1pass7 2=8N1pass7 3=8N1pass8

*V			- Append phone status to destination

(1=off-hook 2-off-hook+carrier 3=on-hook+lineOK

4=Line!OK 5=on-hook+ring 6=off-hook+wait_for_line

7=off-hook+dialing 8=off-hook+wait_for_answer)

+Xc.a1.a2		- Transmit data to network

c : string/value to send [0] (sent AFTER a1 data)

a1: login parm: 0=No 1=name 2=pass 3=loc 4=loc_pass

a2: [0=Nothing] 1=SPACE "break"

+Yc.t.n.a.s	- Receive network data with timeout

c: termination char/value to match [13]

t: time [30000ms] to wait

n: number of tries [1]

a: [0=no output] 1=right 2=left 16 chars till c match

s: # command (+/-) to skip on no data [0]

>> Data Capture and Batch Functions

*As			- Open new batch [s=0] #commands to skip on success

*Ba			- Select batch for read ops

a: [0=Current/Young] 1=Oldest 2=Next 3=Match dest buffer

*Ca.s			- Copy dest buffer to batch

a: [0=Copy to end of open batch] 1=replace current record

s: # commands to skip if successful

*Da.s			- Delete batch, records or headers

a: 0=[Selected batch/headers]  1=All detail records

for selected batch  2=All batches

s: # commands to skip if successful

*Ea.s			- Append batch detail to dest buffer

a: [0=Next detail] 1=1stDetail 2=BatchHeader

3=NextDetail matching data in source buffer

s: # commands to skip if successful

*Fs.a			- Detect data capture errors

s: # commands to skip if error value != a [0]

a: Code identifying type of error [0=no error]

+Ga.s			- Append detail record to batch

a: [0=append to selected batch] 1=Insert at front

s: # commands to skip if successful

*Ss.n.a		- Verify memory available

s: # commands to skip when available > n [0]

n: number of free bytes required

a: Type of memory [0=Program] 1=Batch

*Za			- Calculate # free bytes and append to dest buffer

a: [0=Program] 1=Batch

.p

>> Security/Encryption

*Hi.j.a		- Encrypt PIN and account number

i: Min # pin digits (4-12) [4]

j: Max # pin digits (4-12) [12]

a:	1=PIN from PinPad Account from TRANZ

2=PIN and account number from PinPad

+Hs			- Wait for encrypted block from PIN pad -> dest buffer

s: # commands to skip if successful

>> Fraud control

Ca.s			- Verify check digits

a:[0]=verify LUHN check digit within last data entry

1=Verify LUHN check within the destination buffer

2=Verify MOD11 check digit on the dest buffer

s: # of commands to skip if digit is invalid

.p

> TRANZ-330 MEMORY MAP

000		X	20	Download phone number

001		X	10	Serial number

002		-	-	TRANSMIT buffer (1)

003		-	-	RECEIVE buffer  (2)

004		9	6	Program Data MMDDYY

005		9	4	Message sequence number

006		9	2	Number of chars to scroll

007		9	1	Multi-transaction timeout 1-8x20sec 9=90min

008		9	5	Reserved (Printer type 5=roll 18=slip)

009		9	1	Beeper 0=On 1=Off

010		9	2	Dial 0=Tone 1=std_pulse 2=10-n_pulse 3=n+1

011		9	1	Dial 0=5 1=7 2=10 3=15 4=20 9=user-spec pulses per sec.

012		9	1	Parallel phone 0=not_connected 1=connected

013		9	1	Number of retries

014		9	1	TelTest 0=+A/A1+test 1=+A/A1-test 2=-A/A1+test 3=-A/A1-test

015		9	3	Delay before auto-answer

016		X	120	Encrypted working key  Master key pointer

017		9	1	Recall, Clock, Unit/Unit Restriction (0=no_password)

019		X	7	Application Identification

020-029	X	60	Logon strings #0-9

030		X	120	Idle prompt

031-036	X	120	Function key #1-6 (031-036) control strings

037		X	120	Reserved (Out of memory control string)

038		X	120	Auto answer control string

039		X	120	Function key #9 control string

040-099	X	120	General locations

x00		X	32	Primary phone number

x01		X	32	Secondary phone number

x02		X	32	Call center phone number

x03		X	32	Referral phone number

x04		X	46	Merchant/Terminal ID

x05		9	3	Message format flag

x06		9	3	Fraud control flag

x07		X	120	Transaction control string

x08		X	16	Transaction type prompt

x09		9	4	Floor limit

x10		X	120	Response analysis control string

x11		X	120	Auxiliary control string

x12		9	1	Multi-transaction group code

113-199	X	120	General locations

213-299	X	120	General locations

313-399	X	120	General locations

413-499	X	120	General locations

513-599	X	120	General locations

613-699	X	120	General locations

713-799	X	120	General locations

813-899	X	120	General locations

913-949	X	120	General locations

950		9	1	Printer 0=none 1=GenericRoll 2=200/250 3=100/150

951		9	3	# line feeds for roll printer [6]

952		9	1	GenericRoll baud [0=3] 1=6 2=12 3=24 4=48 5=96 6=192 x100

953		9	1	GenericRoll format [0=7E2] 1=8N2

954		9	1	GenericRoll handshake [0=Hardware] 1=None

955-957	X	60	Reserved

.p

958		9	1	Bell/CCITT [0=Bell] 1=CCITT

959		X	60	Reserved

960		9	1	DialUp Ul/Dl speed 1=300 2=1200

961		X	60	Silence		\

962		X	60	Dial Tone	+- CCITT units only

963		X	60	Busy		+- otherwise Reserved

964		X	60	Ringback	/

965		9	1	AutoAnswer speed 1=300 2=1200

966		9	1	AutoAnswer Processing

967		9	1	AutoAnswer pkt inactive timeout: 0=20s 1-8x20s 9=90min

968-969	X	60	Reserved

970		9	1	DIN6: 0=Empty 1=BarCode 2=PinPad 3=Generic

971		9	3	Extended redial flag

972-974	X	60	Reserved

975		9	1	Line recovery time [0=3 seconds]

976		X	60	Moment of Silence

977		9	1	MemManag reclaim 0=1_Store 1=1_Abort 2=All_Abort >3=None

979		X	120	Abort control string

980		X	120	Delay Executing Idle Loop control string [0=Disable]

981		X	120	Idle loop control string

982		X	60	Idle loop phone number

983		X	120	Idle loop response analysis control string

984		X	60	Idle loop inactivity timeout

985		9	1	Host number for card transaction

986		9	1	Host number for bar-code transaction

987-989	X	60	Reserved

990		X	120	Communication error control string

991-994	X	60	Reserved

995		X	7	SW LOCK KEY

996		X	60	Reserved

997		X	120	VeriFone control string

998		X	60	Reserved

999		X	60	Programming Error Recovery Log

.p

> PROGRAMMING ERROR

.f+

The TRANZ terminal will perform a checksum test of it's internal memory

while performing various operations, and if the checksum fails (indicating

corrupted memory), a "PROGRAMMING ERR X" message will occur, where 'X'

indicates the operation that was in progress:

.f-

0 = Powerup

1 = TCL store begin

2 = TCL store complete

3 = TRANZDDS/ZonTalk download begin

4 = TRANZDDS/ZonTalk download complete

5 = Keypad store begin

6 = Keypad store complete

7 = TCLL/unit-to-unit download begin

8 = TCLL/unit-to-unit download complete

.f+

>> Recovery

.f-

Press 'FUNC'+'1'	= Clear ALL memory, reset to initial state

Press '5'+'3'		= Log error (999) and continue

.p

> PINPAD NOTES

>> General Diagnostics

Press '*'+'3' (together)

Password: 83746

1=Memory (RAM) test

2=Continuous RAM test

3=EPROM checksum

4=Key test

5=Display test

6=Display Serial number

7=Uart Loopback Test

8=Card reader test/Display baud rate

9=Cardreader ON/OFF/Display key management

0=PIN-PAD/PIN-PAL prompt toggle

>> Manufacturing Diagnostics

Press '*'+'2' (together)

Password: 844747746

1=Destructive MEM test

2=Init NV/MKEY RAM

3=Languages

4=/Display ALL messages

5=/Set baud rate

6=/Set KEY management

>> Master key insertion

Press '1'+'#' (201) or '#'+'1' (1000)

Key is entered as 16 decimal numbers from 0 to 15, representing

a 16-digit hexidecimal master key.

>> Physical Interface

+- | | | | -+	1 = Ground

|  1 2 3 4  |	2 = Serial RX (data to PinPad)

+---+   +---+	3 = Serial TX (data from PinPad)

=====		4 = +9v DC unregulated

.p

>> Packet command formats

>>> Interactive Diagnostic routine

<SI>01nn<SI><LRC>	- Execute diagnostic test

nn=	01 = RAM test/one-time

02 = RAM test on/off toggle/continuous

03 = EPROM checksum

04 = Keypad reliability

05 = Display reliability

06 = Serial number check

07 = Uart loopback test

08 = Cardreader test/card display

09 = Cardreader on/off toggle

00 = PIN PAL/PIN PAD display toggle

Response: <ACK> immediately, <EOT> when finished

>>> Transfer Master Key

<SI>02nxxxxxxxxxx<SO><LRC>

n	= Master key slot ('0'-'9')

x..	= Master key (16 hex digits)

Response: <ACK>, then echo of command packet

Controller must verify send <ACK> if correct.

(<NAK> if LRC bad, Send <EOT> if LRC ok but key bad)

Pinpad stores new Mkey if <ACK> received, sends <EOT>

>>> Decrypt/Encrypt Loopback Test

<SI>03rmmmmmmmmmmmmmmmm<SO><LRC>

c	= 0:Encrypt Test  1:Decrypt test

m..	= ASCII message from National Bureau of Standards test data

Response: <ACK>, then: <SI>03xxxxxxxxxxxxxxxx<SO><LRC>

x..	= ASCII message (hex formal) returned by pinpad

Controller must respond with <ACK>, then pinpad sends <EOT>

>>> Check Master Key

<SI>04n<SO><LRC>

n	= Master key slot number ('0'-'9')

Response: <ACK>, then: <SI>04r<SO><LRC>

r	= 0:No master key  F:Master key present

Controller must respond with <ACK>, then pinpad sends <EOT>

>>> Transfer Serial Number

<SI>05vvvyymmddfbbgggg<SO><LRC>

v..	= Version number (3 digits)

yy	= Release date: year

mm	= Release date: month

dd	= Releast date: day

f	= Production facility code

bb	= Production batch code

g..	= Production group code (4 digits 0001 to 9999)

Response: <ACK>, then echo of command packet

Controller must verify send <ACK> if correct (<NAK> if bad).

Pinpad stores new serial number if <ACK> received, sends <EOT>

.p

>>> Request Serial Number

<SI>06<SO><LRC>

Response: <ACK>, then:

<SI>06vvvyymmddfbbgggg<SO><LRC>

.. See above for data elements ..

Controller must send <ACK>, pinpad sends <EOT>

>>> DES Algorithm Reliability Test

<SI>07mck<SO><LRC>

m	= 16 character master key

c	= 16 character clear text

k	= 16 character known cipher text

Pinpad displays test results, sends <ACK>, then <EOT>

>>> Select Master Key

<SI>08n<SO><LRC>

n	= Master key slot ('0'-'9')

Pinpad sends <ACK>, then <EOT>

>>> UART loopback test

<SI>09<SO><LRC>

Response: <ACK>, then: <SI>09<SUB>PROCESSING<SO><LRC>

>>> Request Unencrypted PIN

<SI>10a<SO><LRC>

a	= Amount of transaction

Response: <ACK>, then 71 response packet (see "Request PIN entry")

>>> PIN Pad Connection Test

<SI>11<SO><LRC>

Response: <ACK>

>>> Request PIN entry

<STX>70.c<FS>wa<ETX><LRC>

c 	= Card account number (8-19)

w	= Encryoted working key (16)

a	= Amount 3-8 (must include decimal point)

Response: ACK, then:

<STX>71.fll01p<ETX><LRC>

f	= 0:No function key  1:Func1 2:Func2 3:Func3

ll	= Pin length (00-16)

p	= Encrypted PIN block (16 characters)

Controller must send <ACK>

>>> Cancel Session Request

<STX>72<ETX><LRC>

Returns PINPAD to idle state

Response <ACK>

.p

>>> Card image response

<STX>81.c<STX><LRC>

c	= Card data

Sent by pinpad after card swipe

Controller must send <ACK>

>>> Display SLIDE CARD

<STX>Q1<STX><LRC>

Response: <ACK>

>>> Indicate Host Done

<STX>Q2<ETX><LRC>

Displays "THANK YOU" and return to idle prompt

Response: <ACK>

>>> Ignore Card Swipe

<STX>Q3<ETX><LRC>

Turn off card reader & display "PIN PAL/PROCESSING"

Response: <ACK>

>>> Turn Card reader ON/OFF

<STX>Q4f<ETX><LRC>

f	= 0:On  1:Off

Response: <ACK>

>>> Alternate processing prompt

<STX>Q5f<ETX><LRC>

f	= 0:"PIN PAD"  1:"PIN PAL"

Response: <ACK>

>>> Return to IDLE state

<STX>Z1<ETX><LRC>

Response: <ACK>

>>> Display a string

<STX>Z2[sub]t<ETX><LRC>

t	= Text to display. if begins with <SUB> (1Ah), clear display

Response: <ACK>

>>> Display rotating messages

<STX>Z3c[SUB]m[<FS>m...]<ETX><LRC>

c	= Count of messages ('0'-'9')

m	= Message(s) to display

Response: <ACK>

.p

>>> Turn on/off CANCEL REQUESTED

<STX>Z7f<ETX><LRC>

f	= 0:Show cancel message  1:Do not show message

If enabled "CANCEL REQESTED" shows when CLEAR pressed or controller

requests a cancel termination.

Response: <ACK>

>>> Reset/Set Idle Prompt

<STX>Z8p<ETX><LRC>

p	= Idle prompt text (NULL = display now)

Response: <ACK>

>>> Request Coded Key

<STX>Z80t<ETX><LRC>

t	= Timeout (0-255) seconds to wait for key

Response: <ACK>, then: <STX>Z51k<ETX><LRC>

k	= Key code: 1 2 3 4 5 6 9 10 11 13 14 15 = 123456789*0#

Controller must send <ACK>

>>> Request non-coded key

<STX>Z42t<ETX><LRC>

t	= Timeout (0-255) seconds to wait for key

Response: <ACK>, then: <STX>Z43k<ETX><LRC>

k	= '0'-'9', '*' or '#'

>>> Request String Input

<STX>Z50etttll<ETX><LRC>

e	= 0:Echo as '*'  1:Echo normally  2:Do not echo

t	= Timeout (0-255) seconds to wait for input

l	= Maximum length (01-49 .. default 49)

Response: <ACK>, then: <STX>Z51s<ETX><LRC>

s	= String input (1-49 chars) '?' if timeout

Controller must send <ACK>

>>> Accept and Encrypt PIN

<STX>Z60.c<FS>w<ETX><LRC>

c	= Card account number (8-19)

w	= Encrypted working key (16)

Response: <ACK>, then 71 response packet (see "Request PIN entry")

.p

>>> Accept and Encrypt PIN with custom prompts

<STX>Z62.c<FS>wnnxxfm1<FS>m2<FS>mp<ETX><LRC>

c	= Card account number (8-19)

W	= Encrypted working key (16)

nn	= Minimum acceptable PIN length (01-16)

xx	= Maximum acceptable PIN length (01-16)

f	= Null flag  Y:Null PIN allowed  N:Null PIN not allowed

m1	= Message to alternate with m2 until key pressed (0-16)

m2	= Message to alterdate with m1 until key pressed (0-16)

mp	= Message to display after PIN entry

Response: <ACK>, then 71 response packet (see "Request PIN entry")

>> DUKPT Message Packets

>>> Pre-Authorization: PIN Entry Request

<STX>60c<ETC><LRC>

c	= Card account number (8-19)

Response: <ACK>, then 71 response packet (see "Request PIN entry")

This packet must be preceded by a display packet such as Z2 or Z3,

Pad will transmit pin in message 72, then terminal must send 62.

>>> Pre-Authorization: Transaction Amount Authorization Request

<STX>62ia<ETX><LRC>

i	= Credit/Debit indicator 'C' or 'D' (43/44h)

a	= Transaction amount must include '.' 3-7 chars.

Response: <ACK>, then:

<STX>63r<ETX><LRC>

r	= Response code: 0=Approved/1=Declined

>>> Pre-Authorization: PIN Entry Test Request

<STX>66c<ETX><LRC>

This functions the same as 60, except that PIN is not entered, a

preset PIN of "1234" is used.

>>> Request PIN Entry

<STX>70c<FS>ia<ETX><LRC>

c	= Card account number (8-19)

i	= Credit/Debit indicator 'C' or 'D' (43/44h)

a	= Amount 3-7 (must include decimal point)

Response: ACK, then:

<STX>71fkp<ETX><LRC>

f	= 0:No function key  1:Func1 2:Func2 3:Func3

k	= Key serial number Hex/Leading F's suppressed (20)

p	= Encrypted PIN block (16 HEX digits)

Controller must send <ACK>

>>> PIN Entry Test Request

<STX>76c<FS>ia<ETX><LRC>

This functions the same as 70, except that PIN is not entered, a

preset PIN of "1234" is used.

.p

>>> Load Initial key

<STX>90ek<ETX><LRC>

e	= Initial PIN encryption key (16)

k	= Key serial number (20 - including leading 'F's)

Response: <ACK>, then after 32 total keys:

<STX>91r<ETX><LRC>

r	= Response 0=confirmed, 1=NOT confirmed

>>> Display rotating messages

<STX>Z3c[SUB]m[<FS>m...]<ETX><LRC>

c	= Count of messages ('0'-'9')

m	= Message(s) to display

Response: <ACK>

>>> Accept and Encrypt PIN

<STX>Z60.c<ETX><LRC>

c	= Card account number (8-19)

Response: <ACK>, then 71 response packet (see "Request PIN entry")

Accept and Encrypt PIN with custom prompts

<STX>Z62.c<FS>nnxxm1<FS>m2<FS>mp<ETX><LRC>

c	= Card account number (8-19)

nn	= Minimum acceptable PIN length (01-16)

xx	= Maximum acceptable PIN length (01-16)

m1	= Message to be displayed before key pressed

mp	= Message to display after PIN entry (<FS>m2 is optional)

Response: <ACK>, then 71 response packet (see "Request PIN entry")

Message packets 02, 04, 07, 08 and 10 are disabled for DUKPT.

.P

> PRINTER 250 NOTES

>> Dip switches

1 - Parity: ON=Even Off=Odd

2 - Word length: ON=8-bit OFF=7-bit

3 - Baud:  ON=1200/2400 OFF=4800/9600

4 - Rate:  ON=1200/4800 OFF=2400/9600

>> Standard command set

LF			0A			Print buffer

FF			0C			Eject paper

DC2			12			Change ribbon color

CAN			18			Clear print buffer & cancel attributes

ESC a n;	1B 61 3x 3B	Set line heigh (7-255)

ESC	b n;	1B 62 2x 3B	Eject n lines

ESC c		1B 63		Reset to power-up state

ESC d		1B 64		Request printer status

Response (1 byte):	P F 1 X X X X L

P=Parity F=Fault X=Reserved L=paperLow

FS			1C			Enter Printer-250 (native) mode

GS			1D			Enter Printer-200 emulation mode

NUL			00			Pad (Ignored)

>> Advanced command set

RS			1E			Enable double-width printing

SO			0E			Select high-page graphics

US			1F			Cancel double-width printing

SI			0F			Cancel high-page graphics (select ASCII)

ESC e n;	1B 65 3x 3B	Set right margin

ESC f n;	1B 66 3x 3B	Select line attribute

0=Normal 1=Double-Height

ESC i		1B 69		Printer ID (responds 'A')

ESC r n;	1B 72 3x 3B	Retrieve printer information (0-5)

0=All			1=Model "P250"		2=Serial# "nn-nnn-nnn"

3=HWver "D"		4=SWver "ACnnnnn"	5=Checksum "xxxx"

ESC l ...	1B 6C .. 3B	Download character

ESC	g		1B 67		Switch to dot-matrix graphics

ESC	h n;	1B 68 3x 3B	Select character set (0-10)

0=US			1=France		2=Germany		3=UK

4=Denmark I		5=Sweden		6=Italy			7=Spain

8=Japan			9=Norway	   10=Denmark II