.T XASM/DASM Utilities

.$1FULL

.=M5

.S20

.c XASM / DASM

.s

.c Utility Programs

.s10

.c Revised 05-Jan-98

.s5

.c Dunfield Development Services

.c -----------------------------

.c High quality tools for

.c Embedded Development

.c at low prices.

.s

.c http://www.dunfield.com

.s2

.c ?COPY.TXT 1983-2005 Dave Dunfield

.c  -- see COPY.TXT --

.P

> HEX FILE FORMAT UTILITY

The HEXFMT utility provides several useful functions for manipulating

the HEX output files, usually before programming them into a ROM. It can

also translate from MOTOROLA to INTEL format, and change the record size,

padding etc. of the output file.

The available options to HEXFMT are:

.f-

-b					- Output file in BINARY format

b=value				- Identify BASE address of HEX file

c=value,value,value	- Calculate checksum

d=value				- Set default byte value

-e					- Inhibit end record

f=value				- Maximum number of consecutive FF's

-i					- Output file in INTEL format

l=value				- Set output file load address

-m					- Output file in MOTOROLA format

-q					- Quiet, Inhibit progress messages

r=value				- Set size (in bytes) of output records

s=value				- Set size (in bytes) of target ROM

-t					- Truncate output at maximum size loaded

-w					- Do not write output file

w=filename			- Write to this file

.f+

Anywhere a 'value' is used, it may be specified in any of

the following formats:

.f-

nnnnn				- Decimal number		(0 - 65535)

$xxxx				- Hexadecimal number	($0-$FFFF)

@oooooo				- Octal number			(@@0-@@177777)

%bbbbbbbbbbbbbbbb	- Binary number (%0-%1111111111111111)

.f+

>> Input file

HEXFMT reads its input file in either MOTOROLA or INTEL hex format (Such

as is produced by the cross assemblers), and builds a memory image of the

ROM which is to be produced.

The 's=' option specifies the size (in bytes) of the ROM image

(default 8192).

The 'b=' option specifies the base (starting) address to be assumed

for the ROM image (default is address in first record). The '-t' option

can be used to generate an output file containing only the area of memory

spanning the loaded data.

The 'd=' option specifies the value which is to be assumed for all

locations in the ROM image which are not loaded from the input file

(default is $FF).

.P

>> Output file

Unless told otherwise, HEXFMT will write the data back to the

same file it came from in the same format (MOTOROLA or INTEL) in

which it was read.

The '-w' option tells HEXFMT not to write an output file, it

is useful if you simply want to know the checksum of the input file.

The 'w=' option may be used to specify a different output file.

The 'r=' option may be used to tell HEXFMT how many bytes to

place in each output file record (default is 32).

The 'l=' option tells HEXFMT the beginning load address for the

output file records (default is Base address of input file).

The 'f=' option tells HEXFMT how many consecutive $FF characters to

allow in the output file before breaking the load address sequence

(default is 5). Most EPROM's assume the $FF value when in the blank

state, and therefore do not need to be programmed with this value.

Eliminating consecutive $FF values from the load file will reduce

the output file size and download time.

The '-i' and '-m' options may be used to force HEXFMT to output the file

in INTEL or MOTOROLA hex format, regardless of the input format. The '-b'

option causes HEXFMT to write the file in BINARY form. When this is done, the

output file will be an exact binary (8 bit) image of the target ROM.

.P

>> checksuming

The 'c=' option may be used to cause HEXFMT to calculate a 16 bit

checksum for a section of the ROM, and place it at a specified location.

The format of the 'c=' option is:

.c c=start,end,destination

HEXFMT will calculate a checksum, which consists of the 16 bit

sum of all byte (8 bit values) between the 'start' and 'end' addresses

specified (inclusive). This 16 bit value is then stored at the indicated

'destination' address.

NOTE: The start, end and destination addresses are given relative

to the beginning of the image. ie: 0000 is the first address in the

ROM.

For example, To calculate the checksum of an 8K (8192 byte)

ROM, beginning at the second location, and proceeding to the

end of the ROM, storing this value at location 0000 in the ROM,

you would use:

.c c=2,8191,0 (decimal)

.c or

.c c=$0002,$1fff,$0000 (hex)

The firmware in the ROM could then test the integrity of the

ROM using code similar to this (for 6809):

.f-

* ROUTINE TO TEST ROM CHECKSUM

LDD		#0				BEGIN WITH ZERO VALUE

LDX		#ROM+2			POINT TO SECOND BYTE OF ROM

LOOP	ADDB	,X+				ADD 8 BIT VALUE...

ADCA	#0				TO 16 BIT COUNTER

CMPX	#ROM+8192		ARE WE FINISHED (AT END)?

BNE		LOOP			NO, KEEP GOING

CMPD	>ROM			IS CHECKSUM OK?

BNE		CKFAIL			NO, REPORT ROM ERROR

.f+

Up to ten (10) 'c=' options may be specified in a single HEXFMT

command, and each checksum will be calculated and stored (in the order

in which they occur in the command line).

HEXFMT will also display (but not store) the checksum for the

entire ROM image, calculated in the above manner.

.P

> CROSS REFERENCE UTILITY

CREF is a utility program which will generate a cross reference

listing for an assembly source program. The output listing contains at

least one line for each symbol defined in the source file, the line where

it is defined, and the line numbers of all references to it.

The output listing is written to standard output where it may be

re-directed to a file or printer.

>> Using CREF

The format of the CREF command is:

.c CREF input_file [d=symbol... p=char -q w=value] >output_file

CREF normally does not report references to symbols which are not

defined in the source file. This prevents CREF from reporting accesses

to processor registers in the output listing. The 'd=' option allows

you to define additional symbols (such as processor registers) which

will then be reported. Multiple 'd=' options may be used to define

several additional symbols.

The '-Q' option causes CREF to be quiet, and not display progress

messages.

The 'p=' option specifies a prefix character which identifies temporary

symbols. Any symbol which begins with this character will not be displayed

in the cross reference listing.

The 'w=' option allows you to specify the width of the output file

(or device). The default value assumed if this option is not used is

80 characters.

.?@1=FULL

.P

> PORTING SOURCE GENERATOR

The PSOURCE utility is used to generate a 'porting' source file.

This is useful if you want to give someone the ability to assemble

your programs without giving them the actual source code. This might

be the case when someone is "porting" your program to another computer

with the same CPU but a different operating system environment.

PSOURCE makes a source file which is purposely difficult to read.

It accomplishes this by removing all comments, converting all occurrences

of blanks or tabs not contained within quotes to a single space, and by

changing all label names to meaningless strings. This gives an output file

which is only slightly better than a disassembly of your program, but will

assemble without error.

>> Using PSOURCE

The format of the PSOURCE command is:

.c psource input_file [-q -t k=symbol* K=file*] >output_file

The '-q' option causes PSOURCE to be quiet, and not display

its progress messages.

The '-t' option causes PSOURCE to use TAB characters to separate

the fields in the output files (instead of spaces).

The 'k=' option allows you to specify label names which are to

be 'kept', and not changed to meaningless strings. This allows you to

retain the labels and equates which must be changed during the process

of porting your software.

Up to 50 'k=' operands may be specified, allowing you to keep up

to 50 label names.

The 'K=' option allows you to specify a file containing the names

of symbols which are to be 'kept'. It has the same effect as entering

a 'k=' option for each line of the file.

Normally, PSOURCE removes any comments (lines beginning with '*') from

the output file. If you use ';' to begin a comment line, PSOURCE will convert

it to '*' (for XASM compatibility), and include it in the output file.

.?

.?@1=FULL,SMALL

.P

> SOURCE FILE (RE)TABULATOR

>> Introduction

RETAB is a very useful utility which re-tabulates files, so that they

will be correctly formatted when viewed on systems using different tab

stops. RETAB writes its output to the console, which may be re-directed

to a file using the DOS '>' operation.

The program will also perform a conversion of spaces to tabs, and

retain proper source formatting. This is useful with the output of our

disassembler, should you wish to have tabs in the resulting assembly

file instead of the spaces generated by the disassembler.

>> Command format

The format of the RETAB command is:

.c RETAB [options] <filename> [options <filename>]... >output

The available options are:

.f-

-f		- Inhibit filling with spaces. If this option is

specified, RETAB will only format the file as

close to the original format as it can using the

output tab stops only. If not specified, RETAB

will format the file to exactly the same visual

format, using spaces to fill in any partial tabs.

-s		- Causes RETAB to convert any strings of spaces in

the input file to tabs where possible.

i=n,...	- Specifies the input file tab stops.

o=n,...	- Specifies the output file tab stops.

.f+

NOTE: RETAB processes multiple files, and allows the tab settings

to be defined for each file. The '-f -s i= and o=' parameters must be

specified BEFORE the file name where they are to have effect.

If more tabs are present in the input or output file than were specified

(using I= and O=), RETAB assumes that the tabs continue at the spacing of

the last two stops. Therefore, if the tabs are at regular intervals, only

the first tab stop need be given.

Both input and output tabs default to 8 space intervals.

If 'O=0' is specified, RETAB will convert all tabs in the input file to

the appropriate number of spaces in the output file.

Examples:

.f-

retab i=4 tab4.fil >tab8.fil

retab o=4 tab8.fil >tab4.fil

retab i=4 o=2 tab4.file i=8 tab8.fil >tab2.fil

retab o=0 tab8.file >space.fil

.f+

.?

.P

> BINARY CONVERTION UTILITIES

>> BIN2ASM

BIN2ASM is a program which reads a binary file and converts it to an XASM

compatible source file, using either FCB (motorola) or DB (intel) statements.

.f-

Use: BIN2ASM input_file [output_file -Intel L=length N=name]

input_file  = Name of binary file to read

output_file = Name of ASM file to write. If not specified, BIN2ASM

writes to standard output (the PC screen).

-Intel      = Use intel style 'DB' (Default is motorola style 'FCB')

L=length    = Number of bytes to encode in each ASM line (default=16)

N=name      = Includes a label of this name. If 'N=' is not specified,

BIN2ASM outputs the initialzation values only.

.f+

>> BIN2HEX

BIN2HEX reads a binary file, and outputs a file containing Motorola

or Intel format ASCII-HEX download records.

.f-

Use: BIN2HEX input_file output_file [A=xxxx -I L=1-256]

input_file		= name of binary file to read

output_file	= name of HEX file to write

A=xxxx			= set output file Address	[0000]

-I				= output Intel format records	[Motorola]

L=1-256		= specify output record Length	[32]

-V				= inhibit Verbose output

.f+

.P

> SOURCE CONVERTORS

All DDS assemblers follow a generic syntax, which is similar to the

syntax of "Motorola" style assemblers. Little difficulty is encountered

when transferring code to and from assemblers of this type.

Exchanging XASM code with assemblers which support an "Intel" style

syntax can be a problem however, because there are several major differences

in syntax. The DDS assemblers will accept code in either format, however

you may wish to convert the style when moving the code to or from another

assembler which does not.

The XASM package includes two small utility programs, which perform

most of the work necessary to convert sources from one style to another.

INT2XASM reads an "Intel" style source program, and converts it to XASM

style, while XASM2INT reads XASM style source, and converts it to "Intel"

style. A third program SAM2XASM converts sources for the popular PseudoSam

freeware assembler to be compatible with XASM.

>> Using INT2XASM

The format of the INT2XASM command is:

.c INT2XASM input_file output_file <options>

The contents of the "Intel" style input_file are read and written to the

output file with several conversions to XASM style. The following options may

be used to inhibit the various conversions:

.i+5

.i-5 -C - Inhibit conversion of comment lines beginning with ';' to begin

with '*'.

.i-5 -L - Inhibit removal of trailing ':' from statement labels.

.i-5 -N - Inhibit conversion of any Binary, Octal, Decimal or Hex constants

of the form "nB, (nO or nQ), nD and nH" to the corresponding XASM equivalents

"%n, @n, n and $n".

.i-5 -@ - Inhibit conversion of 8051 indirect references of the form

"@expression" to the XASM equivalent "[expression]".

.i-5

.P

>> Using XASM2INT

The format of the XASM2INT command is:

.c XASM2INT input_file output_file <options>

The contents of the XASM style input_file are read and written to

the output file with several conversions to "Intel" style. The following

options may be used to modify the conversions:

.i+5

.i-5 B= H= O= - These options tell XASM2INT what characters to use as

suffixs for Binary, Hex and Octal constants. (Default is B=B, H=H, O=Q).

.i-5 L= - Defines a character to be appended to statement labels (Default

is ':').

.i-5 K= - Specifies the character which is recognized as a comment delimiter

(default is ';'). Any lines beginning with '*' are converted to this

character. Also, this character is inserted before any non-space character

which occurs after the column defined by (C=).

.i-5 C= - Defines a column number after which text is assumed to be

comments (default is 32). Any non-space characters found after this column

will be preceded by the (K=) comment character.

.i-5 T= - Defines the size of TAB stops in the input file (default is 8).

This information is used to determine the column numbers.

.i-5 -@ - Inhibit the conversion of 8051 references of the form "[expression]"

to "@expression".

.i-5

.?@1=FULL

>> Using SAM2XASM

The format of the SAM2XASM command is:

.c SAM2XASM input_file output_file <options>

The contents of the "PseudoSam" style input_file are read and written to

the output file with several conversions to XASM style. The following options

may be used to modify the conversion:

.i+5

.i-5 -5 - Used when converting 8051 sources. EQUates for the SFR's predefined

in xASM51 are removed, and definitions are added for the BIT's which are

predefined in PseudoSam.

.i-5 -I - Use Intel style directives instead of Motorola.

.i-5

.?