.=M5

.S20

.c XASM

.s

.c Cross Assemblers

.s

.c Users Guide

.s5

.c Release 2.51

.s

.c Revised 16-Feb-99

.s5

.c Dunfield Development Services

.c -----------------------------

.c High quality tools for

.c Embedded Development

.c at low prices.

.s

.c http://www.dunfield.com

.s2

.c ?COPY.TXT 1983-2005 Dave Dunfield

.c  -- see COPY.TXT --

.T XASM Cross Assemblers

.P

> INTRODUCTION

This package contains one or more of the DDS XASM cross assemblers,

and related utility programs. DDS XASM is a series of cross assemblers

for a number of popular microprocessor families:

.f-

asm00		- Motorola 6800/02 family

asm01		- Motorola 6801/6803 family

asm02		- Mostek/Synertek 6502 family

asm05		- Motorola 6805 family

asm08		- Motorola 68HC08 family

asm09		- Motorola 6809 family

asm11		- Motorola 68HC11 family

asm12		- Motorola 68HC12 family

asm16		- Motorola 68HC16 family

asm320		- Dallas 80C320/520 family

asm48		- Intel 8048 family

asm51		- Intel 8051/52 family

asm85		- Intel 8080/8085 family

asm86		- Intel 8086 family

asm88		- Intel 8008 family

asm186		- Intel 80186 family

asm96		- Intel 8096 family

asmAVR		- Atmel AVR family

asmH8		- Hitachi H8/300 family

asmP5X		- Microchip PIC-16C5x family

asmST7		- St Microelectronics ST-7 family

asmZ80		- Zilog Z80 family

asmZ8		- Zilog Z8 family

asmCF		- DDS C-FLEA Virtual Machine

macro		- Macro pre-processor

hexfmt		- HEX file manipulator

cref		- Cross reference utility

psource		- Porting source generator

retab		- Source file (re)tabulator

int2xasm	- Convert "Intel" style source to XASM style

xasm2int	- Convert XASM style source in "Intel" style

.f+

Dunfield Development Services (DDS) offers software and firmware

development services specializing in systems and embedded applications.

For more information, visit: http://www.dunfield.com

XASM is provided on an "as is" basis, with no warranty of any kind.

In no event shall the author be liable for any damages arising from its use

or distribution.

Throughout this document, angle braces ('<>') are used to indicate

operands for which a value must be supplied by the user. Square braces

('[]') are used to identify operands which are optional.

.P

> ASSEMBLER

XASM cross assemblers read a source file (.ASM), and produce a code

file (.HEX) containing either MOTOROLA or INTEL format ASCII-HEX

download records. A optional listing file (.LST) may also be produced.

>> Using the assemblers

An assembler is invoked by entering its name at the command prompt,

in the following format:

.C ASMxx <filename> [options]

The <filename> operand is the name of the file to be assembled, it

is assumed to have the extension ".ASM" if none is supplied.

Unless otherwise specified, the code produced by the assembler is written

to a file with the name '<filename>.HEX', and the listing is written to a

file with the name '<filename>.LST'.

>>> Command line options

The following options may be specified on the command line, following

the <filename> operand:

.i+5

.i-5 -A - [A]lternate jumps/calls (51,320,AVR)

Causes 8051 family assemblers to use AJMP/ACALL instructions

in place of LJMP/LCALL.

Causes the AVR assembler to use RJMP/RCALL in place of JMP/CALL when

an explicit page number is not supplied.

.i-5 -C - [C]ase sensitive

Causes the assembler to make a distinction between upper and lower

case characters in symbol names. If this option is not used, the assembler

will ignore case differences, and assume that the symbols are the same.

NOTE: When using this option, you must enter any register names in UPPER case.

.i-5 C=<filename> - Specify [C]ode file

This option allows you to specify the file to which the output code

is written. If no extension is supplied as part of <filename>, it defaults

to ".HEX".

.i-5 -F - Generate [F]ull listing

Causes the assembler to output a full source listing to the '.LST'

file. By default, only lines containing errors are written to the listing

file.

.P

.i-5 -I - Generate [I]ntel format HEX file

Causes the assembler to output the code to the '.HEX' file in

INTEL hex format. By default the code is written to the file

in MOTOROLA hex format.

.i-5 L=<filename> - Specify [L]isting file

This option allows you to specify the file to which the listing is

written. If no extension is supplied as part of <filename>, it defaults

to ".LST".

.i-5 O=<value> - [O]ptimization (00,01,02,05,08,09,11,12,16,96)

This option allows you to set a limit to the maximum number of

optimization passes which the assembler will perform while attempting to

minimize code size and resolve forward referenced symbols. If a symbol can

not be resolved within this many passes, the assembler will terminate with

an error message.

You may specify between 0 and 127 optimization passes to be run, with

0 being a special case in which no optimization is performed. Without

optimization, all non-specified memory references and offsets default to

their largest form, and forward references in EQU, ORG, or RMB statements

will not work. The code generated will not be optimal, but assembly time

is reduced. This feature is most useful when initially testing and debugging

a program.

The default maximum number of optimization passes is 3.

NOTE: The assemblers which do not support the 'O=' option do not permit

forward references in EQU, ORG or RMB.

.i-5 P=<length> - Set page length

This option specifies the number of lines which will be printed on

each page. The default number of lines per page is 60.

.i-5 -Q - [Q]uiet mode

Causes the assembler to be quiet, inhibiting the display of the

progress messages.

.i-5 -S - Generate [S]ymbol table

Causes the assembler to sort and display the symbol table at the end

of the listing file. By default, the symbol table is not displayed.

.i-5 -T - Output to [T]erminal

Causes the assembler to output the listing to the terminal, (via stdout)

instead of the usual '.LST' file.

.P

.i-5 -W - Wrap relative addresses (AVR)

Causes the AVR assembler to allow RJMP/RCALL to "wrap", making the

entire 1st 8K available. This option should only be used with CPU's that

perform this wrapping (ie: those with only 8K of code memory, and which

do not support JMP/CALL).

.i-5 W=<width> - Set page width

This option control the number of columns which will be used for the

printing of page titles and the symbol table listing. Default page width is

80 columns.

.i-5

.i-5 -X - Translate eXternal reads (51,320)

Causes 8051 family assemblers to translate all occurrences of

"MOVX A,[DPTR]" into "CLR A / MOVC A,[A+DPTR]", for use in systems with

simplified memory decoding logic.

>> Redirecting the listing file

When the listing file is directed to the terminal with the '-t'

option on the command line, it is displayed through the standard output

file, allowing it to be redirected to a printer etc. via the shell

'>' redirection operator (EG: ASMxx filename -f -t >LPT1:). The progress

messages ('First pass...' etc) are output through stderr, and will

therefore not be redirected with the listing. NOTE that the listing may

be more efficiently redirected with the 'L=' option.

.P

>> Source file format

NOTE: All XASM assemblers follow a consistent generic syntax, which is as

close as possible to that published by the majority of CPU makers. In a few

cases, this differs significantly from a unique syntax published for a

specific CPU or vendor. Please read this section carefully, and refer to the

EXAMPLE PROGRAMS to become familiar with the XASM syntax.

The assembly source input lines are in the following format:

.C '<label>[:]  <instruction>  <operands>  [;]<comment>'

Labels must begin in column one, and must be separated from the

instruction field by at least one blank or tab character. Labels may

be followed by a ':' if desired, however this is not required by the

assembler.

The instruction field must be indented by at least one space or

tab (ie: the instruction name cannot begin in column 1).

If the an instruction or directive requires operands, then the

operand field is required, and is separated from the instruction by at

least one blank or tab.

The optional comment field is delimited from the operand field by

at least one blank or tab, and is ignored by the assembler. Blanks or

tabs within the operand field are allowed only if contained within the

delimiters of a character string, otherwise they will be interpreted as

the end of the operand field.

The asm48/H8/P5X/ST7/Z80/Z8 assemblers require a leading ';' comment

delimiter on all line comments. These assemblers also permit blanks within

the operand field.

Any lines beginning with a '*' or ';' character in column one are

considered to be comments, and are not processed by the assembler.

>> Expressions

When an 8 or 16 bit value is required as an operand to an assembler

directive or an instruction, either a simple value, or an expression

consisting of simple values and operators may be used.

All expressions are evaluated using 16 bit values. When an expression

is evaluated in an instruction requiring an eight bit value, the lower

eight bits are used. Expressions are evaluated from left to right, as each

operator is encountered, without precedence. Precedence can be forced with

the use of brackets. Spaces or tabs are not allowed within an expression,

unless they are contained within a character string.

.P

The following operators may be used in an expression:

.=m+

.f-

>>> Unary (one operand) operators:

-	- Negation, returns the negative of the next value.

~	- Complement, returns ones complement of the value.

=	- Swaps the high and low bytes of the next value.

>>> Binary (two operand) operators:

+	- Addition.

-	- Subtraction.

*	- Multiplication.

/	- Division (unsigned).

\	- Modulus, returns remainder after division.

&	- Bitwise AND.

|	- Bitwise OR.

^	- Bitwise EXCLUSIVE OR.

<	- Shift left

>	- Shift right

>>> Values in expressions

The following forms of simple values may be used.

nnn		- Decimal number, eg: 21

nnnD	- "" "": 21d

nnnT	- "" "": 21t

$nnn	- Hexadecimal number, eg: $15

nnnH	- "" "": 15h  (Must begin with 0-9, eg: 0FFh)

%nnn	- Binary number, eg: %10101

nnnB	- "" "": 10101b

@nnn	- Octal number, eg: @@177

nnnO	- "" "": 177o

nnnQ	- "" "": 177q

'cc'	- ASCII characters, eg: 'A'

<label>	- Value of a label from symbol table.

*		- Value of current program counter.

$		- "" ""

.=m-

.f+

.P

>> Addressing Modes

XASM assemblers support all addressing modes for their respective

processors and will determine the type of addressing to use from the

instruction and/or its operands.

For cases where different compatible addressing modes are available,

assemblers having optimization (O= option) will use the smallest (most

efficient) form of unless optimization is disabled. Assemblers without

optimization will use the largest form unless specifically instructed

to use a smaller form.

>>> Immediate addressing

For all assemblers except ASM85, the operand is determined to be an

immediate value if it is preceded by a pound sign ('#'). For instructions

requiring only 8 bits of immediate data, the lower eight bits of the value

will be used. The higher 8 bits of a value can be accessed by preceding it

with '=' (swapping high and low bytes).

The '#' character is NOT used with immediate values in ASM85, because the

immediate addressing context is inherent in the instruction.

>>> Direct/Extended addressing

For assemblers supporting direct addressing (00, 01, 02, 05, 08, 09

11, 12, H8 and ST7), the operand is determined to be a DIRECT (8 bit) address

if it is preceded by a left angle bracket ('<'). For most assemblers, this

references the "zero" page of memory ($0000-$00FF).

Assemblers having optimization (O= option) allow an address to be forced

to a ABSOLUTE or EXTENDED reference by preceding it with a right angle bracket

('>'). Assemblers without optimization will always use this form of addressing

unless direct addressing has been specified (with '<').

>>> Indirect addressing

For processors supporting indirection of other addressing modes, this

is indicated by placing the addressing reference in square braces ('[]').

Additionally, ASM51 family assemblers support the 8051 notation of a leading

'@'.

.P

>>> ASM11/12 use of ';'

The 68HC11 and 68HC12 instruction sets have an ambiguity with regard

to the use of ',' in certain instructions. This has to do with the fact

that ',' can be used to indicate indirect addressing from a register (with

offset), and is also used to separate operands to some instructions.

To resolve this issue, the assemblers allow the use of ';' in the

following cases:

For the BCLR, BSET, BRCLR and BRSET instructions, the bit mask is

separated from the address by a ';':

.f-

eg:		BCLR	MEMLOC;$01

BSET	1,X;$02

BRCLR	MEMLOC;$01,LABEL

BRSET	2,Y;$02,LABEL

.f+

For ASM12's MOV, CALL, DBxx, IBxx and TBxx instructions, ';' is

used to separate the operands:

.f-

MOV		#10;MEMLOC

MOV		1,X;2,Y

CALL	LABEL;1

CALL	1,X;2

DBEQ	A;LABEL

.f+

.P

>> Assembler directives

The following directives (pseudo instructions) are supported by

all of the assemblers:

<label> EQU     <expression>

This directive sets the label on it's line to have the value of

the operand expression.

ORG     <expression>

This directive sets the internal program counter to the value of the

operand expression, such that subsequent code will be generated at that

address.

TITLE   <string value>

This directive sets the title of the program to the text on the

remainder of the line. The title is displayed at the top of each

page in the listing, and by default is set to the name of the file

being assembled. Lines containing this directive will not appear in

the listing.

PAGE

This directive forces a page eject in the listing. Lines

containing this directive will not appear in the listing.

SPACE

This directive causes a blank line to appear in the listing.

Lines containing this directive will not appear in the listing.

NOLIST

This directive turns off the source listing, preventing any further

lines from being displayed, until a LIST directive in encountered.

NOLIST and LIST may be nested, listing is resumed only when all levels

of NOLIST have been turned off. Lines in which errors occur are listed

regardless of the NOLIST directive.

LIST

Re-enables output listing, following a NOLIST directive.

LINE <expression>

resets the assemblers internal line counter to the specified value.

Lines containing this directive will not appear in the listing.

END

Causes the assembler to stop processing, and ignore the remainder

of the source file. Note that an END directive is NOT required.

.P

DB      <expr1>[,<expr2>,<expr3>,...]

.i FCB     <expr1>[,<expr2>,<expr3>,...]

These directives code the values of the operand expressions into

memory as single byte constants.

DW      <expr1>[,<expr2>,<expr3>,...]

.i FDB     <expr1>[,<expr2>,<expr3>,...]

These directives code the values of the operand expressions into

memory as double byte constants.

DRW     <expr1>[,<expr2>,<expr3>,...]

.i RDB     <expr1>[,<expr2>,<expr2>,...]

These directives are similar to DW/FDB, except that the high and low

bytes of the coded values are exchanged.

DS      <expression>

.i RMB     <expression>

These directives reserve a number of bytes of memory equal to

the value of the operand expression. The contents of the reserved

storage is undefined.

STR     'text string'

.i FCC     'text string'

These directive code the string into memory as ASCII byte values.

The string may be delimited by any character which is not part of the

string text.

STRH    'text string'

.i FCCH    'text string'

These directives perform exactly as the STR/FCC directives, except

that the high bit is set on the last character in the string.

STRZ    'text string'

.i FCCZ    'text string'

These directives perform exactly as the STR/FCC directives, except

that the string has a zero byte ($00) appended.

.P

>>> Directives supported by ASM09 only

SETDP    <expression>

This directive sets ASM09's default direct page (DP) register to the

eight bit value of <expression>. Whenever a reference is made to  a memory

location by address, without a specific addressing mode, the assembler will

use direct page addressing if the high byte of the memory address matches the

default DP register, otherwise extended addressing is used. It is the

responsibility of the programmer to insure that the 6809's DP register will

contain the proper high address value during memory references subsequent to

a 'SETDP' directive. The default DP register can be disabled by setting it

to a value greater than 255 (or less than 0), which causes the assembler to

always use extended addressing (this is the default at the beginning of an

assembly). NOTE: To access the high byte of a symbol, use: SETDP 255&=SYMBOL

>>> Directives supported by ASM16 & ASMAVR only

CPAGE <expression>

This directive sets the default code page that is assumed by JMP and

JSR/CALL instructions that do not contain an explicit code page.

>>> Directives supported by ASM51 & ASM320 only:

<label>	BIT		<expression>	(51, 320 only)

Similar to "EQU", except that the label is forced to have the "bit"

type, regardless of the type of expression used.

<label>	DATA	<expression>	(51, 320 only)

Similar to "EQU", except that the label is forced to have the "direct

non-bit" type, regardless of the type of expression used.

.P

>> Error Messages

When the assembler detects an error in the source code, it displays a

message indicating a possible cause of the error, in the source listing

on the line immediately following the line containing the error. Errors

which are encountered in the first pass, are displayed at the top of

the listing, along with the number of the line in which the error was

detected.

Forward references in any of the (EQU, ORG, or RMB/DS) directives

are not allowed, and will cause an 'Undefined symbol' error in the first

pass only, displayed at the top of the listing.

.P

>> Error message summary

>>> Duplicate symbol: <symbol name>

The indicated symbol is defined more that once within  this assembly.

>>> Symbol table overflow.

There are too many symbols (labels) in the program, and the assemblers

symbol table has become full.

>>> Unable to resolve: <symbol name>

The assembler is unable to determine a consistent value for the indicated

symbol. This is most likely due to excessive forward referencing, or a self

referencing loop.

>>> Unknown instruction

The instruction field on the indicated line does not contain a valid

instruction or assembler directive.

>>> Undefined symbol

A symbol referenced in the indicated line is not defined anywhere within

this assembly, and has no value.

>>> Out of range

The operand is not the range of values which can be used with the instruction.

eg: A short branch to an address which is further than +127 or -128 bytes from

the program counter.

>>> Invalid addressing mode

The addressing mode indicated by the operand field of the indicated line does

not apply to the instruction on that line.

>>> Invalid register specification

The instruction on the indicated line specifies a register which is not a

recognized register, or cannot be used in the context specified by the

instruction.

>>> Invalid expression syntax

The expression on the indicated line contains a character which is not

recognized as a valid operator.

>>> Invalid argument format

The indicated line has an operand which is not in proper format for the

instruction.

.P

>>> Improperly delimited string

A character string constant on the indicated line does not have a proper

closing delimiter. This is normally the single quote character, but may be

another character in conjunction with the FCC, FCCH, FCCZ, STR, STRH or STRZ

directives.

>>> Improper alignment

The instruction on this line either references, or resides at an odd

location, when an even location is required.

.P

>> Additional notes on ASM86

The 8086 assembly language was the most difficult to adapt to the

generic syntax used throughout the XASM package. To accommodate doing this,

a few extensions and changes were made:

The RETF instruction is unique in that its operand is optional, and thus

you must use ';' to indicate any comment following RETF without operands. No

special comment delimiter is required for any other instruction.

Symbols are used only to reference addresses, no information is recorded

as to the size of the symbol. At least one of the operands to an instruction

must contain an explicit size (8 or 16 bits). The assembler knows the the

size of the registers, and thus any instructions involving registers will

automatically use the correct size. For instructions which do not reference

registers, you can force one of the arguments to be recognized as an 8 or 16

bit value by using the '<'	or '>' character as a prefix:

.f-

MOV		AL,DATA		Moves 8 bits

MOV		AX,DATA		Moves 16 bits

MOV		DATA,#12	ERROR: "Size not known"

MOV		<DATA,#12	Moves 8 bits

MOV		DATA,#<12	Equivalent to above

MOV		>DATA,#12	Moves 16 bits

MOV		DATA,#>12	Equivalent to above

MOV		<DATA,#>12	ERROR: "Incompatible sizes"

.f+

The '>' and '<' prefixes also have special meaning in JMP and CALL

instructions:

.f-

JMP		label		NEAR 16 bit RELATIVE jump

JMP		<label		NEAR 8 bit RELATIVE jump

JMP		>LABEL		NEAR INDIRECT jump through LABEL

JMP		BX			NEAR Jump to address in BX

JMP		[BX]		NEAR INDIRECT Jump through BX

JMP		SEG:LABEL	FAR DIRECT jump

JMP		SEG:>LABEL	FAR INDIRECT jump *

JMP		SEG:[BX]	FAR INDIRECT jump *

* In the last two examples, the SEG value is ignored, and

the SEGMENT:OFFSET is taken from the operand address.

.f+

Offsets to indirect accesses are specified by placing a constant value

immediately before the opening '['. When both a BASE and an INDEX register

are involved, use '+' to indicate them:

.f-

MOV		AX,[BX]		Indirect through BX, no offset

MOV		AX,[SI]		Indirect through SI, no offset

MOV		AX,10[DI]	Indirect to DI with 10 byte offset

MOV		AX,[BP+SI]	Indirect through BP+SI, no offset

MOV		AX,5[BP+SI]	Indirect thought BP+SI, 5 byte offset

MOV		AX,[BX+10]	This MASM syntax NOT supported!!!

.f+

The REPEAT and LOCK prefixes are implemented as instructions, which must

precede the target instruction on a separate line.

.P

> MACRO PREPROCESSOR

MACRO is an assembly source code pre-processor which provides macro

substitution and conditional assembly facilities at the source code level

to virtually any assembler or cross assembler.

MACRO reads the raw assembler source from one or more files specified

as its arguments, and the processed assembler source file is written to

standard output where it can be redirected into a temporary file for

later assembly. All files specified as arguments are effectively concatenated

into the output file as they are processed, providing a convenient way of

maintaining large single source programs as more manageable separate files.

MACRO provides two types of macro definitions, substitution, and

instruction.

>> Substitution macros

Substitution macros may be used anywhere in any non-comment line of the

source file (lines not beginning with '*'), and when encountered, are simply

replaced by the text specified in the macro definition.

Substitution macros are defined using the 'SET' directive.

If the macro definition text contains spaces, tabs, or commas, it must be

enclosed in double quotes.

The following are examples of substitution macro definitions:

sub1     set		this_gets_substituted

sub2     set		"this gets substituted"

Substitution macros are only recognized of they are surrounded by

non-alphanumeric characters. This prevents substitutions from occurring

within other words which may contain the pattern. It does however

require the restriction that the macro names contain only

alphanumeric characters.

>>> Command line substitutions

Substitution macros may also be defined on the command line with arguments

of the form "<symbol>=<string>".

.C eg: macro source_file1 source_file2 mode=debug >output_file

.P

>> Instruction macros

Instruction macros may only be used in the instruction field of the

source file, and when encountered, are replaced by one or more lines

of assembly source code.

Instruction macros may be passed operands in the operand field of the

assembly source code. If multiple operands are to be passed, they should

be separated from one another by commas. If a single operand must contain

spaces, tabs, or commas, it should be enclosed in double quotes.

>>> Backslash commands

The backslash character '\' has special meaning within an instruction

macro definition as follows:

.f-

\n  - Substitute the <n>'th parameter from  the  macro  invocation

line. <n> may range from zero to  nine,  parameter  zero  is

considered to be the LABEL from that line, actual parameters

from the operand field begin with parameter one.

\#  - Substitute the number of parameters passed.

\$  - Substitute a unique number which the total number  of  macro

invocations to occur in this assembly so far. This is  often

used to create unique labels within the macro.

\.	- Substitute the current source file line number.

\@n - Substitute a portion of the current date based on the digit

'n', as follows:

\@@1 - Day of month, 1 or 2 digits.

\@@2 - Day of month, 2 digits.

\@@3 - Month of year, 2 digits.

\@@4 - Month of year in string form.

\@@5 - Month of year in three character string form.

\@@6 - Current year, 2 digits.

\@@7 - Current hour, 2 digits.

\@@8 - Current minute, 2 digits.

\@@9 - Current second, 2 digits.

NOTE: The time is recorded when MACRO is first invoked.

\\  - Place a backslash character in the macro definition.

.f+

Instruction macros are defined with the MACRO directive, and

are ended with the ENDMAC directive.

.P

>>> Example instruction macro

The following is an example of a instruction type macro using

6809 assembly language:

.f-

* MACRO TO PRINT A CHARACTER STRING

PRINT	MACRO

\0		PSHS	A,X			SAVE REGISTERS

LDX		#LAB2\$		STRING TO PRINT

LAB1\$	LDA		,X+			GET CHAR FROM STRING

BEQ		LAB3\$		END, QUIT

JSR		PRINTCHR	DISPLAY THE CHARACTER

BRA		LAB1\$		GO BACK FOR NEXT

LAB2\$	FCCZ	"\1"		STRING TO PRINT

LAB3\$	PULS	A,X			RESTORE REGISTERS

ENDMAC

.f+

>> Macro directives

MACRO supports the following directives, which may be placed in the

instruction field of an assembler source file. Note that only substitution

type macros may be used as values in operands to MACRO directives, as MACRO

has no knowledge of the labels and symbols used by the assembler.

.i+

<name> SET <string>

The SET defines the substitution type macro with the name specified

by <name> to have the text specified by <string>. Double quotes may be

used if <string> is to contain tabs, spaces, or commas. If the macro

being defined already exists, it is redefined with the new value.

<name> should begin in column one.

<name> ESET <format>,<expression>

The ESET directive sets a symbol to the numeric value of an expression,

as processed through a format string. The content of the format string is

copied to the symbol, with the value substituted where one of these special

sequences is found:

.f-

%u		- Unsigned value of expression

%d		- Signed value of expression

%x		- Hexadecimal value of expression

%o		- Octal value of expression

%b		- Binary value of expression

%c		- ASCII character value of expression

%%		- A '%' sign

.f+

A width value may be specified between the '%', and the output type

denoted above, in which case the number will be output right justified in

a field padded with spaces to the defined width. If the number contains a

leading '0', the field will be padded with '0's. If the number if negative,

the field will be left justified.

.P

.f-

Examples:

A SET 10

B SET 1

C ESET %x,A+B			; 'B'  (11 in HEX)

C ESET %3u,A+B			; ' 11'

C ESET %03u,A+B			; '011'

C ESET %-3u,A+B			; '11 '

.f+

<name> MACRO

The MACRO directive begins the definition of a instruction type macro.

All subsequent lines up to a ENDMAC directive are included in the macro

definition. These lines are not written to the output file.

ENDMAC

This directive terminates an instruction type macro definition.

IF <expression>

The IF directive causes MACRO to process and output all subsequent

lines up to an ELSE of ENDIF directive, only of the result of the

expression is non-zero.

IFEQ <string>,<string>

The IFEQ directive causes MACRO to process and output all subsequent

lines up to an ELSE or ENDIF directive, only if the two argument strings

match.

IFNE <string>,<string>

The IFNE directive causes MACRO to process and output all subsequent

lines up to an ELSE or ENDIF directive only if the two argument strings

do not match.

ELSE

The ELSE directive causes MACRO to process and output all subsequent

lines up to an ENDIF directive only of the preceding IFEQ or IFNE

directive failed.

ENDIF

This directive marks the end of a section of conditional assembly

statements following a IFEQ, IFNE, or ELSE directive.

INCLUDE "<filename>"

The contents of the specified file are included in the output file at the

point where the INCLUDE directive occurs. Macro processing is performed on the

included file.

.P

ABORT "<text>"

Causes MACRO to terminate, displaying an error message containing

the specified text.

PREFIX <char>

Sets a prefix character which must be applied to any MACRO directive

in order for it to be recognized. For example: "PREFIX %" causes MACRO to

only recognize it's directives when '%' is prepended. (ie: "SET" becomes

"%SET"). This can be used to avoid conflicts with processors instructions

in some assemblers. Also note that the prefix can be set on the command

line with a "-P<char>" syntax.

.c eg: macro source_file(s) -P% >output_file

>>> Expressions

The numeric expressions accepted for the IF and ESET directives

consist of values and operators. The values may be of any of these

forms:

.f-

n, nT, nD	- Decimal number

$n, nH		- Hexadecimal number

@n, nO, nQ	- Octal number

%n, nB		- Binary number

'cc'		- ASCII character(s)

.f+

The following operators are supported:

.f-

+   - Addition

-   - Subtraction and unary negation

*   - Multiplication

/   - Division

%   - Modulus (remainder)

&   - Bitwise AND

&&  - Logical AND

|   - Bitwise OR

||  - Logical OR

^   - Bitwise exclusive OR

>   - Test for greater than (True=1, False=0)

>=  - Test for greater than or equal to

>>  - Shift right

<   - Test for less than

<=  - Test for less than or equal to

<<  - Shift left

==  - Test for equal to

!=  - Test for NOT equal to

!   - Unary logical NOT

~   - Unary bitwise compliment

.f+

There are no precedence rules. Operations are performed left to right,

in the order that they they are encountered in the expression. Brackets ()

can be used to force a different order of evaluation.

.i-

.P

>> Source File Synchronization

MACRO removes it's own commands, and adds macro expansions to the source

file as it is processed. A result of this is that lines in the output file

will usually NOT have the same number as the corresponding line in the input

file. This can be a problem when tracking down error messages produced by

the assembler, since the line numbers displayed do not correspond to the

original input file. To overcome this problem when using the DDS XASM cross

assemblers, MACRO has a command line -L option, which causes it to generate

"LINE" directives whenever it determines that the input and output line

numbers could be out of sync. These directives will reset the assemblers

line counter to match the lines of the original source file.

.P

> APPENDICES

>> Object file formats

Object files are produced by the assemblers in either of two standard

formats, both of which represent the binary data as two digit printable

ASCII hex numbers. The exact contents of the two standard format object

files is as follows:

.F-

>>> Motorola hex format

Data Record: 'Stnnaaaadddddddddddddddddddddddddddddddd...cc'

Where:  S = 'S', indicates start of data record

t = Record type, '1'=data, '9'=end of file.

n = Count of number of bytes in record.   (in ASCII/HEX)

a = Load address of data record.          (in ASCII/HEX)

d = Actual data bytes in record.          (in ASCII/HEX)

c = Checksum of count, address, and data. (in ASCII/HEX)

Note1: Checksum is computed as one's complement of eight

bit sum of all values from 'nn' to end of data.

Note2: Count 'nn' is three greater then the number of data

bytes in the record.

>>> Intel hex format

Data Record: ':nnaaaattdddddddddddddddddddddddddddddd...cc'

Where:  : = Indicates start of data record

n = Count of number of bytes in record.   (in ASCII/HEX)

a = Load address of data record.          (in ASCII/HEX)

t = Record type				 (00=Data, 01=End of file)

d = Actual data bytes in record           (in ASCII/HEX)

c = Checksum of count, address, and data. (in ASCII/HEX)

Note1: Checksum is computed as two's complement of eight

bit sum of all values from 'nn' to end of data.

Note2: End of file record contains count of 00.

.P

>> ASCII code chart

.s5

Most Significant Figure

HEX||  0  |  1  |  2  |  3  |  4  |  5  |  6  |  7  |

===++=====+=====+=====+=====+=====+=====+=====+=====+

0 || NUL | DLE |     |  0  |  @  |  P  |  `  |  p  |

---++-----+-----+-----+-----+-----+-----+-----+-----+

1 || SOH | DC1 |  !  |  1  |  A  |  Q  |  a  |  q  |

L   ---++-----+-----+-----+-----+-----+-----+-----+-----+

e    2 || STX | DC2 |  "  |  2  |  B  |  R  |  b  |  r  |

a   ---++-----+-----+-----+-----+-----+-----+-----+-----+

s    3 || ETX | DC3 |  #  |  3  |  C  |  S  |  c  |  s  |

t   ---++-----+-----+-----+-----+-----+-----+-----+-----+

4 || EOT | DC4 |  $  |  4  |  D  |  T  |  d  |  t  |

S   ---++-----+-----+-----+-----+-----+-----+-----+-----+

i    5 || ENQ | NAK |  %  |  5  |  E  |  U  |  e  |  u  |

g   ---++-----+-----+-----+-----+-----+-----+-----+-----+

n    6 || ACK | SYN |  &  |  6  |  F  |  V  |  f  |  v  |

i   ---++-----+-----+-----+-----+-----+-----+-----+-----+

f    7 || BEL | ETB |  '  |  7  |  G  |  W  |  g  |  w  |

i   ---++-----+-----+-----+-----+-----+-----+-----+-----+

c    8 || BS  | CAN |  (  |  8  |  H  |  X  |  h  |  x  |

a   ---++-----+-----+-----+-----+-----+-----+-----+-----+

n    9 || HT  | EM  |  )  |  9  |  I  |  Y  |  i  |  y  |

t   ---++-----+-----+-----+-----+-----+-----+-----+-----+

A || LF  | SUB |  *  |  :  |  J  |  Z  |  j  |  z  |

F   ---++-----+-----+-----+-----+-----+-----+-----+-----+

i    B || VT  | ESC |  +  |  ;  |  K  |  [  |  k  |  {  |

g   ---++-----+-----+-----+-----+-----+-----+-----+-----+

u    C || FF  | FS  |  ,  |  <  |  L  |  \  |  l  |  |  |

r   ---++-----+-----+-----+-----+-----+-----+-----+-----+

e    D || CR  | GS  |  -  |  =  |  M  |  ]  |  m  |  }  |

---++-----+-----+-----+-----+-----+-----+-----+-----+

E || SO  | RS  |  .  |  >  |  N  |  ^  |  n  |  ~  |

---++-----+-----+-----+-----+-----+-----+-----+-----+

F || SI  | US  |  /  |  ?  |  O  |  _  |  o  | DEL |

---++-----+-----+-----+-----+-----+-----+-----+-----+